# A RUST BASED, ECS PATTERNED, INTERACTIVE FLIGHT SIMULATOR

# THESIS

# Chad A. Willis, 2d Lt, USAF

# AFIT-ENG-MS-21-M-XXX

# DEPARTMENT OF THE AIR FORCE

# AIR UNIVERSITY

###### AIR FORCE INSTITUTE OF TECHNOLOGY

# Wright-Patterson Air Force Base, Ohio

# DISTRIBUTION STATEMENT A.

# APPROVED FOR PUBLIC RELEASE; DISTRIBUTION UNLIMITED.

# The views expressed in this thesis are those of the author and do not reflect the official policy or position of the United States Air Force, Department of Defense, or the United States Government.  This material is declared a work of the U.S. Government and is not subject to copyright protection in the United States.
AFIT-ENG-MS-21-M-XXX

# A RUST BASED, ECS PATTERNED, INTERATIVE FLIGHT SIMULATOR

THESIS

Presented to the Faculty

Department of Aeronautics and Astronautics

Graduate School of Engineering and Management

Air Force Institute of Technology

Air University

Air Education and Training Command

In Partial Fulfillment of the Requirements for the

Degree of Master of Science in Computer Science

Chad A. Willis, BS

2d Lt, USAF

March 2021

# DISTRIBUTION STATEMENT A.
APPROVED FOR PUBLIC RELEASE; DISTRIBUTION UNLIMITED.

AFIT-ENG-MS-21-M-XXX

# A Rust based, ecs patterned, interactive flight simulator

# THESIS

Chad A. Willis, BS

2d Lt, USAF

Committee Membership:

Dr. Douglas D. Hodson, PhD

Chair

Maj. Richard A. Dill, PhD

Member

Dr. Scott L. Nykl, PhD

Member

AFIT-ENG-MS-21-M-XXX

# Abstract

This thesis explores the Entity-Component-System (ECS) software design pattern for use in military flight simulation, by implementing a “medium fidelity” flight dynamics model.

A flight dynamics model (FDM) is the math and physics calculations required in a flight simulation. The physics calculations required, including principles such as forces, moments, equations of motion, and quaternions, and their role in the flight model are explained. These principles and their role in the creation of a FDM are explained.

Rust is programming language of choice for building this simulation. Rust is a new systems programming language, with the first stable release in 2015, which was designed on the basis of memory safety and performance. Rust can guarantee software is completely free of all memory issues. Also, the strategy in which Rust handles memory comes with a positive side effect in terms of performance; Rust does not employ a garbage collector, and checks for memory issues at compile time – this is good for performance. (too much detail? Delete part about garbage collector?)

The use of the modern ECS architecture is the design choice to build this flight dynamics model, as opposed to other software programming paradigms, such as Object Oriented Programming (OOP), because of its powerful design. This design simply provides an alternate way of organizing data and allows for improved software performance, and maintainability.

Finally, with the physics integrated into a Rust-based ECS patterned application, network packets are then sent to FlightGear, a flight simulator application exclusively being used a visualization system, with the data that it needs to generate the simulation to the screen. Through user input, the interactive simulation is executed. The overall usefulness of the technologies implemented is then weighed. (? Analysis / results ?).

# Table of Contents

Page

Abstract	1

Table of Contents	3

List of Figures	5

I.  Introduction	6

1.1 Introduction	6

1.2 Problem Statement	8

1.3 Research Objective	8

1.4 Hypothesis	8

1.5 Contribution	9

1.6 Approach	9

1.7 Assumptions/Limitations	10

1.8 Thesis Overview	10

II. Background	11

2.1 Chapter Overview	11

2.6 Physics of Flight Modeling	11

2.5 Entity Component System	18

2.2 Rust	21

2.3 FlightGear	24

2.4 Rust and Networking	25

III.  Methodology	26

Chapter Overview	26

Test Subjects	26

Summary	26

IV.  Analysis and Results	26

Chapter Overview	26

Results of Simulation Scenarios	26

Investigative Questions Answered	26

Summary	26

V.  Conclusions and Recommendations	27

Chapter Overview	27

Conclusions of Research	27

Significance of Research	27

Recommendations for Action	27

Recommendations for Future Research	27

Summary	27

Appendix: Configuring FlightGear as a Visual System	28

Bibliography	31

# List of Figures

Figure 1: Forces Visualized	12

Figure 2: Airplane Components	13

Figure 3: Airfoil Cross-section	14

Figure 4: Airplane Axes of Rotation	15

Figure 5: Body Frame and Earth Frame	16

Figure 7: Entity Component System Visualized	24

# I.  Introduction

## 1.1 Introduction

(Add “purpose of flight sim”/ motivations?...)

The creation of a military oriented flight simulator is not a trivial problem. It requires understanding of flight physics in order to calculate how an airplane should fly through a simulation in real-time. The actual process and product of calculating the physics is referred to as a flight dynamics model (FDM), which is essentially all of the math behind the flight simulator. As a plane is flying, many things are happening. The pilot of the airplane, or in this case, the simulation user, could be deciding to accelerate, turn, and change altitude, sometimes all at once. With that user input, the FDM calculates and determines how the plane will react. Although the creation of FDM is a relatively standard practice, the FDM created in this thesis is unlike any other.

The implementation of the Entity Component System (ECS) design into a flight simulation is a useful endeavor. In the gaming industry, the implementation of ECS in video games and simulations is growing in popularity. Notably, Unity uses an ECS architecture in its game engine. Unity is a platform for creating and operating real-time 3D games [15]. The massively popular battle royale game Fall Guys: Ultimate Knockout, for example, was created with Unity, which relies on the ECS design [16]. The ECS architecture is simply a new way of organizing data, alternatively to using Object-Oriented Programming (OOP) to organize data. ECS is referred to as a Data-Oriented Design (DOD). Where OOP relies on hierarchies of data, ECS relies on compartmentalization of data, where data content and its functionality on the data are separated. This design, which is discussed in more detail in Chapter 2, leads to code that is more reusable, readable, and performant. The software package, SPECS, is the library, or, as the Rust programming language calls it, crate, that is imported in code allows the ECS to be implemented in Rust. With that, the leveraging of the ECS design in a military oriented simulation is of particular interest because it has never been done before.

Along with ECS, Rust is a new programming language with features that are useful in building a flight simulator. Rust, created in 2015, introduces a combination of important features not available in any other language. These provided features stem from two concepts that Rust focuses on: memory safety and performance. Rust will always guarantee memory safety, avoiding issues such as data races, dangling references, and buffer overflows in software. It accomplishes this safety by checking that memory is squared away and cleaned up at compile time. Other languages cannot make this guarantee because they rely on the programmer to be sure the software is memory safe. Although, some languages are also memory safe like Rust, but they are lack in performance in order to accomplish memory safety. This is because they use a garbage collector at runtime to ensure memory safety – this not ideal for performance. Rust’s performance, however, is a positive result of the way it tackles memory issues because it does not need a garbage collector and checks for issues at compile time.  These features are undoubtedly worthwhile to take advantage of in a flight simulator.

(is this too much like the actual background chapter 2? Feel like I may be missing more motivation / purpose info)

## 1.2 Problem Statement

The problem at hand is investigating how to build a flight simulator leveraging the novel technology: ECS (SPECS) and Rust. (how do I expand here without being repetitive)

## 1.3 Research Objective

The primary research objective is to build an interactive ECS-patterned flight simulator in the Rust programming language. (how do I expand here without being repetitive)

## 1.4 Hypothesis

The hypothesis is that the ECS design pattern can be used to build a flight simulator. An ECS is merely a new way of organizing data, and separating responsibility in software. It does this by separating functionality from memory, as opposed to having functionality and memory tied together in hierarchies. This is powerful in designing complex video game engines, which have many game entities, such as bullets, and many functions on those game entities. The same reasons to use the ECS design in a large video game also carries over to the flight simulation created in this thesis, which can use this strategy organizing data.

Having software arranged as an ECS should allow the seamless integration of other software also arranged as an ECS. This is possible due to the way ECS compartmentalizes memory by separating it from functionality.

## 1.5 Contribution

The contribution of this thesis is determining the usefulness of the technologies used to build the flight simulation. The technologies include Rust, and ECS (SPECS). Whether or not these technologies used are advantageous to increase software safety, performance, simplicity, and management is determined.

## 1.6 Approach

The approach to building a flight simulation in Rust begins with research in the programming of flight physics, which is typically implemented in games or simulations. There are two textbooks of great interest: Physics for Game Developers by David M. Bourg, and Physics for Game Programmers by Grant Palmer. Both of these books describe a flight dynamics model capable of being implemented in software. The models in each book are unique; Bourg’s book is written in C++, and Palmer’s is written in Java. The authors also take different fundamental approaches in the math of their model. For example, Bourg uses quaternions to track 3D rotations of an airplane, while Palmer uses 3x3 matrices to keep track of rotations. In this case, the attributes that give the best performance and ease of coding was chosen for use. With that, to build the Rust FDM as an ECS, the overall fundamental calculations stay consistent with the textbooks but the code is very different. Regardless, the existing code references in the textbooks are able to be written and implemented in Rust.

With the FDM complete, the code is ready to be implemented as an ECS with the help of the SPECS crate. ( I suppose it could be made into ecs at the time of being written… that’s not how ive been doing this. I haven’t been worrying about ecs code right now). To do this, an understanding of ECS was grasped primarily by watching YouTube videos, some of which were sponsored by Unity. With that covered, The Specs Book, provided an introduction to the SPECS crate for Rust. This was all the necessary research to implement the FDM as an ECS. (im sure there will be more to add here later in regards to my approach)

## 1.7 Assumptions/Limitations

The flight simulation built is not a standalone application. The flight simulator built in this thesis does not provide a visualization system on its own. It requires an external scenery generator in order to have graphics displayed on the screen. In this case, FlightGear is being used as the visualization system. An assumption is that the user has a visualization system to receive the network packets from the FDM. A limitation is that the packets received by the visualization system may not be interpreted correctly because the FDM was written with FlightGear as the host.

## 1.8 Thesis Overview

This thesis is arranged into five chapters. Chapter II explains relevant background information on flight dynamics, Entity Component Systems, Rust programming and FlightGear. Chapter III provides the experiment methodology. Chapter IV shows the analysis of the data obtained from Chapter III. Finally, Chapter V summarizes the research and impact of the research.

(should any of these headings be deleted, or lumped together as a different heading name? or different heading names entirely? The headings choices change from thesis to thesis. Problem statement, research object, and hypothesis seem pretty similar. )

# II. Background

## 2.1 Chapter Overview

This chapter will provide an overview of the components that have gone into building this flight simulation. First, the background explains the basics of modeling flight physics in a FDM. Next, the ECS architecture, and an implementation if it, SPECS, will be described. Furthermore, the fundamentals of the Rust Programming language will be laid out. Next, the use of the FlightGear flight simulator will be explained. Lastly, networking and sending packets of data to FlightGear with Rust will be described.

## 2.6 Physics of Flight Modeling

A flight dynamics model is the process and product used to calculate all of the math and physics required to simulate an airplane. Specifically, we are simulating a rigid-body airplane moving in real-time through a 3D world. With that, this section will provide an overview of what is involved in the FDM in order to approximate real-life flight characteristics of an airplane. The final result is integrating the equations of motion. This boils down to determining the position of the airplane, as well as the airplane’s velocity, which is the speed and direction of the airplane, over time. [20]. But before the equations of motion are solved, an understanding of forces, airplane components, airfoils, rotations, and the coordinate systems is needed.

There are four major forces that act on an airplane in flight: gravity, lift, thrust, and drag. These forces govern the motion of the airplane. Newton’s Second Law of Motion says: the acceleration of a body is proportional to the resultant force acting on the body. The resulting equation is F = ma. Gravity pulls the airplane to the ground and lift is generated by the airplane wings to counteract gravity to keep the airplane aloft. Thrust is generated by the airplanes jet engine or propeller to increase velocity and to keep the airplane generating lift. Drag is the opposite of thrust; drag impedes an airplanes motion [14]. Torque, also known as moment, is similar to force, but force is what causes linear acceleration, and torque is what causes rotational acceleration.  In order to fly, an airplane must have a lift force acting upon it greater than or equal to gravity. Figure 1 is a visual of the four forces [14].

Figure 1: Forces Visualized

An airplane is composed of different parts that affect the state of the airplane. The wings are the rectangular parts coming out of the main body. The span is the length of the wing, whereas the chord is the width of the wing. The ratio of span squared to wing area is called aspect ratio. The ailerons are on the outside end of the wings, and the flaps are on the inside of the wings. The elevators are on the tail, and look like small flaps. The rudder is the vertical flap on the tail. Figure 2 shows these airplane components [14].

Figure 2: Airplane Components

The cross-sectional structure of the wings, called an airfoil, is also important in understanding forces. Camber represents the asymmetry between the top surface and bottom surface of the airfoil which creates a curvature. (probably delete these details)////////The mean camber line is the midpoint line of the thickness of the airfoil. The actual measure of camber is the maximum difference between the mean camber line and the chord line. The chord line is the straight line from the trailing edge to the leading edge. ////////The angle of attack is the angle between the chord line and the direction of travel, or oncoming flow that the plane is moving through. Figure 4 shows a labeled cross-section of an airfoil with an angle of attack [23].

Figure 3: Airfoil Cross-section

When an airplane is flying, it can rotate about three axes relative to the pilot. The orientation of the airplane when it is rotating can be described by three angles: pitch, roll, and yaw. These angles are called the Euler angles and represent the x, y, and z axis, on a coordinate plane, respectively. The ailerons create the roll rotation. The elevators create the pitch rotation. The rudder creates the yaw rotation. The flaps do not rotate the plane, but they do affect lift by altering the chord and camber of the wing, which has to do with the shape of the cross-section of each wing. Figure 3 shows the three types of airplane rotations [14].

Figure 4: Airplane Axes of Rotation

Coordinate systems are important to understand when talking about 3D rotations. Coordinates are used to track the position of the airplane. There are two different x, y, z coordinate systems: the earth frame, and the body frame. The earth frame is a frame of reference that is fixed somewhere on the earth, sometimes the center. The body frame is fixed to the center of gravity of the rigid body airplane, so it moves with the airplane [20]. Rotations happen along each of the three x, y, z axes. Figure 4 shows the body frame located on the airplane, along with the arbitrarily placed earth frame located on the bottom-right [21].

Figure 5: Body Frame and Earth Frame

With an understanding of the airplane components, airfoils, forces, rotations, and the coordinate systems, we can begin to explain the steps the FDM takes.

- The mass properties of the airplane are calculated

- The load, or forces and moments on the airplane are calculated

- The equations of motion are integrated

The mass properties involve calculating the center of gravity of the rigid-body airplane, and calculating the moment of inertia tensor. The center of gravity, or center of mass, is where the body frame reference is centered on the airplane. Also, the center of gravity is where the forces are applied on the airplane. The moment of inertia is essentially the airplanes resistance to rotation. It can also be described as a distribution of mass. It is the amount of torque needed to make the airplane rotate about an axis relative to another point (i.e. center of mass). The measure of how fast the airplane actually rotates over time is called angular velocity [22]. The moment of inertia tensor is the mathematical expression, represented as a 3x3 matrix, that has magnitude and direction, and holds the values of the moments of inertia as body coordinates.

With mass properties handled, the forces and moments acting on the airplane at all times can be calculated. Forces acting on the airplane are first calculated in body-fixed coordinates and then converted to earth-fixed coordinates before the gravity is applied.

Lift and drag are not as simple to calculate compared to gravity and thrust. There are many components on the airplane that affect lift and drag. But to generalize, the total lift force on an airfoil is composed of two components: the lift due to camber and the lift due to attack angle. The airplane’s flaps are used to alter camber. Usually the flaps are used are low speeds to increase lift. Drag increases with increased angle of attack. If an airplane is flying at an angle of attack high enough, it will not be able to create any lift and will stall. Figure 6 shows a graph of increasing lift with increase angle of attack, however, at some point a potential stall occurs with lift dropping off rapidly. Also, shown in Figure 6 is that with flaps lowered, lift is increased [14].

Figure 6: Lift Coefficient vs Angle of Attack

There are also other variables that affect lift such as air density, speed, viscosity, surface area, and aspect ratio. External forces also exist that act on the airplane such as air drag, and winds. (haven’t gotten this far…. I think the program sets a constant wind, at least that’s in chapter 8 for particles, don’t think they do it in the flight model)

Gravity is simply based on mass of the airplane and the acceleration of gravity, which is a constant -9.8 m/s2. So, gravity can be calculated by the mass of the airplane x -9.8 m/s2, which is added to the force acting on the z axis (down). Similarly, thrust, expressed in pounds, is force added to the x axis (forward). Without thrust propulsion, the airplane cannot make lift. The thrust will change depending on the simulation users input with the use of the throttle.

With forces dealt with, the equations of motion are integrated so that the simulation can progress through time, determining the position of the airplane, as well as the airplane’s velocity in each step of time. In computer simulations, the equations of motion are approximated by solving nonlinear ordinary differential equations using numerical integration algorithms [20]. The integration of the equations of motion are considered an approximation because there is always some error introduced depending on the technique chosen [14]. Using Euler angles is by far the most common numerical integration technique. In this case, we use an improved and more accurate version of Euler’s method, called the Range-Kutta method, which the respective code is laid out in Bourg’s textbook.

Simply put, the integration of the equations of motion, given everything else talked about, approximates velocity and position based on the previous values in time. During the approximation, the orientation of the plane is tracked with the use of a quaternion. A quaternion is a way to represent orientation of something in three dimensions. It is like a vector, made of four components. Quaternions get updated with new Euler angles in each time step to reflect the new orientation using the angular velocities of that instant.

With the FDM described, fidelity is the degree to which the characteristics of a flight simulation matches that of the real world [17]. In this case, the FDM is of “medium” fidelity because the characteristics of the flight simulation match that of real life, but is not computationally expensive, and requires minimal data [18]. One may think that a flight model with the highest fidelity, or accuracy, is the best – this is not always true. Very advanced flight modeling will require large, detailed sets of data about the stability and performance of an aircraft that it is modeling. Wind tunnel data analysis is an example of the kind of data needed for a high fidelity flight model. This data is not readily available and often costs hundreds of thousands of dollars [13].  (this paragraph on fidelity probably not needed… how can we actually measure the degree of fidelity? How do we know the movement lines up with the real world?)

/////There is some change in time Δt that is defined as the time increment in which the simulation steps thru time progressively. There is also a change in velocity Δv. To approximate the velocity of the airplane, it, the velocity of the previous time step needs to be known. The initial velocity condition being 0. The equations of motion can be integrated once more to approximate displacement, position (displacement). ////

Outline These things are being considered in the calculations:

(these are the things to expand on based on what is coded in the model)

- Initialize plane

- Calc Mass properties: center of gravity, Moments and product of intertia. Inertia matrix is in body frame. determines the torque needed for a desired angular acceleration about a rotational axis; similar to how mass determines the force needed for a desired acceleration

- Calc airplane loads (Forces): Lift and drag calculations. Just about all of the forces acting on the airplane are first calculated in body-fixed coordinates and then converted to earth-fixed coordinates before the gravity force is applied. Lift and drag are functions of air density, speed, viscosity, surface area, aspect ratio, and angle of attack. Thrust, gravity added.

- Stepsimulation (Integration equations of motion): actually integrate the equations of motion so that thesimulation can progress through time. Eulers basic method (chp 7 better methods). This function also updates the orientation via quaternion. Air speed. Lastly, the three Euler angles—roll, pitch, and yaw—are extracted from the orientation quaternion. Func happens once a rotation cycle.

- Flight controls: flight controls allow you to interact with the airplane’s various controls surfaces in order to actually fly the plane via keyboard.

(lift + quat garbage that will be deleted below here in physics section)

A quaternion is a way to represent orientation of something in three dimensions. Specifically, we are about unit quaternions. It is like a vector, made of four components, which satisfies this equation: q02 + qx2 + qy2 + qz2 = 1. Figure x visualizes that the rigid body can rotate about some arbitrary axis u at the rigid bodies center of gravity.

First, you set up a quaternion that represents the initial orientation of the rigid body at time 0. Then you update the orientation to reflect the new orientation at a given instant in time using the angular velocities that are calculated for that instant. A quaternion can be made given the from each Euler angles.

qroll = [cos(φ/2), (sin(φ/2)) i + 0 j + 0 k]

qpitch = [cos(τ /2), 0 i + (sin(τ /2)) j + 0 k]

qyaw = [cos(ψ /2), 0 i + 0 j + (sin(ψ /2)) k]

The resulting quaternions are then multiplied together to get one quaternion representing the airplanes orientation.

We will begin with calculating lift and drag. Because different components of the airplane affect lift and drag differently, components need to be divided up into groups that have similar affects to then do the calculations on each group. To start with the lift and drag forces:

  - Break up airplane components into chunks

  - The geometric and foil performance data of the airplane is collected.

  - Next, relative air velocity over the wings needs to be calculated.

The formula is: vR = vcg + (ω × r)

vcg is the vector representing air speed and flight direction
	ω (omega) is the angular velocity of the airplane

r is the distance vector from the airplanes center of gravity to the group of components being worked on

  - Next, the angle of attack needs to be calculated. This is calculated by taking the dot product of lift force vector and the normal vector perpendicular to the wing section plane.

  - Finally, the lift and drag can be calculated. They are both functions of air density, speed, viscosity, surface area, aspect ratio, and angle of attack. They are also expressed in terms of coefficients CL and CD.

The formulas are: CL = L / [(1/2) ρ V2 S]

CD = D / [(1/2) ρ V2 S]

S is span times chord, L is lift force, D is drag force, V is speed through the air, and ρ (rho) is air density.

Thrust is equal to the thrust produced by the propeller multiplied by the airplane velocity, v (palmer). The formula is: PT FT v

## 2.5 Entity Component System

The FDM will be organized, and implemented in code as an ECS architecture. ECS is a different way to organize data, and a viable alternative to organizing code with the Object Oriented Programming (OOP). The ECS structure is popular in designing game engines, such as Unity [8]. Where OOP focuses on inheritance and hierarchies, ECS focuses on composition. ECS gets rid of the troubles that arise with inheritance. Suppose we have a space or world that we want to populate with things. These things may have similar attributes as well as unique attributes. With OOP, you would have a base class called Thing that all other Thing types will inherit. The limitation is that one child class can only inherit directly from one parent. For example, a Mammal type may have a child Dog which inherits a fur attribute, and a Bird may have a child Duck which inherits the lays eggs attribute. But in this case, a platypus, which has fur and lays eggs needs to inherit both attributes [9]. This complicates things. The hierarchy chains that are created in game design can become too large to work with and code becomes too highly dependent on other code [10]. This makes maintenance of code and adding new features a pain with OOP. This is where ECS shines.

In an ECS structure, you do not have hierarchy levels – you have these things called Entities, Components, and Systems. You start with an Entity, which represent concrete objects in the world, like a bullet. Entities on their own are useless; they are just containers which Components can be added to. Components hold data and represent a feature. For example, Position is a Component of an Entity. The Components called Position, Render, and Spawning may be added to the Bullet Entity. The mixing and matching of Components can create unique Entities. A Player Entity can be created by adding the Components Position, Render, and Health. Systems are what give functionality to an ECS architecture. A System could be created called Render, which handles all of the rendering functionality of the world. Or a System called Health could handle all of the health functions. Systems have something called a filter. Filters look for every Entity which has some combination of specified Components. For example, the Render System’s filter wants to find every Entity with the Render data Component and Position data Component. Once these Entities are found, they are returned to the Render System to perform some job on it [11]. This is this example visualized [11]:

Figure 7: Entity Component System Visualized

As seen in Figure 1, the Systems are separate from the Components. The Systems, when performing a job, does not care what Entity it is operating on. The whole point is that the data Components are completely separated from the Systems functionality [12]. This separates code dependencies, making code easy to refactor, and makes it simple to implement concurrency. With data separated from functionality, a job can be operated on multiple Entities concurrently. Also, because a Systems does a job on specified data Components, they only iterate over Entities that they care about. This is all good for performance.

The FDM created will use this ECS structure. To implement the ECS in Rust, something called Specs will be used. Specs is library, usually used for games and simulations, which makes it easy to build a program leveraging the ECS structure. Specs maintains the reason one would use an ECS, code is highly parallel, flexible, and fast. This is good for the FDM because it will be easy to use parallelism, and add features – all with excellent performance. [12].

## 2.2 Rust

Rust is the programming language being leveraged to build a flight dynamics model (FDM). It is the language of choice given its advantageous features, namely, its memory safety and performance. Rust was released in 2015 by Mozilla, which has been behind the language’s upbringing and success. Rust will guarantee that a program will be memory safe, with no invalid data accesses, all while remaining just as fast as other popular programming languages [1]. The C++ programming language, for example, is a systems level language, giving developers complete access to data inside memory, with no memory safety precautions. On the other hand, the Python programming language takes away a developer’s access of low level memory, but does provide complete memory safety by using a garbage collector to clean up unneeded memory – this imposes a runtime cost [1].  Rust is unique because it is a systems level language, it does not have a garbage collector, and it will guarantee memory safety. This means Rust provides control, is fast, and protects against memory issues such as data races, and dangling pointers [1]. These attributes touted by Rust make it the choice programming language for building a FDM. 
	The safety assurance of Rust is accomplished by its system of ownership, borrowing, and lifetimes. All languages have a way to manage memory while a program is running. Some languages have a garbage collector which actively checks for memory that is no longer being used during runtime. Other languages make the programmer manually allocate and then free memory. Rust takes neither approach; it instead has something called a “borrow checker.” At compile time, the Rust borrow checker will check that all accesses to data is legal and valid [1]. The borrow checker deals with three important Rust concepts: ownership, borrowing, and lifetimes.

Ownership refers to the fact that every use of a value needs to be valid and cleaned up when it is done being used. Ownership is accomplished with three rules that the programmer must follow in Rust [2]:

- Every value has a variable which is called its owner,

- There may be only one owner at a time

Borrowing is the next important concept in Rust. Borrowing allows data to be “borrowed” from one owner (variable binding) to another owner and accessed for a moment without actually changing ownership of that data. When ownership of memory is transferred to another owner, the original owner cannot legally be used, so borrowing is the solution. The borrow checker will statically guarantee that references always point to valid objects. So, if a reference to an object exists, it cannot be destroyed. There are two rules to borrowing [2]:

- Any borrow must last for a scope no greater than that of the owner,

- You may have one or the other of these two kinds of borrows, but not both at the same time:

- One or more references (&T) to a resource

- exactly one mutable reference (&mut T)

These rules avoid the possibility of a data race because although there can be more than one pointer to a resource, only one can be accessing and changing that resource at a time. This is good for performance, and uses less memory overhead [1].

The next important Rust concept is lifetimes. A value has a lifetime, which is the period that accessing that value is valid to do [2]. A lifetime starts when a value is created and that lifetime ends when it is destroyed. Rust’s compiler, or borrow checker, is smart enough to know a values lifetime in many common circumstances [3]. In these common circumstances, the lifetime of a value does not need to be explicitly written, but otherwise, a lifetime parameter needs to be written. The borrow checker tries to limit the period of a lifetime. A value should not be alive any longer than it absolutely needs to be because, otherwise, it could be referenced by accident. This could cause a dangling reference, which is a reference to unintended data [1]. So, to keep some piece of data alive for only as long as completely necessary, a lifetime is specified for that data.

With safety dealt with by the borrow checker at compile time, the programmer may be at ease when it comes to memory safety. At the same time, there is no cost in performance at runtime because everything is checked at compile time without a garbage collector. The downsides to Rust are minimal, but they do exist. For one, compile time is longer. Also, the compiler is very strict. New Rust programmers often “fight with the borrow checker” to get their code to compile because it is more difficult to write code that abides by Rusts rules [1]. But the compiler is like a safety net because once the code compiles, the programmer knows it is safe and correct. Less time is spent fixing bugs, and more time is spent getting the compiler right [4].

Overall, Rust can hold its own against more popular languages, and the learning curve of the language is met with staggering benefits in safety and performance. One will learn that programming in Rust is less about execution flow and what the CPU is doing, and is more about how data is laid out in memory, and how ownership of memory is given to different parts of the program [4]. The Rust community is active and quickly growing. The Rust ecosystem is also rapidly expanding, with now over 46,000 crates available to download [19].

## 2.3 FlightGear

FlightGear is an open-source flight simulator application. However, it is being used solely as visual system so as to avoid having to handle graphics generation in this thesis. FlightGear, although it has its own built-in FDM, is easily configurable to instead use an external, custom FDM. FlightGear can be configured to be interfaced, or communicated with, via network packets to achieve a desired display output. In this case, these packets of data will be sent from the Rust application to FlightGear and the packets will contain all of the real-time physics calculations of the FDM. FlightGear will interpret these packets to then know how to display the subsequent simulation to the screen.

To properly configure FlightGear for the simulation, it must manually be told to accept the new external FDM, as well as listen for the respective packets [5]. This is done from the command prompt upon running the executable. The command line arguments are listed in Appendix A: Configuring FlightGear as a Visual System.

With FlightGear configured, it will process the received network packets constructed by the Rust application. But FlightGear requires that these packets are defined in a particular format so as to understand them. The format of the packet, before being converted to bytes, must be a struct, which is outlined in the FlightGear source code. The struct contains all of the data needed to define an airplane and how it flies throughout the simulation. This struct, called FGNetFDM, in Rust code is located in Appendix A: Configuring FlightGear as a visual System.

## 2.4 Rust and Networking

Now, it is understood that networking is needed to send packets of data from the Rust application to FlightGear so that it can display the desired simulation. Networking can simply be defined as two programs communicating with each other, sending and receiving data. To communicate with each other, two sockets are required at each end of communication. In this case, the type of socket being used is the Datagram socket. The protocol involving Datagram sockets is called the User Datagram Protocol (UDP). This protocol is considered “connectionless” because an open connection does not need to be maintained.  Packets containing bytes of data may simply be built and sent to a destination without a connection needing to be established prior [7].

In Rust, a UdpSocket can be created and binded to an internet protocol address and port on the local machine. After this, data can be sent and received from any other socket address. The socket created can then be connected to the other socket with its internet protocol address and port. From here, data can be sent, and received with a function call [2]. In regard to interfacing FlightGear’s socket, before data packets are sent, the literal order of the bytes need to be converted from least significant byte first, to most significant byte first [7].

# III.  Methodology

## Chapter Overview

The purpose of this chapter is

## Test Subjects

Text

## Summary

Text

# IV.  Analysis and Results

## Chapter Overview

Text

## Results of Simulation Scenarios

Text

## Investigative Questions Answered

Text

## Summary

Text

# V.  Conclusions and Recommendations

## Chapter Overview

Text

## Conclusions of Research

Text

## Significance of Research

Text

## Recommendations for Action

Text

## Recommendations for Future Research

Text

## Summary

Text

# Appendix: Configuring FlightGear as a Visual System

fgfs.exe – runs the executable
--aircraft=ufo – select an aircraft name
--disable-panel – disable instrument panel
--disable-sound – disable sounds
--enable-hud – enable heads up display 
--disable-random-objects – disable random scenery objects
--fdm=null – turn off the built-in FDM
--vc=0 – specify initial airspeed
--timeofday=noon – time of day 
--native-fdm=socket,in,30,,5500,udp – sets up FlightGear to receive data from an external source. Open a socket, which takes in packets at a speed of 30 hz on port 5500 [6].

Altogether, when running FlightGear, the command line is:

fgfs.exe --aircraft=ufo --disable-panel --disable-sound --enable-hud --disable-random-objects --fdm=null --vc=0 --timeofday=noon --native-fdm=socket,in,30,,5500,udp

The struct representing an airplane in FlightGear, written in Rust:

struct FGNetFDM

{

version: u32, // increment when data values change

padding: f32, // padding

// // Positions

longitude: f64, // geodetic (radians)

latitude: f64, // geodetic (radians)

altitude: f64, // above sea level (meters)

agl: f32, // above ground level (meters)

phi: f32, // roll (radians)

theta: f32, // pitch (radians)

psi: f32, // yaw or true heading (radians)

alpha: f32, // angle of attack (radians)

beta: f32, // side slip angle (radians)

// // Velocities

phidot: f32, // roll rate (radians/sec)

thetadot: f32, // pitch rate (radians/sec)

psidot: f32, // yaw rate (radians/sec)

vcas: f32, // calibrated airspeed

climb_rate: f32, // feet per second

v_north: f32, // north velocity in local/body frame, fps

v_east: f32, // east velocity in local/body frame, fps

v_down: f32, // down/vertical velocity in local/body frame, fps

v_body_u: f32, // ECEF velocity in body frame

v_body_v: f32, // ECEF velocity in body frame

v_body_w: f32, // ECEF velocity in body frame

// // Accelerations

A_X_pilot: f32, // X accel in body frame ft/sec^2

A_y_pilot: f32, // Y accel in body frame ft/sec^2

A_Z_pilot: f32, // Z accel in body frame ft/sec^2

// // Stall

stall_warning: f32, // 0.0 - 1.0 indicating the amount of stall

slip_deg: f32, // slip ball deflection

// // Pressure

// // Engine status

num_engines: u32, // Number of valid engines

eng_state: [f32; 4], // Engine state (off, cranking, running)

rpm: [f32; 4], // // Engine RPM rev/min

fuel_flow: [f32; 4], // Fuel flow gallons/hr

fuel_px: [f32; 4], // Fuel pressure psi

egt: [f32; 4], // Exhuast gas temp deg F

cht: [f32; 4], // Cylinder head temp deg F

mp_osi: [f32; 4], // Manifold pressure

tit: [f32; 4], // Turbine Inlet Temperature

oil_temp: [f32; 4], // Oil temp deg F

oil_px: [f32; 4], // Oil pressure psi

// // Consumables

num_tanks: u32, // Max number of fuel tanks

fuel_quantity: [f32; 4],

// // Gear status

num_wheels: u32,

wow: [f32; 3],

gear_pos: [f32; 3],

gear_steer: [f32; 3],

gear_compression: [f32; 3],

// // Environment

cur_time: f32, // current unix time

warp: f32, // offset in seconds to unix time

visibility: f32, // visibility in meters (for env. effects)

// // Control surface positions (normalized values)

elevator: f32,

elevator_trim_tab: f32,

left_flap: f32,

right_flap: f32,

left_aileron: f32,

right_aileron: f32,

rudder: f32,

nose_wheel: f32,

speedbrake: f32,

spoilers: f32

}

# Bibliography

[1] T.S. McNamara. Rust In Action, 2019.

[2] Steve Klabnik and Carol Nichols. The Rust Programming Language, 2019.

[3] Richard L. Apodaca. Rust Ownership By Example, 2020. [Online; accessed 07/01/2020].

[4] Oliver Gould and Lachie Everson. “All About Rust.” YouTube, uploaded by Microsoft Developer, 2019.

[5] wiki.flightgear.org. Command line options, 2019. [Online accessed 07/02/2020].

[6] wiki.flightgear.org. Property Tree/Sockets, 2019. [Online accessed 07/02/2020].

[7] Brian Hall. Beej’s Guide to Network Programming Using Internet Sockets, 2019.

[8] Mike Geig. “ECS Overview - Intro To The Entity Component System And C# Job System 1/5.” YouTube, uploaded by Unity, 2018.

[9] Ben. “Entity Component System Overview in 7 Minutes.” YouTube, uploaded by Board To Bits Gaming, 2019.

[10] Adam. Entity Systems are the future of MMOG development – Part 2, 2007. . [Online; accessed 07/01/2020].

[11] Mike Geig. “ECS Overview - Intro To The Entity Component System And C# Job System 2/5.” YouTube, uploaded by Unity, 2018.

[12] Thomas Schaller. The Specs Book. Crates.io, 1st edition, 2018.

[13] wow what was this source…

[14] bourg book

[15] https://unity.com/solutions/game

[16] https://unity.com/case-study/mediatonic-fall-guys

[17] A Handbook of Flight Simulation Fidelity Requirements for Human Factors Research

[18] Flight Dynamic Modelling and Simulation of Large Flexible Aircraft

[19] https://crates.io/ # of crates

[20] AIRCRAFT FLIGHT DYNAMICS AND CONTROL wayne durham

[21] https://www.youtube.com/watch?v=5AR-zrb93fM&ab_channel=VDEngineering

[22] https://en.wikipedia.org/wiki/Moment_of_inertia

[23] https://en.wikipedia.org/wiki/Airfoil airfoil diagram