# An ecs-based flight dynamics model written in rust

# THESIS

# Chad A. Willis, 2d Lt, USAF

# AFIT-ENG-MS-21-M-XXX

# DEPARTMENT OF THE AIR FORCE

# AIR UNIVERSITY

###### AIR FORCE INSTITUTE OF TECHNOLOGY

# Wright-Patterson Air Force Base, Ohio

# DISTRIBUTION STATEMENT A.

# APPROVED FOR PUBLIC RELEASE; DISTRIBUTION UNLIMITED.

# The views expressed in this thesis are those of the author and do not reflect the official policy or position of the United States Air Force, Department of Defense, or the United States Government.  This material is declared a work of the U.S. Government and is not subject to copyright protection in the United States.
AFIT-ENG-MS-21-M-XXX

# An ecs-based flight dynamics model written in rust

THESIS

Presented to the Faculty

Department of Aeronautics and Astronautics

Graduate School of Engineering and Management

Air Force Institute of Technology

Air University

Air Education and Training Command

In Partial Fulfillment of the Requirements for the

Degree of Master of Science in Computer Science

Chad A. Willis, BS

2d Lt, USAF

March 2021

# DISTRIBUTION STATEMENT A.
APPROVED FOR PUBLIC RELEASE; DISTRIBUTION UNLIMITED.

AFIT-ENG-MS-21-M-XXX

# An ecs-based flight dynamics model written in rust

# THESIS

Chad A. Willis, BS

2d Lt, USAF

Committee Membership:

Dr. Douglas D. Hodson, PhD

Chair

Maj. Richard A. Dill, PhD

Member

Dr. Scott L. Nykl, PhD

Member

AFIT-ENG-MS-21-M-XXX

# Abstract

This thesis explores the implementations of a two flight dynamics models (FDM) to re-implement them structured as an Entity-Component-System (ECS) software design pattern for use in military computer flight simulation. The first FDM is based on the works of Grant Palmer in his textbook titled Physics for Game Programmers. The second FDM is described in the textbook Physics for Game Developers, authored by David Bourg. The ECS architecture is based on the Data-Oriented Design (DOD) paradigm. The intent of the research was to divide and define both the Components and the Systems for this FDM. In an ECS architecture, Components contain the data in the software, and the Systems are the operations on that data.

This thesis defines the Systems as the computations associated to: the equations of motion, and flight control. The main aspect of this thesis consists of a System that computes the FDM’s equations of motion, and a Component that holds the resulting data. The specific ECS selected for this research is the Rust-based SPECS framework. These ECS compatible Systems and Components, when applicable, were verified to be accurate in modeling flight when compared against David Bourg’s C++ and Object-Oriented Programming (OOP) designed FDM.

# Table of Contents

Page

Abstract	1

Table of Contents	3

List of Figures	5

List of Tables	6

I.  Introduction	7

1.1 Problem Statement	10

1.2 Research Objective	10

1.3 Hypothesis	11

1.4 Contribution	12

1.5 Approach	12

1.6 Assumptions/Limitations	13

1.7 Thesis Overview	13

II. Background	14

2.1 Physics of Flight Modeling	14

2.2 Entity Component System	24

2.3 Rust	29

2.4 FlightGear	33

2.5 Networking with Rust	34

III.  Methodology	35

3.1 Building the FDM Overview	35

3.2 Set Up	37

3.3 SPECS Components	39

3.4 SPECS Systems	39

3.5 FDM Equivalency Verification	42

IV.  Analysis and Results	46

4.1 Simulation Application	47

4.2 FDM Equivalency Verification	47

V.  Conclusions and Recommendations	49

5.1 Research Conclusions	49

5.2 Significance of Research	50

5.3 Recommendations for Future Research	50

Summary	50

Appendix: Configuring FlightGear as a Visual System	51

Bibliography	54

Acronyms	57

# List of Figures

Figure 1: Forces Visualized	15

Figure 2: Airplane Components	16

Figure 3: Airfoil Cross-section	17

Figure 4: Airplane Axes of Rotation	18

Figure 5: Earth Space and Body Space Visualized	19

Figure 6: Lift Coefficient vs Angle of Attack	21

Figure 7: Entity Component System Visualized	27

# List of Tables

Table 1: Flight Control Instructions for Test One	41

Table 2: Flight Control Instructions for Test Two	41

Table 3: Flight Control Instructions for Test Three	41

Table 4: Flight Control Instructions for Test Four	42

Table 5: Initial Parameter Values for Each Test	43

Table 6: Flight Control Instructions for Test One	44

Table 7: Flight Control Instructions for Test Two	44

Table 8: Flight Control Instructions for Test Three	45

Table 9: Flight Control Instructions for Test Four	45

Table 10: Flight Control Instructions for Test Five	45

Table 11: Flight Control Instructions for Test Six	46

Table 12: Numerical Results for Test One	51

Table 13: Numerical Results for Test Two	52

Table 13: Numerical Results for Test Two	52

Table 13: Numerical Results for Test Two	52

Table 12: Numerical Results for Test One	53

Table 13: Numerical Results for Test Two	53

Table 14: Numerical Results for Test Three	54

Table 15: Numerical Results for Test Four	54

Table 16: Numerical Results for Test Five	54

Table 17: Numerical Results for Test Six	55

# I.  Introduction

Real-time computer simulations are important tools for the military [29]. The reasons to use a virtual computer simulation over real-life simulation is because it is less expensive, more accessible, more versatile, and completely safe. Virtual simulations are cost effective as they do not require real-life vehicles and equipment, which require fuel, maintenance and manpower to operate. They are more accessible since all you need is a capable computer to use it. They are more versatile because you can simulate scenarios that might not be possible to put together in real-life. And finally, they are completely safe due to the fact that users cannot hurt themselves.

Today, military computer simulations exist to simulate everything that could take place at war, such as battle planning or individual pilot training [29]. The simulation software used is often written using the Object-Oriented Programming (OOP) paradigm, which revolves around classes of data and subsequent objects [***NONE]. But there is an alternative, modern paradigm, called Data-Oriented Design (DOD), which focuses on the data itself and how it is laid out in memory. The DOD paradigm provides some powerful benefits over OOP that we will explore in this thesis as it relates to flight simulation. Specifically, we will discuss an implementation of DOD, called the Entity Component System (ECS) architecture. ECS is a programming design architecture that is primarily used in the video game industry to design game engines. But simulations are similar enough to games to be able to take advantage of this architecture. In this thesis, the leveraging of the ECS design in a military oriented simulation is of particular interest because it has never been done before. The ECS architecture is simply a new way of organizing data, alternatively to using OOP to organize data. Where OOP relies on hierarchies of data, ECS relies on compartmentalization of data, where data content and its functionality on the data is separated. Notably, Unity, a platform for creating and operating real-time 3-Dimensional (3D) games, uses an ECS architecture in its game engine [15]. The massively popular battle royale game Fall Guys: Ultimate Knockout, for example, was created with Unity, which relies on the ECS design [16]. The ECS architecture, which is discussed in more detail in Chapter 2, leads to code that is more reusable, readable, and performant. But we will also discuss possible pitfalls in software created using an ECS.

In this thesis, we focus on the research and development of flight dynamics models (FDM). This is the important piece of flight simulation software that calculates realistic flight behavior. The sequence of physics calculations in the FDM designed to mimic real-life flight are called the equations of motion. The equations of motion calculate how an airplane should be translated and rotated, while taking into account external forces, like gravity, as well as the physical properties of the airplane. The degrees of freedom (DOF) classification of an FDM represents the possible translation and rotation of the airplane supported by the FDMs mechanics. Three degrees of freedom are associated with translation (e.g., x, y, z) and three are associated with rotation (e.g., pitch, roll, yaw) for a total of six possible degrees that could be calculated. This thesis describes creating two FDMs, one is 4-DOF and the other is 6-DOF, that are built on a specific DOD-based pattern: ECS. Although an FDM is only the piece of code that calculates how an airplane flies, we subsequently connect both FDMs to a keyboard flight control system, and a graphics visualization system – constructing an interactive flight simulator.

To actually implement the ECS architecture in a FDM, the Rust programming language was chosen. Rust, created in 2015, introduces a combination of important features not available in any other language. These provided features stem from two concepts that Rust focuses on: memory safety and performance. Rust will always guarantee memory safety, avoiding issues such as data races, dangling references, and buffer overflows in software. It accomplishes this safety by checking that memory is squared away and cleaned up at compile time. Other languages cannot make this guarantee because they rely on a highly-experienced programmer to be sure the software is memory safe. Although some languages are also memory safe like Rust, they sacrifice performance in order to accomplish memory safety. This is because they use a garbage collector that cleans up memory at runtime to ensure memory safety – this not ideal for performance [1]. Rust’s performance, however, is a positive result of the way it tackles memory issues because it does not need a garbage collector and checks for issues at compile time. These features are undoubtedly worthwhile to take advantage of in a flight simulator. But Rust is not without drawbacks compared to other languages, which we will discuss. In order to implement the ECS design within the Rust language, the framework, SPECS, is the library, or, as the Rust programming language calls it, crate, that is imported in code.

## 1.1 Problem Statement

In real-time military flight simulations, it is common to see the software using an OOP design approach, and be written in a systems level programming language like C++ [***NONE]. It is likely that the simulation could spin up hundreds or even thousands of moving entities, or game objects, such as airplanes, or soldiers. These entities have many different functions acting on potentially large sums of data associated with the entities. Unfortunately, with OOP, the entanglement of functionality and data memory in hierarchies impedes performance and also causes messy code dependencies – which complicates the addition or modification of functionality or entities. Furthermore, when programming with systems level programming languages, it is common for beginner and intermediate level software developers to introduce memory vulnerabilities in code. An expert programmer would be required to ensure that memory is cleaned up properly.

## 1.2 Research Objective

The scenario stated above could benefit from the compartmentalization that the ECS approach employs in game engines. With functionality and data decoupled, software can be as performant as possible in terms of memory-management and parallelization. With that, the decoupling nature of ECS improves ease of code development because pieces of code are not dependent on each other in order to work [34]. To summarize, the modern ECS architecture is a viable solution which can provide some notable performance advantages and ease of development advantages over OOP in video games, which carries over to simulations. Furthermore, Rust solves the memory safety burden that plagues systems level languages by enforcing strict rules at compile time. The goal of this research is to build two interactive, easily extensible, FDMs implemented as Rust-based, ECS-based architectures. The 4-DOF FDM is based on the works by Grant Palmer in [30], while the 6-DOF FDM is based on the works by David Bourg in [14].

## 1.3 Hypothesis

The thesis hypothesizes that a FDM implemented as a Rust-based, ECS design will not only serve as a viable design alternative, but also introduces some powerful features that other FDMs do not possess. By using ECS to organize data, we can decouple functionality from the actual data in memory, as opposed to having functionality and memory tied together in hierarchies. This is a powerful and popular strategy in designing complex video game engines, which need to process updates to large amounts of game entities dealing with large amounts of data. This decoupling allows for the methods that operate on game entities to be easily parallelized and fast. In addition to performance benefits, because ECS decouples code, the ability to modify existing functionality or entities, or add new functionality or entities is a much smoother process compared to OOP. This will be explained in more detail in the next chapter. So, for the same reasons game designers choose to use the ECS design in a video game, we can also benefit by using the ECS design in our FDMs.

## 1.4 Contribution

The contribution of this thesis lies in the field of flight modeling and simulations. The successful build of a Rust-based, ECS designed, real-time FDM demonstrates an alternate, modern, and powerful design strategy over current FDMs. Not only is this strategy available for use, but it is useful to increase performance through efficient memory-management and parallelization. This strategy also increases code maintainability due to the decoupling nature of the ECS architecture; this is a valuable feature that could save time and effort when maintaining the codebase later in its lifespan.

## 1.5 Approach

The methodology of this research is explained in full detail in Chapter 3, but an

overview of the approach will be given here.

The focus of this research is building a Rust-based, ECS-based, 6-DOF FDM based on the works by David Bourg in [14]. However, a 4-DOF, Rust-based, ECS-based FDM is also created based on the works by Grant Palmer in [30]. The building of this 4-DOF FDM was a stepping stone to reach the goal of creating the 6-DOF version.

In terms of ECS compatible aspects, the 4-DOF FDM is composed of one Component and one System, which play a part in the airplane Entities data storage and functionality, respectively. The Component, which contains all of the data used by the Systems is: DataFDM. The System, which adds functionality to the Component data of the airplane Entities, is: EquationsOfMotion. With the 4-DOF FDM built, to ensure its accuracy of modeling flight, we will find out if it replicates the realistic flight data generated by its benchmark FDM as described in Palmer’s textbook.

Similar to the 4-DOF FDM, the 6-DOF FDM is composed of one Component and one System. The Component is: DataFDM. The System is also: EquationsOfMotion. With the final 6-DOF FDM built, to ensure its accuracy of modeling flight, we will find out if it replicates the realistic flight data generated by its C++ benchmark FDM as described in Bourg’s textbook.

With the two FDMs built in this thesis, three additional Systems were built for each in order to support flight control via keyboard input, and networking to provide graphics within the FlightGear visualization system. The three Systems are: FlightControl, MakePacket, and SendPacket. The Systems operate on the Components named: KeyboardState, DataFDM, and Packet. These Systems and Components that support keyboard input and graphics, when added in conjunction with the EquationsOfMotion System used by the FDM, construct an interactive flight simulator.

## 1.6 Assumptions/Limitations

The first limitation is that the FDM built is not a standalone application; a graphics generation program is needed to display the resulting simulation. However, if a user does not require graphics and only the flight data text output is needed, the FDM can be executed without an external visualization system. But otherwise, the FDM is only configured to work with FlightGear as the visualization system. A specific packet structure is required by FlightGear to be able to interpret the data, however, the structure could be retrofit for different visualization systems.

We assume the simulation is running on a modern computer. The simulation loop is designed to run at a fixed frame rate, and a very slow computer may not be able to keep up with the predetermined frame rate of 30 Frames Per Second (FPS).

Another limitation is that the airplane being simulated is only set up to match that of the airplane modeled in Bourg’s textbook. This no-name airplane is shown in all of the figures shown in the next chapter. The airplane characteristics could be modified in the code, however.

The airplane is set to fly at a predetermined location. And the latitude, longitude, and elevation of a desired location is needed to fly there, along with plugging in those values to the starting position coordinates.

## 1.7 Thesis Overview

This thesis is arranged into five chapters. Chapter II explains relevant background information on the physics of flight modeling, Entity Component Systems, Rust programming, FlightGear, and networking with Rust. Chapter III provides the thesis methodology. Chapter IV is a results analysis of Chapter III. Finally, Chapter V summarizes the research and impact of the research.

# II. Background

This chapter will provide an overview of the components that have gone into building this FDM. First, the background explains the basics of modeling flight physics in a FDM. Next, the ECS architecture, and an implementation of it, SPECS, will be described. Furthermore, the fundamentals of the Rust programming language will be laid out. Next, the use of the FlightGear flight simulator will be explained. Lastly, networking and sending packets of data to FlightGear with Rust will be described.

## 2.1 Physics of Flight Modeling

A flight dynamics model is defined as the set of math equations that compute all of the physical forces on a rigid-body airplane, like thrust and gravity [33]. A rigid-body can be defined as a body made up of particles that stay fixed and do not rotate or translate relative to the other particles. In this thesis, we focus on a FDM that approximates the physics of a rigid-body airplane. Along the way, we will point out the differences between the 4-DOF FDM and the 6-DOF FDM because they both have different considerations when determining an airplanes position and orientation, which we will describe – although they are mostly similar in how they get to a final result. Regardless, the final result is integrating the equations of motion. This boils down to determining the position of the airplane, as well as the airplane’s velocity, which is the speed and direction of the airplane, over time [20]. But before the equations of motion are solved, an understanding of DOF, forces, airplane components, rotations, and coordinate systems is needed.

### 2.1.1 Degrees of Freedom

Flight dynamics is the science of air vehicle orientation and control in three dimensions [35]. Flight dynamics models define equations of motions to account for the various forces (e.g., engine thrust, gravity, etc.) acting upon it to determine an airplane’s position and orientation (i.e., translation and rotation). Three degrees of freedom are associated with translation (e.g., x, y, z) and three are associated with rotation (e.g., pitch, roll, yaw) for a total of six possible degrees that could be calculated starting with initial forces.

For example, the equations of motion for a particular FDM might only compute translational dynamics - which would classify it as a 3-DOF model. A more complete model that computes an airplane’s translations and rotations associated with external forces would be classified as a 6-DOF model.

For this research, two OOP-based flight dynamics models are re-implemented using the Rust programming language and organized as Systems and Components in the ECS architecture. Palmer [30] defines a relatively simple 4-DOF model that considers two degrees associated with translation, and two degrees associated with rotation. It is the case that Palmer’s 4-DOF model does not support the remaining 2-DOF that allows translation on the y-axis and that allows yaw rotation. Whereas Bourg [14] provides a more complex 6-DOF model.

### 2.1.2 Forces

There are four major forces that act on an airplane in flight: gravity, lift, thrust, and drag. These forces govern the motion of the airplane. Newton’s Second Law of Motion states: the acceleration of a body is proportional to the resultant force acting on the body. The resulting equation is F = m * a. Gravity pulls the airplane to the ground and lift is generated by the airplane wings to counteract gravity to keep the airplane aloft. Thrust is generated by the airplane’s jet engine or propeller to increase velocity and to keep the airplane generating lift. Drag is the opposite of thrust; drag impedes an airplane’s motion. Torque, also known as moment, is similar to force, but force is what causes linear acceleration, and torque is what causes rotational acceleration. In order to fly, an airplane must have a lift force acting upon it greater than or equal to gravity. Figure 1 is a visual of the four forces.

Figure 1: Forces Visualized [14]

### 2.1.3 Airplane Components

An airplane is composed of different components that affect the state of the airplane. The wings are the rectangular parts coming out of the main body. The span is the length of the wing, whereas the chord is the width of the wing. The ratio of span squared to wing area is called aspect ratio. The ailerons are on the outside end of the wings, and the flaps are on the inside of the wings. The elevators are on the tail, and look like small flaps. The rudder is the vertical flap on the tail. Figure 2 shows these airplane components.

Figure 2: Airplane Components [14]

#### 2.1.3.1 Airfoil

The cross-sectional structure of the wings component, called an airfoil, plays an important role in how the lift force acts on the airplane. Camber on the airfoil represents the asymmetry between the top surface and bottom surface of the airfoil which creates a curvature. The angle of attack is the angle between the chord line and the direction of travel, or oncoming flow that the plane is moving through. The total lift force produced on the airfoil is caused by camber and the angle of attack [14]. Figure 3 shows a labeled cross-section of an airfoil with an angle of attack [23].

Figure 3: Airfoil Cross-section

### 2.1.4 Rotations

The orientation of the airplane when it is rotating can be described by three angles: pitch, roll, and yaw. These angles are called the Euler angles and represent the x, y, and z axis, on a coordinate plane, respectively. When an airplane is flying using Palmer’s model, it is can only rotate on two axes relative to the pilot. It can pitch, and roll, while yaw is fixed. On the other hand, with Bourg’s model, the airplane can rotate about all three axes relative to the pilot. The ailerons create the roll rotation. The elevators create the pitch rotation. The rudder creates the yaw rotation. The flaps do not rotate the plane, but they do affect lift by altering the chord and camber of the wing, which has to do with the shape of the cross-section of each wing. Figure 4 shows the three types of airplane rotations.

Figure 4: Airplane Axes of Rotation [14]

### 2.1.5 Coordinate Systems

Coordinate systems are important to understand when determining the location of the airplane on the Earth. Earth-Fixed, Earth-Centered (ECEF) coordinates tell us where the plane is in relation to a fixed point, which is the center of the Earth. This is the coordinate system, represented by Cartesian coordinates (x, y, z), native to Palmer’s 4-DOF FDM [25].

A geodetic coordinate system uses latitude, longitude, and altitude to represent location [25]. ECEF coordinates calculated by the 4-DOF FDM need to be transformed into geodetic coordinates. To perform the transformation, a Rust crate, named coord transforms, is used in the 4-DOF FDM. Figure 5, in the bottom-right of the figure, shows the ECEF coordinate plane that is relative to that point.

Along with coordinates that describe location, called the Earth space, another coordinate system exists to describe the frame of reference in order to track all 3-DOF rotations of the airplane. It is called the body Space, and it is an x, y, z coordinate system like ECEF. But unlike the Earth space, which is fixed somewhere on the Earth, the body space is fixed to the center of gravity of the rigid body airplane, so it moves with the airplane [20]. Rotations happen along each of the three x, y, z axes. Figure 5 also visualizes the body space coordinate system representing rotations about the fixed point on the center of gravity of the airplane.

Figure 5: Earth Space and Body Space Visualized [21]

### 2.1.6 FDM Calculation Steps

With an understanding of the airplane components, airfoils, forces, rotations, and the coordinate systems, we can begin to explain the steps the FDM takes:

- The mass properties of the airplane are calculated

- The load, or forces and moments on the airplane are calculated

- The equations of motion are integrated

#### 2.1.5.1 Mass Properties

The mass properties involve calculating how the mass and physical components of the airplane will affect the forces acting upon it. To begin, one must decide which airplane they are trying to model. In Palmer’s model, the Cessna 172 is being modeled. In Bourg’s model, an actual airplane is not being modeled, but the model representation is that of the picture of Figure 2. With that, the mass properties are chosen to represent that desired airplane. For example, the airplane’s total wing area will affect both lift and drag forces, and total mass will affect gravity.

In terms of the 6-DOF FDM, mass properties become more complicated. The rigid body airplane’s center of gravity needs to be calculated, along with the airplane’s moment of inertia tensor. The center of gravity, or center of mass, is where the forces are applied on the airplane. The moment of inertia is essentially the airplane’s resistance to rotation. It can also be described as a distribution of mass. It is the amount of torque needed to make the airplane rotate about an axis relative to another point (i.e. center of mass). The measure of how fast the airplane actually rotates is called angular velocity [22]. The moment of inertia tensor is the mathematical expression, represented as a 3x3 matrix, that has magnitude and direction, and holds the values of the moments of inertia as body coordinates [14].

#### 2.1.5.2 Forces

With mass properties handled, the forces and moments acting on the airplane at all times can be calculated. Lift and drag are not as simple to calculate compared to gravity and thrust, but we will simply explain the general steps. The lift and drag forces are calculated by first determining the direction in which they will act. Next, based on air speed, angle of attack, and the airfoil performance data, the total lift force coefficient of the airplane is computed. Although there are many components on the airplane that affect lift and drag, the total lift force on an airfoil is mainly composed of two components: the lift due to camber and the lift due to attack angle. The airplane’s flaps are used to alter camber to create lift. Usually the flaps are used at low speeds to increase lift. Drag increases with increased angle of attack. If an airplane is flying at an angle of attack high enough, it will not be able to create any lift and will stall. Figure 6 is a visual that shows lift coefficient versus attack angle, and that when attack angle is increased, lift increases. However, at some point a stall occurs with lift dropping off rapidly. Also, shown in Figure 6 is that with flaps lowered, lift is increased.

Figure 6: Lift Coefficient vs Angle of Attack [14]

There are also other variables that affect lift, and that are considered in the FDM, such as air density, component surface area, and wing aspect ratio. External forces can also exist that can act on the airplane such as winds, which are not considered in the FDM.

Gravity is simply based on mass of the airplane and the acceleration of gravity, which is a constant -9.8 m/s2. So, gravity can be calculated by the mass of the airplane multiplied by -9.8 m/s2, which is added to the force acting on the z axis (down). Similarly, thrust, expressed in pounds, is force added to the x axis (forward). Without thrust propulsion, the airplane cannot make lift.

#### 2.1.5.3 Equations of Motion

With forces considered, the equations of motion are integrated so that the simulation progresses through time. This begins by calculating the updated position of the airplane, as well as the airplane’s velocity in each step of time. In computer simulations, the equations of motion are approximated by solving nonlinear ordinary differential equations using numerical integration algorithms [20]. The integration of the equations of motion are considered an approximation because there is always some error introduced depending on the technique chosen [14]. Simply put, the integration of the equations of motion, given everything else talked about, approximates the future velocity and position based on the previous values from the last frame of the simulation [26].

The amount of finite time chosen to represent a frame (i.e. delta time) in the real-time simulation loop is important in numerical integration. A correctly chosen time step will result in stable numerical integration. Stability has to do with how closely an approximate solution converges to the exact solution [14]. A smaller time step will give a result closer to the exact solution but this also means that there are more steps and a larger buildup of numerical precision error. On the other hand, there is a breaking point when the time step is too big where the simulation crashes [27]. With that said, there is a practical balance as to what the time step should be to achieve a stable solution, and also to minimize the buildup of error [14].

There are multiple methods to actually implement numerical integration to calculate the equations of motion. Using Euler’s method is by far the most common numerical integration technique [20]. In both the 4-DOF and 6-DOF model, we use an improved and more accurate version of Euler’s method, called the Runge-Kutta method, which is laid out as code in Bourg’s textbook and Palmer’s textbook.

An important role of the equations of motion is that, in the 6-DOF model, the orientation of the plane is tracked with the use of a quaternion. A quaternion is a way to represent orientation of something about three axes. Quaternions get updated with new Euler angles in each time step to reflect the new orientation using the angular velocities of that instant [14].

##### 2.1.5.3.1 Simulation Loop

Another thing to think about when choosing a time step is that there are different design strategies to actually employing the simulation loop. The easiest way to manage the simulation loop is to just loop as fast as possible with no time step. The problem with that is that there is no control. If you are on a fast computer it the simulation will go too fast, and if you are on a slow computer it will be too slow [32]. But, as stated before, this is not how we want to run the simulation loop because we want a fixed time step. The next easiest way to manage the loop, and the strategy we chose, is to have a fixed time step. This is ideal when it is known that the update loop takes less than one frame worth of real time [27]. For example, if the time step chosen is 33 milliseconds, then we need to reliably know that the real processing time will take less time than that. And for the remaining time the program will go to sleep until it is time for the next update [32]. The more advanced way to employ a simulation loop is to have a variable time step. It involves choosing a time step based on how much real processing time the current update takes versus the previous update’s real processing time [32]. This strategy, however, introduces some issues, which we will not get into.

### 2.1.7 Fidelity

With the FDM described, fidelity is the degree to which the characteristics of a flight simulation matches that of the real world [17]. In this case, we characterized this FDM as medium “fidelity,” because the characteristics of the flight simulation match that of real life, but are not computationally expensive, and require minimal data [18]. One may think that a flight model with the highest fidelity, or accuracy, is the best – this is not always true. Advanced flight modeling will require large, detailed sets of data about the stability and performance of an aircraft that it is modeling. Wind tunnel data analysis is an example of the kind of data needed for a high fidelity flight model. This data is not readily available and often costs hundreds of thousands of dollars [13].

## 2.2 Entity Component System

The FDM will be organized, and re-implemented in code as a DOD, ECS-based architecture. ECS is a way to organize data, and a viable alternative to organizing code with OOP. The ECS design is popular in designing game engines. The Unity game engine, for example, uses the ECS design pattern [8]. ECS focuses on the composition of data, whereas OOP focuses on inheritance and hierarchies. ECS gets rid of the troubles that arise with inheritance, and improves upon performance and ease of code development.

### 2.2.1 Object-Oriented Problem

Suppose we have a space or world that we want to populate with things. These things may have similar attributes as well as unique attributes. With OOP, you might have a base class in which everything is derived, called Thing. The limitation is that one child class can only inherit directly from one parent. For example, a Mammal type may have a child Dog which inherits a fur attribute, and a Bird may have a child Duck which inherits the lays eggs attribute. But in this case, a platypus, which has fur and lays eggs needs to inherit both attributes [9]. This complicates things, and becomes a larger problem with a larger hierarchy. The hierarchy chains that are created in game design can become too large to work with and code becomes too highly dependent on other code [10]. This makes code readability, maintenance, and the addition of features a pain with OOP. However, this is where ECS shines.

Along with code flexibility, performance can be an issue with the OOP design when talking about memory-management in terms of the cache. When a class instance, or object, is instantiated, it is not most efficiently put into cache. When an object is created, all its data is put into cache right next to all the other object’s data. This is a problem because if a thread shares an object’s data, there is not always enough room on the cache for all the data that the thread actually wants to work on. And when unneeded data is in cache, the chance of cache misses happening increases [34]. This is bad for optimal performance and ECS improves upon this.

### 2.2.2 ECS Explained

In an ECS structure, because it is based on the DOD paradigm, you do not have hierarchy levels – you have these pieces called Entities, Components, and Systems. You start with an Entity, which represents concrete objects in the world, like a bullet. Entities on their own are useless; Entities are just a unique id used to access data elsewhere [34]. Components hold the Entities data, commonly stored as a vector in memory, and represent a feature. For example, Position is a Component of an Entity. The Components called Position, Render, and Spawning may be added to the Bullet Entity. The mixing and matching of Components can create unique Entities. A Player Entity can be created by adding the Components: Position, Render, and Health. Systems are what give functionality to an ECS architecture. A System could be created called UpdateRender, which handles all of the rendering functionality of the world. Or a System called UpdateHealth could handle all of the health functions. Systems have something called a filter. Filters look for every Entity which has some combination of specified Components. For example, the UpdateRender System’s filter wants to find every Entity with the Render data Component and Position data Component. Once these Entities are found, they are returned to the UpdateRender System to perform some job on it [11]. This is this example visualized:

Figure 7: Entity Component System Visualized [11]

As seen in Figure 7, the Systems are separate from the Components. The Systems, when performing a job, does not care what Entity it is operating on. The whole point is that the data Components are completely separated, or decoupled, from the Systems functionality [12]. This separates code dependencies, making it simple to add new Systems or Entities without needing to tweak anything else in code – a huge benefit that OOP does not offer. This decoupling also makes it simple to implement concurrency where a multiple Systems can be working on an Entity concurrently.  Also, because a System does a job on specified data Components, it only iterates over Entities that have the necessary Components.

The ECS design also optimizes performance by more efficiently managing memory. It does this by laying out the data in cache more efficiently than the OOP design does. The goal for any software is to retrieve data from cache as quick as possible, and this is an aspect that is easily overlooked when choosing a certain design approach to a program because it happens in the background. But ECS, minimizes cache misses by not having data organized by classes, and instead having data broken into smaller chunks – the data Components, which are stored as a vector. So, for example, the Render System will only need the Render and Position data for all the Entities. Then, only that required Component data that is needed to perform the System is pulled into cache for that Entity being worked on. This is how the cache holds only what it needs, and does not bombard itself with all the data that an entire class in OOP holds. The vector which stores the Component data in the cache is guaranteed to hold data in each index. This is good for performance because no time is wasted determining if an index has null data, and therefore pulling more data into cache. [34].

### 2.2.4 SPECS Framework

SPECS is an implementation of an ECS. SPECS is developed by and used by Amethyst, which is an open-source game engine written in Rust [12].

### 2.2.5 ECS Overall

The ECS architecture has proven itself as a dominant strategy in improving game programming. It allows simple parallelization of complex Systems, simple code modification, and quick accesses to the data that matters. With all of these useful attributes, however, ECS does come with some downsides. To go from coding OOP to ECS, one must change their way of thinking because there is no inheritance, which most programmers are already familiar with. Also, planning ahead when coding in ECS is a necessity; one must know what systems will use what components, and changing components after the fact is not ideal. Since Systems are sometimes dependent on each other to complete, it may be difficult to add another System in the middle of two consecutive Systems. Planning ahead could help to avoid that situation. Overall, ECS introduces some nice benefits once the programmer wraps their head around it.

## 2.3 Rust

Rust is the programming language being leveraged to build the flight dynamics models. It is the language of choice given its advantageous features: memory safety and performance. Rust was released in 2015 by Mozilla, which has been behind the language’s success. Rust will guarantee that a program will be memory safe, with no invalid data accesses, all while remaining just as fast as other popular programming languages [1]. The C++ programming language, for example, is a systems level language, giving developers complete access to data inside memory, with no memory safety precautions. This is only great when the programmer is an expert on writing code that is free of memory issues – this is hard to do. On the other hand, the Python programming language takes away a developer’s access of low level memory, but does provide complete memory safety by using a garbage collector to clean up unneeded memory – but this imposes a runtime cost [1]. Rust is unique because it is a systems level language, it does not have a garbage collector, and it will still guarantee memory safety. This means Rust provides control, is fast, and protects against memory issues such as data races, and dangling pointers [1]. These features built-in to Rust make it the choice programming language for building a FDM.

The safety assurance of Rust is accomplished by its system of Ownership, Borrowing, and Lifetimes. All languages have a way to manage memory while a program is running. Some languages have a garbage collector which actively checks for memory that is no longer being used during runtime. Other languages make the programmer manually allocate and then free memory. Rust takes neither approach; it instead has something called a “borrow checker.” At compile time, the Rust borrow checker will check that all accesses to data is legal and valid [1]. The borrow checker deals with three important Rust concepts: Ownership, Borrowing, and Lifetimes.

### 2.3.1 Safe Rust and Unsafe Rust

Before we begin to discuss what specific features make Rust a “safe” programming language, it is important to know that there is an “unsafe” side of Rust, like C++. By safe, it is meant that the code is completely bug free of memory issues, and undefined behavior such as data races (two processes writing data in the same memory location at the same time), and dangling references (accessing unintended data). The unsafe side of Rust will let the programmer do things that are not safe. And sometimes it is necessary to do unsafe things. For example, sometimes it is required to go into unsafe mode to dereference raw pointers, which are possibly invalid, or out of scope at access [24]. In safe Rust, this is impossible to do because of the protection feature implemented called Lifetimes. The other implemented features, Borrowing and Ownership, also empower safe Rust. We will discuss these features in this chapter now. Overall, the great thing about Rust is that it allows the programmer the choice to write code in safe mode or unsafe mode, whereas in other languages, that mode is unchangeable.

### 2.3.2 Ownership

Ownership refers to the fact that every use of a value needs to be valid and cleaned up when it is done being used. Ownership is accomplished with three rules that the programmer must follow in Rust [2]:

- Every value has a variable which is called its owner

- There may be only one owner at a time

- When an owner goes out of scope, the value is dropped

### 2.3.3 Borrowing

Borrowing is the next important concept in Rust. Borrowing allows data to be “borrowed” from one owner (variable binding) to another owner and accessed for a moment without actually changing Ownership of that data. When Ownership of memory is transferred to another owner, the original owner cannot legally be used, so Borrowing is the solution. The borrow checker will statically guarantee that references always point to valid objects. So, if a reference to an object exists, it cannot be destroyed. There are two rules to Borrowing [2]:

- Any borrow must last for a scope no greater than that of the owner,

- You may have one or the other of these two kinds of borrows, but not both at the same time:

- One or more references to a resource

- Exactly one mutable reference

These rules avoid the possibility of a data race because although there can be more than one pointer to a resource, only one can be accessing and changing that resource at a time. This is good for performance, and uses less memory overhead [1].

### 2.3.4 Lifetimes

The next important Rust concept is Lifetimes. A value has a Lifetime, which is the period that accessing that value is valid to do [2]. A Lifetime starts when a value is created and that Lifetime ends when it is destroyed. Rust’s compiler, or borrow checker, is smart enough to know a values Lifetime in many common circumstances [3]. In these common circumstances, the Lifetime of a value does not need to be explicitly written, but otherwise, a Lifetime parameter needs to be written. The borrow checker tries to limit the period of a Lifetime. A value should not be alive any longer than it absolutely needs to be because, otherwise, it could be referenced by accident. This could cause a dangling reference, which is a reference to unintended data [1]. So, to keep some piece of data alive for only as long as completely necessary, a Lifetime is specified for that data.

### 2.3.5 Crates

Crates are essentially libraries that can be added to a Rust project for additional functionality, written by public users. Crates do not come pre-packaged when Rust is installed on a machine, they must be manually installed. In this thesis, we use multiple crates to code the final result, but the most notable crate is SPECS, which is a framework to easily program the ECS design in Rust.

The Rust community is also active and quickly growing, with now over 46,000 crates available to download [19]. Rust makes it easy to import these crates into code and begin using them, and the crate to implement ECS into code, SPECS, is no exception. Only two steps are needed to get started with a crate. First, add the crate dependency to the cargo.toml file, which is in every Rust project. Second, add a line at the top of the main.rs to indicate that this crate is being used. Now the desired crate can be utilized in code.

### 2.3.5 Rust Overall

Overall, Rust can hold its own against more popular languages, and the learning curve of the language is met with staggering benefits in safety and performance. And with safety dealt with by the borrow checker at compile time, the programmer does not need to be concerned with issues of memory. At the same time, there is no cost in performance at runtime because everything is naturally checked at compile time without a garbage collector. The downsides to Rust are minimal, but they do exist. For one, compile time is longer. Also, the compiler is very strict. New Rust programmers often “fight with the borrow checker” to get their code to compile because it is more difficult to write code that abides by Rusts rules [1]. So less time is spent fixing bugs, and more time is spent getting the code to compile. But the compiler is like a safety net because once the code compiles, the programmer knows the result is safe and free of memory errors.

## 2.4 FlightGear

FlightGear is an open-source flight simulator application. However, in this thesis, it is being used as a standalone visual system. To do this, it supports a network protocol that defines airplane position as geodetic coordinates and airplane orientation as Euler angles within a simulated world. FlightGear, although it has its own built-in FDM, is easily configurable to instead use an external, custom FDM. FlightGear can be configured to be interfaced, or communicated with, via network packets to achieve a desired display output. In this case, these packets of data will be sent from the ECS patterned, Rust application to FlightGear and the packets will contain the position and orientation. FlightGear reads these packets and displays the airplane visuals.

To properly configure FlightGear for the simulation, it must manually be told to accept the new external FDM, as well as listen for the respective packets [5]. This is done from the command prompt upon running the executable. The command line arguments are listed in Appendix A: Configuring FlightGear as a Visual System.

With FlightGear configured, it will process the received network packets constructed by the Rust application. But FlightGear requires that these packets are defined in a particular format so as to understand them. The format of the packet, before being converted to bytes, must be a struct, which is outlined in the FlightGear source code. The struct contains all of the data needed to define an airplane and how it flies throughout the simulation. This struct, called FGNetFDM, in Rust code is located in Appendix A: Configuring FlightGear as a visual System.

## 2.5 Networking with Rust

Networking is used to send packets of data to FlightGear so that it can display the desired simulation. To communicate with FlightGear, two sockets are required at each end of communication. In this case, the type of socket being used is the Datagram socket. The protocol involving Datagram sockets is called the User Datagram Protocol (UDP). This protocol is considered “connectionless” because an open connection does not need to be maintained. Packets containing bytes of data may simply be built and sent to a destination without a connection needing to be established prior [7]. FlightGear, when commanded on startup, is able to open a socket to receive UDP packets on.

In Rust, a UDP Socket is created and binded to an internet protocol address and port on the local machine. After this, data can be sent and received from any other socket address. The socket created can then be connected to the other socket with its internet protocol address and port. From here, data can be sent, and received with a function call [2]. One important thing to be aware of is that before data packets are sent out to FlightGear, the literal order of the bytes need to be converted from least significant byte first (little endian), to most significant byte first (big endian) [7].

# III.  Methodology

This chapter will discuss the research methods used in this thesis, including what has been accomplished and how it has been accomplished. We will describe the process of re-implementing Palmer’s FDM and Bourg’s FDM with SPECS, and how we verified that they do work as expected. We will then explain the how the FDMs were set up as flight simulators.

## 3.1 Prerequisites

The approach to building the Rust-based, ECS-based FDMs began with acquiring an understanding of basic flight physics and the Rust programming language. Starting with zero physics experience, a general knowledge of flight physics was needed to be able follow along with what was going on in the FDMs. Both of the textbooks describing the FDMs, Physics for Game Programmers by Grant Palmer and Physics for Game Developers by David Bourg, were the primary references, and were easily digestible.

In regard to programming in Rust, the two most utilized resources for writing code in this thesis was the official Rust book, The Rust Programming Language, and the official code collection called Rust by Example, which showcases executable code snippets. The first task in Rust for this thesis had to do with networking. The task was to send a UDP packet in Rust to FlightGear to make it “do something”. To help accomplish this, FlightGear’s GitHub repository offers a program that does exactly this in C++. This task taught Rust basics as well as the networking needed for this thesis. But Rust proficiency skyrocketed when beginning to write the FDMs from their native languages into Rust. Being familiar with Python, Java, and C++ helped immensely in learning another language.

In regard to ECS, it was not a focus until after the FDMs were working in Rust. Ideally, one would begin programming their software from the ground up as an ECS, but it was simpler to convert the code directly from the FDMs native language to Rust in the same OOP design before it would be broken up into Systems and Components representing the ECS. But once it was time to convert the FDMs into an ECS, there were very limited resources available describing the process of writing code as an ECS, especially in Rust. The ECS resources used in this thesis boil down to YouTube videos sponsored by Unity, a high-profile proponent of the architecture, and The Specs Book, which is the official documentation on the SPECS framework for Rust.

## 3.1 Building the FDMs

The first FDM built was the 4-DOF version because it was inherently less complicated because the physics are simpler. Our 4-DOF FDM is a re-implementation of the 4-DOF FDM described in the textbook, Physics for Game Programmers, by Grant Palmer. The textbook provided the C code for his FDM. This FDM was ported from C into Rust. From here, the FDM was restructured into an ECS design using the SPECS Rust crate, which made this process straightforward. The building of this 4-DOF FDM was an important stepping stone to reach the goal of creating the 6-DOF version.

The 6-DOF FDM is based on the works by David Bourg and his textbook, Physics for Game Developers, which outlines a 6-DOF FDM in C++ with an OOP design. Similar to before, this C++ FDM from Bourg was meticulously re-implemented into Rust. From here, it was structured into an ECS design by using SPECS.

Although the FDMs have fundamental differences in what they consider when calculating the equations of motion, they are similar in many ways. Specifically, they are similar in terms of their organization with SPECS and the Systems and Components that they consist of. This section will describe the FDMs general similarities in their construction.

SPECS laid the framework to create the Entities, Components, and Systems. For both FDMs, they consisted of only one Component and only one System. However, later in this chapter, we will describe the Systems and Components added to the FDM create the interactive flight simulator Components were created similarly to how a regular Rust struct type is created, but the struct created is specified to be implemented as a Component using a vector as its storage type. Systems were created similarly to functions, but when created, it was specified what Components were needed by the System to access. This is the specification of Components for the Systems that determined what Entities the System would work on. Although, in this thesis’ case, only one Entity is used for an FDM.

The Components specified for a System were given either WriteStorage access or ReadStorage access, meaning they can be mutated or not, respectively. This ReadStorage or WriteStorage access type is important because the set up determines what Systems may be run in parallel with each other. Only one WriteStorage access may be used by a System at a time in order to avoid data races during parallel execution. So, if two Systems require the same WriteStorage access to a Component resource, they must only be run sequentially as to not write over the same memory [12]. But if two Systems require the same Component, and one System needs WriteAccess, and the other System needs ReadAccess, the Systems can still run in parallel. We will explain how this plays into the Systems created for the flight simulator later in this chapter.

With all that said, ECS made the organization of FDMs simple – there is a single System and a single Component. The main files of both the FDMs were built similarly. The SPECS crate was imported, along with the modules containing the System and the Component. In the main function, the simulation world was created. The Components were registered into the world. A delta time was calculated based a desired FSP. This delta time was added as a resource for the Systems to use. A dispatcher was opened to manage the execution of the Systems. Then, the airplane Entity was built with predetermined starting values, like thrust or throttle. Finally, the main simulation loop was made to process each frame as fast as possible because the simulation pace is not a factor in the raw FDM, but it is a factor when using it as a flight simulator.

## 3.2 4-DOF Set Up

## 3.3 6-DOF Set Up //bourg considerations

Bourg defines a series of classes in C++ that are used to represent a custom vector, quaternion, and matrix. These classes were not originally planned to be used because Rust has a crate available called nalgebra that already defines those structures, and we did not want to re-implement them not necessary. However, to no avail, we fixed problems with rotations by staying true to the Bourg model. So, the custom classes defined by Bourg were reinterpreted in Rust and used in our 6-DOF FDM. The module that was imported in code which contains the custom structures is named common.

//use resources and rewrite this
//this is flight sim stuff

To ensure numerical stability, the time step chosen for the simulation loop inside the main of the FDM was 33 milliseconds. This means that the simulation steps through 33 milliseconds of time from frame to frame. This equals 30 frames per second. With this time step, the simulation is stable. This simulation loop strategy makes frames per second dependent on a constant game speed (33 ms) [28]. This basically means we have a fixed change in time from frame to frame. This strategy was chosen for simplicity sake. This strategy works because we know that the actual integration of the equations of motion reliably takes less time than to update than the chosen time step, and so the simulation will be able to keep up, and maintain a smooth frame rate [27]. This strategy might normally be an issue for slow computers, but the FDM is not very demanding on hardware. The downside is that for fast computers, we leave some frames per second on the table because we have a fixed time step [28]. In the main, there is a variable in the main called frame_rate, representing FPS, which may be modified to a realistic value. There is also a variable representing the time step, called dt (Delta Time). The dt is dependent on the FPS, and is calculated by dividing 1.0 by the FPS. The dt is added as a SPECS resource, which is data able to be shared between Systems [12]. Resources are useful to avoid global variables.

//make into resourceThe other global variables that may be modified pertain to thrust. The maximum thrust, MAX_THRUST, is set at 3000.0 pounds of force, but is able to be altered. Also, the delta thrust, D_THRUST, which represents the amount of thrust incremented when increasing or decreasing thrust with keypresses, is able to be altered. D_THRUST is set at 100.0 pounds of thrust per keypress.

//talk about components and system only needed for ecs fdm

## 3.4 SPECS FDM Component

### 3.4.1 DataFDM

## 3.5 SPECS FDM System

### 3.5.1 EquationsOfMotion

## 3.5 4-DOF FDM Equivalency Verification

40 seconds, what we can play with in his mdoel aoa, bank,  each aspect is tested

### 3.5.1 Test One (Throttle)

This test was designed to find out if the FDM works as expected when simply accelerating the airplane with the throttle fully applied, and therefore with the maximum thrust being applied. Every other instruction is not activated.

Flight Control Instructions

Throttle (%): 100

Angle of Attack (deg): 0.0

Bank (deg): 0.0

Flaps (deg): 0.0

Table 4: Flight Control Instructions for Test One

### 3.5.2 Test Two (Angle of Attack)

This test was designed to find out if a positive degree in angle of attack while the airplane is accelerating at full-throttle will work as expected.

Flight Control Instructions

Throttle (%): 100

Angle of Attack (deg): 4.0

Bank (deg): 0.0

Flaps (deg): 0.0

Table 4: Flight Control Instructions for Test Two

### 3.5.3 Test Three (Bank)

This test was designed to examine if the bank action works properly. The airplane is accelerating at full-throttle. The degrees of bank will be set to 10.0, which will bank the airplane to the right. The angle of attack will be set to 10.0 degrees. //talk more about the range of each value here?

Flight Control Instructions

Throttle (%): 100

Angle of Attack (deg): 10.0

Bank (deg): 5.0

Flaps (deg): 0.0

Table 4: Flight Control Instructions for Test Three

### 3.5.2 Test Four (Flaps)

The final test for this FDM examines the deflection, or lowering of the flaps on the airplane. The flaps can either be completely down, or at two other angles: 20 degrees, and 40 degrees. In this test, we find out if the flaps work properly when they are deflected at 20 degrees. The airplane will be accelerating at 100%.

Flight Control Instructions

Throttle (%): 100

Angle of Attack (deg): 4.0

Bank (deg): 0.0

Flaps (deg): 20.0

Table 4: Flight Control Instructions for Test Four

## 3.5 6-DOF FDM Equivalency Verification

Since the focus of this research was building a Rust-based, ECS patterned, 6-DOF FDM based on the works by David Bourg in [14], we needed to find out if it indeed worked as expected. When flying the airplane in FlightGear, from the naked eye, the flight mechanics look to be correct. But to ensure our FDM’s works properly, we investigated if it replicates the same realistic flight data generated by its C++ benchmark FDM as described in Bourg’s textbook. This investigation ran a series of different flight tests between both FDMs with the objective to check if our FDM’s generated flight data was equivalent to the benchmark in multiple scenarios. The flight data used in the comparison was: position coordinates, Euler angles, and speed. This data represents exactly where the airplane is at a given time, and how the airplane is oriented. The code used to perform the tests did not involve any of the code required to interactively fly in FlightGear, and dismissed FlightGear entirely.

For each set of tests, both the benchmark FDM and our FDM was compiled and ran with an identical airplane flying under the same starting parameters to get some output. The initial parameters include: coordinate position, speed, velocity, forces, and execution time. The starting parameter values chosen to be used in these tests are the same values that Bourg sets in his FDM source code. They were chosen because the values are set up so that the plane is in motion as soon as the simulation is started – with velocity, force, thrust, and speed defined greater than zero as to have the airplane flying forward. The starting position coordinates represents a location relative to a fixed point (0.0, 0.0, 0.0). The distance from that fixed point is measured in feet. For example, the starting z coordinate of 2000.0 represents height, or altitude, in feet from the ground. Execution time of the tests, although not a variable that affects an airplane’s motion, is defined to be 30 seconds of simulation time, which equates to 900 total frames, or updates, at 30 FPS. The starting flying values chosen while testing the C++/OOP FDM and the Rust/ECS FDM are:

| Variables | Initial Values |
| --- | --- |
| Position Coordinates (x, y, z) | -5000.0, 0.0, 2000.0 |
| Velocity Vector (x, y, z) | 60.0, 0.0, 0.0 |
| Force Vector (x, y, z) | 500.0, 0.0, 0.0 |
| Thrust Force (pounds) | 500.0 |
| Speed (knots) | 60.0 |
| Execution Time (s) / Total Updates (frames) @ 30 FPS | 30 / 900 |

Table 5: Initial Parameter Values for Each Test

For each set of tests, the two FDMs were given the same flight control maneuvering instructions for the airplanes to follow. The possible flight controls we test include: Roll, Pitch, Yaw, Flaps, and Thrust. Each flight control was examined in each of five tests. With one more test simply not evoking any flight controls. The flight controls are implemented into the test for a defined amount of simulation frames. For example, from frame 1 through 5, thrust is increased, and so on. The flight data at the end of each test execution was recorded for each FDM. We only test one side of each of the flight controls (i.e. roll right vs roll left) to reduce redundancy.

The benchmark C++/OOP FDM was compiled with Visual Studio 2019 Community Edition, and the Rust/ECS FDM was compiled with rustc version 1.47.0. The tests were performed on a Lenovo ThinkPad T440p machine, running Windows 10, with an intel i74800mq processor.

### 3.5.1 Test One (No Flight Control)

This test was designed to find out if the FDM works when absolutely no flight control actions are taking place to maneuver the airplane as it is flying.

Frame by Frame Flight Control Instructions

None

Table 6: Flight Control Instructions for Test One

### 3.5.2 Test Two (Thrust)

This test was designed to find out if toggling thrust, specifically decreasing it, works as expected.

Frame by Frame Flight Control Instructions

1 – 5: Decrease Thrust

6 – 900: None

Table 7: Flight Control Instructions for Test Two

### 3.5.3 Test Three (Roll)

This test was designed to examine if the roll action works properly. First thrust is increased, then the airplane pitches up for 8 seconds (240 frames) and finally the airplane ailerons are evoked to roll right the airplane for 2 seconds (60 frames). The airplane then holds that rolling position for the remainder of the test. A pitch of 8 seconds was chosen in order to stabilize the airplane once the simulation is started. 8 seconds is just enough time to get the airplane flying straight ahead. When the simulation starts, the airplane would dip down quite dramatically without increasing the thrust and pitching up, so that is why we need to pitch before rolling.

Frame by Frame Flight Control Instructions

1 – 5: Increase Thrust

6 – 246: Pitch Up

247 – 307: Roll Right

308 – 900: None

Table 8: Flight Control Instructions for Test Three

### 3.5.2 Test Four (Pitch)

This test was designed to be sure that pitch on its own works. This test applies the pitch control to every frame throughout the entire execution time in order to ascend the airplane. The pitch flight control evokes the elevators. If the airplane were going faster caused by more than 1000 pounds of thrust force, it is possible that the airplane could stall – but in this case, the airplane ascends smoothly.

Frame by Frame Flight Control Instructions

1 – 5: Increase Thrust

6 – 900: Pitch Up

Table 9: Flight Control Instructions for Test Four

### 3.5.4 Test Five (Yaw)

This test examines the yaw flight control. First, the thrust is increased for 5 frames to 1000 pounds of thrust total. Next, the airplane is stabilized by pitching up for 8 seconds (240 frames). Once the airplane is flying straight, the airplane rudders are evoked to yaw right every 9 frames. So, if the current frame number is divisible by 9, the airplane was instructed to yaw right.

Frame by Frame Flight Control Instructions

1 – 5: Increase Thrust

6 – 246: Pitch Up

247 – 900: Yaw Right Every 9 Frames

Table 10: Flight Control Instructions for Test Five

### 3.5.4 Test Six (Flaps)

The final test examines the lowering of the flaps on the airplane. The flaps can either be up or down during the simulation; the flaps are toggled on and off. In this test, we find out if the flaps work properly when they are lowered during the entirety of the test, and that is all.

Frame by Frame Flight Control Instructions

1 – 900: Flaps Down

Table 11: Flight Control Instructions for Test Six

## 3.7 Building the Flight Simulator Overview

//add a new section called building flight simulator overview? And talk about how it was easy to add the new systems with ecs

A dispatcher was opened to manage the parallelization of the Systems. The dispatcher can be told to have a System depend on other Systems (i.e. a system must finish before another system begins).

## 3.8 SPECS Simulator Components

Components contain the data variables to be accessed by a System, if required. The three Components we have created are: KeyboardState, DataFDM, and Packet.

### 3.9.1 KeyboardState

The KeyoardState Component holds the boolean variables that correspond to the action that will take place in the simulation when a specified key is pressed on the keyboard. This Component is used by the EquationsOfMotion System and the FlightControl System.

### 3.9.2 DataFDM

The DataFDM Component holds all of the data variables describing an airplane, which is a rigid body by definition. This Component is used by the EquationsOfMotion System and the MakePacket System.

### 3.9.3 Packet

The Packet Component consists of a vector which holds the serialized, or made into bytes, FGNetFDM struct, which contains all of the data necessary to be sent to FlightGear in the form of a packet. The FGNetFDM structure that FlightGear accepts is laid out in FlightGear’s GitHub repository. The source code for the structure is located in src/Network/net_fdm.hxx. This Component is used by the MakePacket System and the SendPacket System.

//this should be consistent for both 4dof and 6dof

## 3.10 SPECS Simulator Systems

There are four Systems required to manage all of the functionality: FlightControl, EquationsOfMotion, MakePacket, and SendPacket. Systems access certain Components that are needed for reading or writing to, and the type of access must be specified. ReadStorage is just read access to the Compoment data, and WriteStorage is reading and also writing over the Component data.

### 3.8.1 FlightControl

This System ultimately handles getting the keypresses from the keyboard and then alters the KeyboardState Component data. So, it will be given WriteStorage access to KeyboardState.

To actually detect the keyboard keypresses, we implemented a Rust crate called Device Query, which is able to read the current state of the keys on the keyboard (i.e. key is being pressed or not). In code, Device Query gets list of all the keys being pressed at the current instance, and we check if any of the keys in that list match the keys that we have tied with a function in our FDM. Since more than one key can be pressed at once, we check all of the keys that we care about, and we toggle true the KeyboardState data that corresponds to that key press. Note: before we toggle to true any of the KeyboardState data, we default the data to false. With all of the states set for this instance, the EquationsOfMotion System will take care of the rest, until the next iteration.

### 3.8.2 EquationsOfMotion

This System is what computes the equations of motion for the FDM. This System requires ReadStorage access to KeyboardState and WriteStorage to DataFDM.

The System begins by reading the KeyboardStates and if a state is toggled true, the DataFDM is modified in a certain way depending on the key. For example, if the user is pressing the keyboard to increase thrust, the KeyboardState data representing thrust is toggled to true, and the FDM will take that into account for the calculation of that frame. That is the part that makes the FDM interactive. Following accounting for keyboard input, this System calculated the loads, or forces, acting on the airplane. From here, the acceleration of the airplane is calculated in Earth space. Then, the velocity of the airplane is calculated in Earth space. The velocity is then directly used to find the displacement in position since the last frame, that displacement is then added to the current position coordinates to determine the new position in Earth space. In terms of rotations, the angular velocity in body space is then computed. The angular velocity is then used to determine the quaternion, which can be extrapolated to give us the Euler angles. Finally, the airspeed is calculated by getting the magnitude of the velocity. These are all of the steps taken in the System to compute the equations of motion in the FDM.

### 3.8.3 MakePacket

This System is what turns the FDM into a functional flight simulator because it evokes the graphics of FlightGear. It is responsible for sending packets of data to FlightGear via a network socket. The packets contain the data required for FlightGear to know how to position the plane in the simulation, and how to orient the plane accordingly. It accesses both the DataFDM Component and the Packet Component as ReadStorage.

The Packet Component holds a vector which contains the FGNetFDM structure serialized into bytes. Inside of this System, an instance of the FGNetFDM structure is created and updated with the current calculations from the EquationsOfMotion System, which are stored in the DataFDM Component data. But when the new data variables are passed into the FGNetFDM structure, the data has to be first converted into an array of bytes in native byte order, meaning the byte order native to the machine compiling the software. From here, the byte array is then read as a big endian representation and turned into an integer value that FlightGear can interpret.

The data updated is the position of the airplane in space, and also the orientation of that airplane. The FGNetFDM position data updated is latitude, longitude, and altitude. The orientation variables updated are phy, theta, and psi, which represents the roll, pitch, and yaw, respectively. It is updated with the Euler angles. The FlightGear version is also required to be passed on to the FGNetFDM struct. Once all of the FGNetFDM data has been updated with new values from the current DataFDM Component, and converted to big endian byte order, the data can be arranged into a packet of u8 bytes in the form of a vector, and passed into the Packet Component.

### 3.8.4 SendPacket

With the packet constructed and updated, this System simply serves to send that packet off to FlightGear via a socket connected on its port. This system only needs read access to the Packet Component.

# IV.  Analysis and Results

This section explains the results obtained by the tests performed in Chapter III. We first discuss the final application built together as an ECS to make an airplane fly, and then we discuss the tests performed to determine equivalency of our Rust/ECS FDM to the benchmark C++/OOP FDM.

## 4.1 Flight Simulation Applications

The FDMs built and discussed in Chapter III ultimately are able to fully function as a flight simulation application when paired with FlightGear’s graphics. By re-implementing Palmer’s FDM, and Bourg’s FDM, we were able to mimic their realistic flight dynamics and determine the position and the orientation to be sent to FlightGear via a UDP packet to visualize the airplane on the screen.

## 4.2 4-DOF FDM Equivalency Verification

### 4.2.1 Test One (Throttle)

|  | Rust/ECS Output | C Benchmark Output |
| --- | --- | --- |
| Total Distance Traveled (m) | 1470.183480 | 1470.183480 |
| Altitude (m) | 4.107990 | 4.107990 |
| Speed (m/s) | 56.192869 | 56.192869 |

Table 12: Numerical Results for Test One

### 4.2.2 Test Two (Angle of Attack)

|  | Rust/ECS Output | C Benchmark Output |
| --- | --- | --- |
| Total Distance Traveled (m) | 1341.911816 | 1341.911816 |
| Altitude (m) | 72.797432 | 72.797432 |
| Speed (m/s) | 43.417180 | 43.417180 |

Table 13: Numerical Results for Test Two

### 4.2.3 Test Three (Bank)

|  | Rust/ECS Output | C Benchmark Output |
| --- | --- | --- |
| Total Distance Traveled (m) | 893.011117 | 893.011117 |
| Altitude (m) | 99.300874 | 99.300874 |
| Speed (m/s) | 32.637523 | 32.637523 |

Table 13: Numerical Results for Test Three

### 3.5.4 Test Four (Flaps)

|  | Rust/ECS Output | C Benchmark Output |
| --- | --- | --- |
| Total Distance Traveled (m) | 1175.254786 | 1175.254786 |
| Altitude (m) | 96.364421 | 96.364421 |
| Speed (m/s) | 37.797770 | 37.797770 |

Table 13: Numerical Results for Test Four

## 4.3 6-DOF FDM Equivalency Verification

The series of tests investigating our Rust/ECS FDM versus the C++/OOP FDM benchmark under some specific scenarios successfully proved equivalency given that the flight data closely matched in every test. Specifically, each flight control tested was found to be working as expected when compared to Bourg’s C++/OOP benchmark FDM. Each test section displays the resultant flight data at the end of test execution for both our FDM and the benchmark FDM for that given test.

( need a way to do actual analysis of the results, ideally they are exactly the same, but none of them are. They are pretty close. The yaw test and roll test have the most error. And it seems the most error occurs in y position and yaw angle. I have no idea why.

Ideally with the same results we do not need to do any statistics to determine how equivalent they actually are, and thus no real explanation of the results for each experiment would be needed)

### 4.2.1 Test One (No Flight Control)

|  | Rust/ECS Output | C++/OOP Benchmark Output |
| --- | --- | --- |
| Position (x, y, z) | -1045.2303 -0.008528658 420.12427 | -1045.37, -0.0444084, 421.399 |
| Euler Angles (roll, pitch, yaw) | -0.0028137087 -16.14772 -0.0042060204 | -0.00279299, -16.1436, -0.00508352 |
| Speed (knots) | 85.98892180 | 85.9906 |

Table 12: Numerical Results for Test One

### 4.2.2 Test Two (Thrust)

|  | Rust/ECS Output | C++/OOP Benchmark Output |
| --- | --- | --- |
| Position (x, y, z) | -1636.1309 0.041476656 -193.3939 | -1636.11 -0.0220277 -191.543 |
| Euler Angles (roll, pitch, yaw) | -0.0034582962 -27.698172 -0.0031659165 | -0.00348351 -27.6939 -0.00458124 |
| Speed (knots) | 82.64964 | 82.6589 |

Table 13: Numerical Results for Test Two

### 4.2.3 Test Three (Roll)

|  | Rust/ECS Output | C++/OOP Benchmark Output |
| --- | --- | --- |
| Position (x, y, z) | -4517.709 -2214.6902 638.1875 | -4794.57 -1924.94 482.128 |
| Euler Angles (roll, pitch, yaw) | 16.679296 -3.1004717 -155.08517 | 19.3067 -4.04701 -168.199 |
| Speed (knots) | 98.25334 | 100.042 |

Table 14: Numerical Results for Test Three

### 3.5.4 Test Four (Pitch)

|  | Rust/ECS Output | C++/OOP Benchmark Output |
| --- | --- | --- |
| Position (x, y, z) | -2095.0435 -1.021704 2452.5166 | -2093.35 -3.95869 2451.71 |
| Euler Angles (roll, pitch, yaw) | 0.030059163 34.988907 -0.082931094 | 0.404863 34.7533 -0.581428 |
| Speed (knots) | 51.53415 | 51.8425 |

Table 15: Numerical Results for Test Four

### 3.5.4 Test Five (Yaw)

|  | Rust/ECS Output | C++/OOP Benchmark Output |
| --- | --- | --- |
| Position (x, y, z) | -1086.669 -13.406829 1686.8999 | -1085.7 -24.4326 1687.13 |
| Euler Angles (roll, pitch, yaw) | -0.034989346 -2.983541 -0.5547009 | 0.141311 -2.98676 -0.975992 |
| Speed (knots) | 91.104195 | 91.0854 |

Table 16: Numerical Results for Test Five

### 3.5.4 Test Six (Flaps)

|  | Rust/ECS Output | C++/OOP Benchmark Output |
| --- | --- | --- |
| Position (x, y, z) | -1995.7982 -0.1490945 1778.0629 | -1995.63 -0.401624 1778.12 |
| Euler Angles (roll, pitch, yaw) | 0.0010366426 9.436404 -0.009251288 | 0.0124067 9.38473 -0.0325167 |
| Speed (knots) | 56.96767 | 57.0701 |

Table 17: Numerical Results for Test Six

# V.  Conclusions and Recommendations

(should the contribution part of the intro go here)

## 5.1 Research Conclusions

## 5.2 Significance of Research

Text

## 5.3 Recommendations for Future Research

Text

## Summary

Text

# Appendix: Configuring FlightGear as a Visual System

fgfs.exe – runs the executable
--aircraft=ufo – select an aircraft name
--disable-panel – disable instrument panel
--disable-sound – disable sounds
--enable-hud – enable heads up display 
--disable-random-objects – disable random scenery objects
--fdm=null – turn off the built-in FDM
--timeofday=noon – time of day 
--native-fdm=socket,in,30,,5500,udp – sets up FlightGear to receive data from an external source. Open a socket, which takes in packets at a speed of 30 hz on port 5500 [6].

Altogether, when running FlightGear, the command line is:

fgfs.exe --aircraft=ufo --disable-panel --disable-sound --enable-hud --disable-random-objects --fdm=null --timeofday=noon --native-fdm=socket,in,30,,5500,udp

Note: Replacing –aircraft=ufo with –aircraft=c172p will display a Cessna Skyhawk.

The structure representing a rigid body airplane in FlightGear, written in Rust:

struct FGNetFDM

{

version: u32, // increment when data values change

padding: f32, // padding

// // Positions

longitude: f64, // geodetic (radians)

latitude: f64, // geodetic (radians)

altitude: f64, // above sea level (meters)

agl: f32, // above ground level (meters)

phi: f32, // roll (radians)

theta: f32, // pitch (radians)

psi: f32, // yaw or true heading (radians)

alpha: f32, // angle of attack (radians)

beta: f32, // side slip angle (radians)

// // Velocities

phidot: f32, // roll rate (radians/sec)

thetadot: f32, // pitch rate (radians/sec)

psidot: f32, // yaw rate (radians/sec)

vcas: f32, // calibrated airspeed

climb_rate: f32, // feet per second

v_north: f32, // north velocity in local/body frame, fps

v_east: f32, // east velocity in local/body frame, fps

v_down: f32, // down/vertical velocity in local/body frame, fps

v_body_u: f32, // ECEF velocity in body frame

v_body_v: f32, // ECEF velocity in body frame

v_body_w: f32, // ECEF velocity in body frame

// // Accelerations

A_X_pilot: f32, // X accel in body frame ft/sec^2

A_y_pilot: f32, // Y accel in body frame ft/sec^2

A_Z_pilot: f32, // Z accel in body frame ft/sec^2

// // Stall

stall_warning: f32, // 0.0 - 1.0 indicating the amount of stall

slip_deg: f32, // slip ball deflection

// // Pressure

// // Engine status

num_engines: u32, // Number of valid engines

eng_state: [f32; 4], // Engine state (off, cranking, running)

rpm: [f32; 4], // // Engine RPM rev/min

fuel_flow: [f32; 4], // Fuel flow gallons/hr

fuel_px: [f32; 4], // Fuel pressure psi

egt: [f32; 4], // Exhuast gas temp deg F

cht: [f32; 4], // Cylinder head temp deg F

mp_osi: [f32; 4], // Manifold pressure

tit: [f32; 4], // Turbine Inlet Temperature

oil_temp: [f32; 4], // Oil temp deg F

oil_px: [f32; 4], // Oil pressure psi

// // Consumables

num_tanks: u32, // Max number of fuel tanks

fuel_quantity: [f32; 4],

// // Gear status

num_wheels: u32,

wow: [f32; 3],

gear_pos: [f32; 3],

gear_steer: [f32; 3],

gear_compression: [f32; 3],

// // Environment

cur_time: f32, // current unix time

warp: f32, // offset in seconds to unix time

visibility: f32, // visibility in meters (for env. effects)

// // Control surface positions (normalized values)

elevator: f32,

elevator_trim_tab: f32,

left_flap: f32,

right_flap: f32,

left_aileron: f32,

right_aileron: f32,

rudder: f32,

nose_wheel: f32,

speedbrake: f32,

spoilers: f32

}

# Bibliography

[1] T.S. McNamara. Rust In Action, 2019.

[2] Steve Klabnik and Carol Nichols. The Rust Programming Language, 2019.

[3] Richard Apodaca. Rust Ownership By Example, 2020. [Online; accessed 07/01/2020.

[4] Oliver Gould and Lachie Everson. “All About Rust.” YouTube, uploaded by Microsoft Developer, 2019.

[5] wiki.flightgear.org. Command line options, 2019. [Online; accessed 07/02/2020.

[6] wiki.flightgear.org. Property Tree/Sockets, 2019. [Online; accessed 07/02/2020.

[7] Brian Hall. Beej’s Guide to Network Programming Using Internet Sockets, 2019.

[8] Mike Geig. “ECS Overview - Intro To The Entity Component System And C# Job System 1/5.” YouTube, uploaded by Unity, 2018.

[9] Ben. “Entity Component System Overview in 7 Minutes.” YouTube, uploaded by Board To Bits Gaming, 2019.

[10] Adam. Entity Systems are the future of MMOG development – Part 2, 2007. [Online; accessed 07/01/2020.

[11] Mike Geig. “ECS Overview - Intro To The Entity Component System And C# Job System 2/5.” YouTube, uploaded by Unity, 2018.

[12] Thomas Schaller. The Specs Book. Crates.io, 1st edition, 2018.

[13] Matthew Duquette. The Development and Application of SimpleFlightm a Variable-Fidelity Flight Dynamics Model, 2007.

[14] David Bourg and Bryan Bywalec. Physics for Game Developers, 2nd Edition, 2013.

[15] https://unity.com/solutions/game. Create a world with more play, 2020. [Online; accessed 09/01/2020.

[16] https://unity.com/case-study/mediatonic-fall-guys. Unity helps bring the masses together in Mediatonic’s absurd multiplayer, 2020. [Online; accessed 09/01/2020.

[17] Albert Rhemann. A Handbook of Flight Simulation Fidelity Requirements for Human Factors Research, 1995.

[18] Gaétan Dussart, Vilius Portapas, Alessandro Pontillo and Mudassir Lone. Flight Dynamic Modelling and Simulation of Large Flexible Aircraft, 2018.

[19] https://crates.io/. The Rust’s Community Crate Registry. [Online accessed; 09/01/2020.

[20] Wayne Durham. Aircraft Flight Dynamics and Control, 2013.

[21] Vinayak D. “What is Flight Dynamics? - Derivation of Equations of Motion for an Aircraft.” YouTube, uploaded by VDEngineering, 2019.

[22] https://en.wikipedia.org/wiki/Moment_of_inertia. Moment of Inertia, 2020. [Online accessed 09/01/2020.

[23] https://en.wikipedia.org/wiki/Airfoil. Airfoil, 2020. [Online; accessed 09/01/2020.

[24] Steve Klabnik and Carol Nichols. The Rustonomicon, 2019.

[25] D.Rose. http://danceswithcode.net/engineeringnotes/geodetic_to_ecef/geodetic
_to_ecef.html. Converting Between Earth-Centered, Earth-Fixed and Geodetic Coordinates, 2014. [Online; accessed 09/01/2020.

[26] Glen Fiedler. Integration Basics. Gaffer On Games, 2004.

[27] Glen Fiedler. Fix Your Timestep. Gaffer On Games, 2004.

[28] Koen Witters. deWiTTERS Game Loop, 2009.

[29] Raymond R. Hill and J. O. Miller. A History of United States Military Simulation, 2017.

[30] Grant Palmer. Physics For Game Programmers, 2005

[31] https://doc.rust-lang.org/stable/rust-by-example/. Rust by Example. [Online; accessed 07/01/2020.

[32] Robert Nystrom. https://gameprogrammingpatterns.com/game-loop.html. Game Loop, 2014. [Online; accessed 08/01/2020.

[33] http://wiki.flightgear.org/Flight_Dynamics_Model. Flight Dynamics Model, 2020. [Online; accessed 07/01/2020.

[34] Joseph Vagedes. A Study of Execution Performance for Rust-Based Object vs Data Oriented Architectures, 2020.

[35] https://en.wikipedia.org/wiki/Flight_dynamics_(fixed-wing_aircraft), 2020. [Online; accessed 12/10/2020.

# Acronyms

ECS

DOF
OOP
FPS

3D

2D

FDM

DOD

ECEF

UDP

DT