# A RUST BASED, ECS PATTERNED, INTERACTIVE FLIGHT SIMULATOR

# THESIS

# Chad A. Willis, 2d Lt, USAF

# AFIT-ENG-MS-21-M-XXX

# DEPARTMENT OF THE AIR FORCE

# AIR UNIVERSITY

###### AIR FORCE INSTITUTE OF TECHNOLOGY

# Wright-Patterson Air Force Base, Ohio

# DISTRIBUTION STATEMENT A.

# APPROVED FOR PUBLIC RELEASE; DISTRIBUTION UNLIMITED.

# The views expressed in this thesis are those of the author and do not reflect the official policy or position of the United States Air Force, Department of Defense, or the United States Government.  This material is declared a work of the U.S. Government and is not subject to copyright protection in the United States.
AFIT-ENG-MS-21-M-XXX

# A RUST BASED, ECS PATTERNED, INTERATIVE FLIGHT SIMULATOR

THESIS

Presented to the Faculty

Department of Aeronautics and Astronautics

Graduate School of Engineering and Management

Air Force Institute of Technology

Air University

Air Education and Training Command

In Partial Fulfillment of the Requirements for the

Degree of Master of Science in Computer Science

Chad A. Willis, BS

2d Lt, USAF

March 2021

# DISTRIBUTION STATEMENT A.
APPROVED FOR PUBLIC RELEASE; DISTRIBUTION UNLIMITED.

AFIT-ENG-MS-21-M-XXX

# A Rust based, ecs patterned, interactive flight simulator

# THESIS

Chad A. Willis, BS

2d Lt, USAF

Committee Membership:

Dr. Douglas D. Hodson, PhD

Chair

Maj. Richard A. Dill, PhD

Member

Dr. Scott L. Nykl, PhD

Member

AFIT-ENG-MS-21-M-XXX

# Abstract

This thesis explores the Entity-Component-System (ECS) software design pattern for use in military computer training-based flight simulation, by implementing a 3-dimensional (3D) flight dynamics model (FDM) using this design pattern. The FDM is based on the works of David Bourg in his textbook titled Physics for Game Developers.  The ECS architecture is based on the Data-Oriented Design (DOD) paradigm. The intent of the research was to divide and define both the Components and the Systems for this FDM. This thesis defines four ECS compatible aspects: computing the equations of motion, handling flight control, making a packet, and sending a packet. The main aspect of this thesis consists of a System that computes the FDM’s equations of motion, and a Component that holds the resulting data. The three other ECS compatible aspects support the testing and visualization of the FDM. A System is responsible to check for keyboard presses that manipulate the aircraft’s control system, and a Component stores those keyboard states to taken into account by the equations of motion System. Another System constructs packets based on the output of the aircrafts equations of motion, and a Component holds that packet data. Lastly, a System is responsible for actually sending the packet held by that Component to the FlightGear simulator to provide a graphics display. The specific ECS selected for this research is the Rust-based SPECS framework. These ECS compatible Systems and Components, when applicable, were verified to be accurate in modeling flight when compared against David Bourg’s C++ and Object-Oriented Programming (OOP) designed FDM.

# Table of Contents

Page

Abstract	1

Table of Contents	3

List of Figures	4

I.  Introduction	5

1.1 Problem Statement	9

1.2 Research Objective	9

1.3 Hypothesis	10

1.4 Contribution	11

1.5 Approach	11

1.6 Assumptions/Limitations	12

1.7 Thesis Overview	13

II. Background	13

2.1 Physics of Flight Modeling	13

2.2 Entity Component System	24

2.3 Rust	29

2.4 FlightGear	33

2.5 Networking with Rust	34

III.  Methodology	35

3.1 Building the FDM	35

3.2 Baseline Comparison	42

3.3 ECS and OOP Performance Experiment (need working program first…)	43

IV.  Analysis and Results	43

ECS and OOP Performance	43

Baseline Comparison	43

Investigative Questions Answered	44

Summary	44

V.  Conclusions and Recommendations	44

Chapter Overview	44

Conclusions of Research	44

Significance of Research	44

Recommendations for Action	44

Recommendations for Future Research	44

Summary	44

Appendix: Configuring FlightGear as a Visual System	45

Bibliography	48

# List of Figures

Figure 1: Forces Visualized	13

Figure 2: Airplane Components	14

Figure 3: Airfoil Cross-section	15

Figure 4: Airplane Axes of Rotation	16

Figure 5: ECEF Coordinates Visualized	17

Figure 6: Lift Coefficient vs Angle of Attack	19

Figure 7: Entity Component System Visualized	23

# I.  Introduction

Real-time computer training-based simulations are important tools for the military. They are used in order to prepare servicemen and servicewomen for a possible conflict and practice strategies that hope to achieve a positive outcome during a real-life conflict [29].  Purssian Field Marshal Helmuth von Moltke, regarded as the grandfather of the modern military simulations, once asserted that “No Battle Plan Survives Contact with the Enemy” [29]. This assertion only reinforces the need to train, and simulations are an effective method of training.

The reasons to use a virtual computer simulation over real-life simulation is because it is less expensive, more accessible, more versatile, and completely safe. Virtual simulations are less expensive because you do not have vehicles, like airplanes, burning gas and requiring manpower, like maintenance. They are more accessible because all you need is a capable computer to use it. They are more versatile because you can simulate scenarios that might not be possible to put together in real-life. And finally, they are completely safe because users cannot hurt themselves.

Today, military computer training-based simulations exist to simulate everything that could take place at war, such as battle planning or individual pilot training. The simulation software used is often written using the Object-Oriented Programming (OOP) paradigm. But there is an alternative, modern paradigm, called Data-Oriented Design (DOD), that provides some powerful benefits over OOP, and that we will explore in this thesis as it relates to flight simulation. Specifically, we will discuss an implementation of DOD, called the Entity Component System (ECS) architecture. ECS is a programming design architecture that is primarily used in the video game industry to design game engines. But simulations are similar enough to games to be able to take advantage of this architecture. In this thesis, the leveraging of the ECS design in a military oriented simulation is of particular interest because it has never been done before. The ECS architecture is simply a new way of organizing data, alternatively to using OOP to organize data. Where OOP relies on hierarchies of data, ECS relies on compartmentalization of data, where data content and its functionality on the data is separated. Notably, Unity, a platform for creating and operating real-time 3D games.  uses an ECS architecture in its game engine [15]. The massively popular battle royale game Fall Guys: Ultimate Knockout, for example, was created with Unity, which relies on the ECS design [16]. The ECS architecture, which is discussed in more detail in Chapter 2, leads to code that is more reusable, readable, and performant. But we will also discuss any possible pitfalls in software created as an ECS.

In this thesis, we focus on the research and development of 3-Dimensional (3D) flight dynamics model (FDM). This is the important piece of a flight simulation software that actually calculates how an airplane should fly in real-life and is essentially all of the math that makes an airplane act like an airplane. As a plane is flying, many things are happening. The pilot of the airplane, or in this case, the simulation user, could be deciding to accelerate, turn, and change altitude, sometimes all at once. With that user input, the FDM calculates and determines how the plane should react, while taking into account external forces, like gravity, as well as the physical properties of the airplane. Although the software engineering of FDMs has been solved, and the math to mimic realistic physics is a standard practice, this thesis describes building a FDM that is unlike any other in terms of how the code is organized as an ECS.

To actually implement the ECS architecture in a FDM, the Rust programming language was chosen. Rust, created in 2015, introduces a combination of important features not available in any other language. These provided features stem from two concepts that Rust focuses on: memory safety and performance. Rust will always guarantee memory safety, avoiding issues such as data races, dangling references, and buffer overflows in software. It accomplishes this safety by checking that memory is squared away and cleaned up at compile time. Other languages cannot make this guarantee because they rely on a highly-experienced programmer to be sure the software is memory safe. Although some languages are also memory safe like Rust, they lack in performance in order to accomplish memory safety. This is because they use a garbage collector at runtime to ensure memory safety – this not ideal for performance. Rust’s performance, however, is a positive result of the way it tackles memory issues because it does not need a garbage collector and checks for issues at compile time.  These features are undoubtedly worthwhile to take advantage of in a flight simulator. But Rust is not without drawbacks compared to other languages, which we will discuss. In order to implement the ECS design within the Rust language, the framework, SPECS, is the library, or, as the Rust programming language calls it, crate, that is imported in code.

## 1.1 Problem Statement

In real-time military flight simulations, including flight simulations, it is common to see the software using an OOP design approach, and be written in a systems level programming language like C++. It is likely that the simulation could spin up hundreds or even thousands of moving entities, or game objects, such as airplanes, or soldiers. These entities have many different functions acting on potentially large sums of data associated with the entities. Unfortunately, with OOP, the entanglement of functionality and data memory in hierarchies impedes performance, and also causes a mess when trying to modify or add functionality or entities. Also, when programming with systems level programming languages, it is all too easy to introduce memory vulnerabilities in code. An expert programmer would be required to ensure that memory is cleaned up properly.

## 1.2 Research Objective

The scenario stated above could benefit from the compartmentalization that the ECS approach employs in game engines. With functionality and data decoupled, or separated, software can be as performant as possible in terms of memory-management and parallelization. With that, the decoupling nature of ECS improved ease of code development because pieces of the code are not dependent on each other in order to work [34]. To summarize, the modern ECS architecture is a viable solution which can provide some notable performance advantages and ease of development advantages over OOP in video games, which carries over to simulations. Furthermore, Rust solves the memory safety burden that plagues systems level languages by enforcing strict rules at compile time. The goal of this research is to build an interactive, easily extensible, 3D FDM implemented as a Rust based, ECS architecture. The FDM is based on the works by David Bourg in [14].

## 1.3 Hypothesis

The thesis hypothesizes that a 3D FDM implemented as a Rust based, ECS design will, not only serves as a viable design alternative, but also introduces some powerful features that other FDMs certainly do not possess. By using ECS to organize data differently, we can separate, or decouple, functionality from the actual data in memory, as opposed to having functionality and memory tied together in hierarchies. This is a powerful and popular strategy in designing complex video game engines, which need to process updates to large amounts of game entities dealing with large amounts of data. This decoupling allows for the functionality on game entities to be easily parallelized and fast. In addition to performance benefits, because ECS decouples code, the ability to modify existing functionality or entities, or add new functionality or entities is a much smoother process compared to OOP. This will be explained in more detail in the next chapter. So, for the same reasons game designers choose to use the ECS design in a video game, we can also benefit by using the ECS design in our FDM.

## 1.4 Contribution

The contribution of this thesis lies in the field of flight modeling and flight simulations for military use. The successful build of a 3D Rust based, ECS designed, real-time FDM demonstrates an alternate, modern, and powerful design strategy over current FDMs. Not only is this strategy available for use, but it is useful to increase performance through efficient memory-management and parallelization. This strategy also increases code maintainability due to the nature of the ECS architecture; this is a valuable feature that could save time and effort when updating a FDMs code later in its lifespan.

## 1.5 Approach

The methodology of this research is explained in full detail in Chapter 3, but an

overview of the approach will be given here.

The focus of this research is building a Rust based, ECS patterned, 3D FDM based on the works by David Bourg in [14]. However, along the way, a 2D, Rust based, ECS patterned FDM is created based on the works by Grant Palmer in [30]. The building of this 2D FDM was essentially a stepping stone to reach the goal of creating the 3D version. In terms of ECS compatible aspects, the 3D FDM is composed of three Components and four Systems, all of which play a part in the airplane Entities data storage and functionality. The Components, which contain all of the data used by the Systems are: KeyboardState, RigidBody, and Packet. The Systems, which add functionality to the Component data of the airplane Entities, are: FlightControl, EquationsOfMotion, MakePacket, and SendPacket. With the final 3D FDM built, to ensure its accuracy of modeling flight, we will find out if it replicates the realistic flight data generated by its C++ benchmark FDM as described in Bourg’s textbook.

## 1.6 Assumptions/Limitations

The first limitation is that the FDM built is not a standalone application; a graphics generation program is needed to display the resulting simulation. However, if a user does not require graphics and only the flight data text output is needed, the FDM can be executed without an external visualization system. But otherwise, the FDM is only configured to work with FlightGear as the visualization system. A specific packet structure is required by FlightGear to be able to interpret the data, however, the structure could be retrofit for different visualization systems.

We assume the simulation is not running on a very slow computer. The simulation loop is designed to run at a fixed frame rate, and a very slow computer may not be able to keep up with the predetermined frame rate.

Another limitation is that the airplane being simulated is only set up to match that of the airplane modeled in Bourg’s textbook. This no-name airplane is shown in all of the figures shown in the next chapter. The airplane characteristics could be modified in the code, however.

The airplane is set to fly at a predetermined location. And the latitude, longitude, and elevation of a desired location is needed to fly there, along with plugging in those numbers to the starting position coordinates.

## 1.7 Thesis Overview

This thesis is arranged into five chapters. Chapter II explains relevant background information on the physics of flight modeling, Entity Component Systems, Rust programming and FlightGear. Chapter III provides the thesis methodology. Chapter IV is a results analysis of Chapter III. Finally, Chapter V summarizes the research and impact of the research.

# II. Background

This chapter will provide an overview of the components that have gone into building this FDM. First, the background explains the basics of modeling flight physics in a FDM. Next, the ECS architecture, and an implementation if it, SPECS, will be described. Furthermore, the fundamentals of the Rust programming language will be laid out. Next, the use of the FlightGear flight simulator will be explained. Lastly, networking and sending packets of data to FlightGear with Rust will be described.

## 2.1 Physics of Flight Modeling

A flight dynamics model can be defined as the set of math equations that compute all of the physical forces on a rigid-body airplane, like thrust, and gravity [33]. A rigid-body can be defined as a body made up of particles that stay fixed and do not rotate or translate relative to the other particles. In this thesis, we focus on a FDM that attempts to approximate the real-life physics of a rigid-body airplane which can fly in 3D. Along the way, we will point out the differences between a 2D FDM and 3D FDM. Each FDM, 2D and 3D, are similar in many ways, but they do have different considerations in regards to the airplanes rotations, which will be described. Regardless, the final result is integrating the equations of motion. This boils down to determining the position of the airplane, as well as the airplane’s velocity, which is the speed and direction of the airplane, over time [20]. But before the equations of motion are solved, an understanding of forces, airplane components, airfoils, rotations, and coordinate systems is needed.

### 2.1.1 Forces

There are four major forces that act on an airplane in flight: gravity, lift, thrust, and drag. These forces govern the motion of the airplane. Newton’s Second Law of Motion says: the acceleration of a body is proportional to the resultant force acting on the body. The resulting equation is F = m * a. Gravity pulls the airplane to the ground and lift is generated by the airplane wings to counteract gravity to keep the airplane aloft. Thrust is generated by the airplanes jet engine or propeller to increase velocity and to keep the airplane generating lift. Drag is the opposite of thrust; drag impedes an airplanes motion. Torque, also known as moment, is similar to force, but force is what causes linear acceleration, and torque is what causes rotational acceleration.  In order to fly, an airplane must have a lift force acting upon it greater than or equal to gravity. Figure 1 is a visual of the four forces [14].

Figure 1: Forces Visualized

### 2.1.2 Airplane Components

An airplane is composed of different parts that affect the state of the airplane. The wings are the rectangular parts coming out of the main body. The span is the length of the wing, whereas the chord is the width of the wing. The ratio of span squared to wing area is called aspect ratio. The ailerons are on the outside end of the wings, and the flaps are on the inside of the wings. The elevators are on the tail, and look like small flaps. The rudder is the vertical flap on the tail. Figure 2 shows these airplane components [14].

Figure 2: Airplane Components

### 2.1.3 Airfoil

The cross-sectional structure of the wings, called an airfoil, is also important in understanding forces. Camber represents the asymmetry between the top surface and bottom surface of the airfoil which creates a curvature. The angle of attack is the angle between the chord line and the direction of travel, or oncoming flow that the plane is moving through [14]. Figure 3 shows a labeled cross-section of an airfoil with an angle of attack [23].

Figure 3: Airfoil Cross-section

### 2.1.4 Rotations

The orientation of the airplane when it is rotating can be described by three angles: pitch, roll, and yaw. These angles are called the Euler angles and represent the x, y, and z axis, on a coordinate plane, respectively. When an airplane is flying in 2D, it can only rotate about 2 axes relative to the pilot. It can pitch, and roll, while yaw is fixed. On the other hand, in 3D, it can rotate about all three axes relative to the pilot. The ailerons create the roll rotation. The elevators create the pitch rotation. The rudder creates the yaw rotation. The flaps do not rotate the plane, but they do affect lift by altering the chord and camber of the wing, which has to do with the shape of the cross-section of each wing. Figure 4 shows the three types of airplane rotations [14].

Figure 4: Airplane Axes of Rotation

### 2.1.5 Coordinate Systems

Coordinate systems are important to understand when determining the location of the airplane on the Earth. Earth-Fixed, Earth-Centered (ECEF) coordinates tell us where the plane is in relation a fixed point, which is the center of the Earth. This is the coordinate system, represented by Cartesian coordinates (x, y, z), used in the FDMs [25]. However, it is important to realize that FlightGear, the visual system, describes an airplane on Earth with the geodetic coordinate system, and not ECEF the coordinate system. The geodetic coordinate system uses latitude, longitude, and altitude to represent location [25]. To work around this, the ECEF coordinates calculated by the FDM need to be transformed into geodetic coordinates so that FlightGear can understand where the airplane is located on Earth. To actually do the conversion to geodetic coordinates, a Rust crate, called coord transforms is used in the FDM given the ECEF coordinates. Figure 5, in the bottom-right of the figure, shows the ECEF coordinates plane that is relative to that point [21].

Along with the ECEF coordinates, also called the earth frame, another coordinate system exists to describe the frame of reference in order to track 3D rotations of the airplane. It is called the body frame, and it is an x, y, z coordinate systemsm like ECEF, but unlike the the earth frame, which is fixed somewhere on the Earth, the body frame is fixed to the center of gravity of the rigid body airplane, so it moves with the airplane [20]. Rotations happen along each of the three x, y, z axes. Figure 5 also visualizes the body frame coordinate system representing rotations about the fixed point on the center of gravity of the airplane [21].

Figure 5: Earth Frame and Body Frame Visualized

### 2.1.6 FDM Calculation Steps

With an understanding of the airplane components, airfoils, forces, rotations, and the coordinate systems, we can begin to explain the steps the FDM takes:

- The mass properties of the airplane are calculated, or decided

- The load, or forces and moments on the airplane are calculated

- The equations of motion are integrated

#### 2.1.6.1 Mass Properties

The mass properties involve calculating how the mass and physical components of the airplane will affect the forces acting upon it. To begin, one must decide which airplane they are trying to model. In Palmer’s model, the Cessna 172 is being modeled. In Bourg’s model, an actual airplane is not being modeled, but the model representation is that of the picture of Figure 2. With that, the mass properties are chosen to represent that desired airplane. For example, the airplanes total wing area will affect both lift and drag forces, and mass will affect gravity.

In terms of 3D, mass properties become more complicated. The rigid body airplane’s center of gravity needs to be calculated, along with airplane’s moment of inertia tensor. The center of gravity, or center of mass, is where the forces are applied on the airplane. The moment of inertia is essentially the airplanes resistance to rotation. It can also be described as a distribution of mass. It is the amount of torque needed to make the airplane rotate about an axis relative to another point (i.e. center of mass). The measure of how fast the airplane actually rotates is called angular velocity [22]. The moment of inertia tensor is the mathematical expression, represented as a 3x3 matrix, that has magnitude and direction, and holds the values of the moments of inertia as body coordinates [14].

#### 2.1.6.2 Forces

With mass properties handled, the forces and moments acting on the airplane at all times can be calculated. Lift and drag are not as simple to calculate compared to gravity and thrust. There are many components on the airplane that affect lift and drag. But to generalize, the total lift force on an airfoil is composed of two components: the lift due to camber and the lift due to attack angle. The airplane’s flaps are used to alter camber. Usually the flaps are used are low speeds to increase lift. Drag increases with increased angle of attack. If an airplane is flying at an angle of attack high enough, it will not be able to create any lift and will stall. Figure 6 shows a graph of increasing lift with increase angle of attack, however, at some point a potential stall occurs with lift dropping off rapidly. Also, shown in Figure 6 is that with flaps lowered, lift is increased [14].

Figure 6: Lift Coefficient vs Angle of Attack

There are also other variables that affect lift such as air density, speed, viscosity, surface area, and aspect ratio. External forces also exist that can act on the airplane such as air drag, and winds.

Gravity is simply based on mass of the airplane and the acceleration of gravity, which is a constant -9.8 m/s2. So, gravity can be calculated by the mass of the airplane x -9.8 m/s2, which is added to the force acting on the z axis (down). Similarly, thrust, expressed in pounds, is force added to the x axis (forward). Without thrust propulsion, the airplane cannot make lift. The thrust will change depending on the simulation users input with the use of the throttle.

#### 2.1.6.3 Equations of Motion

With forces dealt with, the equations of motion are integrated so that the simulations can progress through time, determining the position of the airplane, as well as the airplane’s velocity in each step of time. In computer simulations, the equations of motion are approximated by solving nonlinear ordinary differential equations using numerical integration algorithms [20]. The integration of the equations of motion are considered an approximation because there is always some error introduced depending on the technique chosen [14]. Simply put, the integration of the equations of motion, given everything else talked about, approximates the future velocity and position based on the previous values from the last tick of the simulation [26].

The amount of finite time chosen to represent a tick in the real-time simulation loop is important in numerical integration. A correctly chosen time step will result in stable numerical integration. Stability has to do with how closely an approximate solution converges to the exact solution [14]. A smaller time step will give a result closer to the exact solution but this also means that there are more steps and a larger buildup of numerical precision error. On the other hand, there is a breaking point when the time step is too big where the simulation physics to explode to infinity [27]. With that said, there is a practical balance as to what the time step should be to achieve a stable solution, and also minimize the buildup of error [14].

Another thing to think about when choosing a time step is that there are different design strategies to actually employing the simulation loop. The easiest way to manage the simulation loop is to just loop as fast as possible with no time step. The problem with that is that there is no control. If you are on a fast computer it the simulation will go to fast, and if you are on a slow computer it will crawl [32]. But, as stated before, this is not how we want to run the simulation loop because we want a time step. The next easiest way to manage the loop, and the strategy we choose, is to have a fixed delta time, or a fixed time step. This is ideal when it is known that the update loop takes less than one frame worth of real time [27]. For example, if the time step chosen is 33 milliseconds, then we need to reliably know that the real processing time will take less than that. And for the remaining time the program will go to sleep until it is time for the next update [32]. The more advanced way to employ a simulation loop is to have a variable time step. It involves choosing a time step based on how much real processing time the current update takes versus the previous update’s real processing time [32]. This strategy, however, introduces some issues, which we will not get into.

There are multiple methods to actually implement numerical integration to calculate the equations of motion. Using Euler’s method is by far the most common numerical integration technique [20]. In both the 2D and 3D model, we use an improved and more accurate version of Euler’s method, called the Range-Kutta method, which is laid out as code in Bourg’s textbook and Palmer’s textbook.

The last aspect to mention about the equations of motion is that, during this process, in the 3D model, the orientation of the plane is tracked with the use of a quaternion. A quaternion is a way to represent orientation of something in 3D. Quaternions get updated with new Euler angles in each time step to reflect the new orientation using the angular velocities of that instant [14].

### 2.1.5 Fidelity

(do we even care about this?) maybe bits of this paragraph can go elsewhere

With the FDMs described, fidelity is the degree to which the characteristics of a flight simulation matches that of the real world [17]. In this case, the FDMs are of “medium” fidelity because the characteristics of the flight simulation match that of real life, but is not computationally expensive, and requires minimal data [18]. One may think that a flight model with the highest fidelity, or accuracy, is the best – this is not always true. Very advanced flight modeling will require large, detailed sets of data about the stability and performance of an aircraft that it is modeling. Wind tunnel data analysis is an example of the kind of data needed for a high fidelity flight model. This data is not readily available and often costs hundreds of thousands of dollars [13].  The physics implemented in these FDM are real and plenty accurate to compared the real world.

(this paragraph on fidelity probably not needed but it does help add merit to the models, is any of this paragraph worth keeping?)

## 2.2 Entity Component System

The FDM will be organized, and written in code as a ECS architecture, which is an implementation of the DOD. ECS is a different way to organize data, and a viable alternative to organizing code with OOP. The ECS design is popular in designing game engines. The Unity game engine, for example, uses the ECS design pattern [8]. ECS focuses on the composition of data, whereas OOP focuses on inheritance and hierarchies. ECS gets rid of the troubles that arise with inheritance, and improves upon performance and ease of code development.

### 2.2.1 Object-Oriented Problem

Suppose we have a space or world that we want to populate with things. These things may have similar attributes as well as unique attributes. With OOP, you would have a base class called Thing that all other Thing types will inherit. The limitation is that one child class can only inherit directly from one parent. For example, a Mammal type may have a child Dog which inherits a fur attribute, and a Bird may have a child Duck which inherits the lays eggs attribute. But in this case, a platypus, which has fur and lays eggs needs to inherit both attributes [9]. This complicates things, and becomes a larger problem with a larger hierarchy. The hierarchy chains that are created in game design can become too large to work with and code becomes too highly dependent on other code [10]. This makes code readability, maintenance, and the addition of features a pain with OOP. However, this is where ECS shines.

Along with code flexibility, performance can be an issue with the OOP design when talking about memory-management in terms of the cache. When a class instance, or object, is instantiated, it is not most efficiently put into cache. When an object is created, all its data is put into cache right next to other all the other object’s data. This is a problem when using multithreading because if a thread shares an object’s data, there is not always enough room on the cache to all the data that the thread actually wants to work on. And when unneeded data is in cache, the chance of cache misses happening increases. This is bad for performance and ECS improves upon this.

### 2.2.2 ECS Explained

In an ECS structure, you do not have hierarchy levels – you have these things called Entities, Components, and Systems. You start with an Entity, which represent concrete objects in the world, like a bullet. Entities on their own are useless; they are just containers which Components can be added to. Components hold data, usually stored as a vector in memory, and represent a feature. For example, Position is a Component of an Entity. The Components called Position, Render, and Spawning may be added to the Bullet Entity. The mixing and matching of Components can create unique Entities. A Player Entity can be created by adding the Components Position, Render, and Health. Systems are what give functionality to an ECS architecture. A System could be created called Render, which handles all of the rendering functionality of the world. Or a System called Health could handle all of the health functions. Systems have something called a filter. Filters look for every Entity which has some combination of specified Components. For example, the Render System’s filter wants to find every Entity with the Render data Component and Position data Component. Once these Entities are found, they are returned to the Render System to perform some job on it [11]. This is this example visualized [11]:

Figure 7: Entity Component System Visualized

As seen in Figure 7, the Systems are separate from the Components. The Systems, when performing a job, does not care what Entity it is operating on. The whole point is that the data Components are completely separated, or decoupled, from the Systems functionality [12]. This separates code dependencies, making it simple to add new Systems or Entities without needing to tweak anything else in code – a huge benefit that OOP does not offer. This decoupling also makes it simple to implement concurrency where a multiple Systems can be working on an Entity concurrently. Also, because a System does a job on specified data Components, it only iterates over Entities that it cares about.

The ECS design also optimizes performance by more efficiently managing memory. It does this by laying out the data in cache more efficiently than the OOP design does. The goal for any software is to retrieve data from cache as quick as possible, and this is an aspect that is easily overlooked when choosing a certain design approach to a program because it happens in the background. But ECS, minimizes cache misses by not having data organized by classes, and instead having data broken into smaller chunks – the data Components, which are commonly stored as a vector. So, for example, the Render System will only need the Render and Position data for all the Entities. Then, only that Component data that is needed to perform the System is put into cache for that Entity being worked on. This is how the cache holds what it needed, and does not bombard itself with all the data in a class. The vector which stores the Component data in the cache is guaranteed to hold data in each index. This is good for performance because no time is wasted determining if an index has null data [34].

### 2.2.4 SPECS Framework

To implement the ECS, we will use the SPECS framework, which is a Rust crate that is imported in the code. SPECS is developed by and used by Amethyst, which is an open-source game engine written in Rust. SPECS makes it easy to define Entities, Components, and Systems to leverage the ECS. SPECS maintains the reasons one would choose to use an ECS design: the resulting code is highly parallel, flexible, and fast [12].

### 2.2.5 ECS Overall

The ECS architecture has proven itself as a dominant strategy in improving game programming. It allows simple parallelization of complex Systems, simple code modification, and quick accesses to the data that matters. With all of these useful attributes, however, ECS does come with some downsides. The learning curve of ECS is absolutely present. To go from coding OOP to ECS, one must change their way of thinking because there is no inheritance, which every programmer is already experienced in. Also, planning ahead when coding in ECS is a necessity; one must know what systems will use what components, and changing components after the fact is not ideal. Since Systems are sometimes dependent on each other to complete, it may be difficult to add another System in the middle of two consecutive Systems. Planning ahead could help to avoid that situation. Also, because parallelization is a big benefit of ECS, it may not be worth using ECS if a program does not need concurrency. For example, a program that computes something and only consist of one system does not benefit from being an ECS. But overall, ECS introduces some nice benefits once the programmer wraps their head around it.

## 2.3 Rust

Rust is the programming language being leveraged to build the flight dynamics models. It is the language of choice given its advantageous features: memory safety and performance. Rust was released in 2015 by Mozilla, which has been behind the language’s success. Rust will guarantee that a program will be memory safe, with no invalid data accesses, all while remaining just as fast as other popular programming languages [1]. The C++ programming language, for example, is a systems level language, giving developers complete access to data inside memory, with no memory safety precautions. This is only great when the programmer is an expert on writing code that is free of memory issues – this is hard to do. On the other hand, the Python programming language takes away a developer’s access of low level memory, but does provide complete memory safety by using a garbage collector to clean up unneeded memory – but this imposes a runtime cost [1]. Rust is unique because it is a systems level language, it does not have a garbage collector, and it will still guarantee memory safety. This means Rust provides control, is fast, and protects against memory issues such as data races, and dangling pointers [1]. These features built-in to Rust make it the choice programming language for building a FDM.

The safety assurance of Rust is accomplished by its system of Ownership, Borrowing, and Lifetimes. All languages have a way to manage memory while a program is running. Some languages have a garbage collector which actively checks for memory that is no longer being used during runtime. Other languages make the programmer manually allocate and then free memory. Rust takes neither approach; it instead has something called a “borrow checker.” At compile time, the Rust borrow checker will check that all accesses to data is legal and valid [1]. The borrow checker deals with three important Rust concepts: Ownership, Borrowing, and Lifetimes.

### 2.3.1 Safe Rust and Unsafe Rust

Before we begin to discuss what specific features make Rust a “safe” programming language, it is important to know that there is an “unsafe” side of Rust, like C++. By safe, it is meant that the code is completely bug free of memory issues, and undefined behavior such as data races (two processes writing data in the same memory location at the same time), and dangling references (accessing unintended data). The unsafe side of Rust will let the programmer do things that are not safe. And sometimes it is necessary to do unsafe things. For example, sometimes it is required to go into unsafe mode to dereference raw pointers, which are possibly invalid, or out of scope at access [24]. In safe Rust, this is impossible to do because of the protection feature implemented called Lifetimes. The other implemented features, Borrowing and Ownership, also empower safe Rust. We will discuss these features in this chapter now. Overall, the great thing about Rust is that it allows the programmer the choice to write code in safe mode or unsafe mode, whereas in other languages, that mode is unchangeable.

### 2.3.2 Ownership

Ownership refers to the fact that every use of a value needs to be valid and cleaned up when it is done being used. Ownership is accomplished with three rules that the programmer must follow in Rust [2]:

- Every value has a variable which is called its owner

- There may be only one owner at a time

- When an owner goes out of scope, the value is dropped

### 2.3.3 Borrowing

Borrowing is the next important concept in Rust. Borrowing allows data to be “borrowed” from one owner (variable binding) to another owner and accessed for a moment without actually changing Ownership of that data. When Ownership of memory is transferred to another owner, the original owner cannot legally be used, so Borrowing is the solution. The borrow checker will statically guarantee that references always point to valid objects. So, if a reference to an object exists, it cannot be destroyed. There are two rules to Borrowing [2]:

- Any borrow must last for a scope no greater than that of the owner,

- You may have one or the other of these two kinds of borrows, but not both at the same time:

- One or more references to a resource

- Exactly one mutable reference

These rules avoid the possibility of a data race because although there can be more than one pointer to a resource, only one can be accessing and changing that resource at a time. This is good for performance, and uses less memory overhead [1].

### 2.3.4 Lifetimes

The next important Rust concept is Lifetimes. A value has a Lifetime, which is the period that accessing that value is valid to do [2]. A Lifetime starts when a value is created and that Lifetime ends when it is destroyed. Rust’s compiler, or borrow checker, is smart enough to know a values Lifetime in many common circumstances [3]. In these common circumstances, the Lifetime of a value does not need to be explicitly written, but otherwise, a Lifetime parameter needs to be written. The borrow checker tries to limit the period of a Lifetime. A value should not be alive any longer than it absolutely needs to be because, otherwise, it could be referenced by accident. This could cause a dangling reference, which is a reference to unintended data [1]. So, to keep some piece of data alive for only as long as completely necessary, a Lifetime is specified for that data.

### 2.3.5 Crates

Crates are essentially libraries that can be added to a Rust project for additional functionality. In this thesis, we use multiple crates to code the final result, but the most notable crate is SPECS, which is a framework to easily program the ECS design in Rust.

The Rust community is also active and quickly growing. The Rust ecosystem is also rapidly expanding, with now over 46,000 crates available to download [19]. Rust makes it easy to import these crates into code and begin using them, and the crate to implement ECS into code, SPECS, is no exception. Only two steps are needed to get started with a crate. First, add the crate dependency to the cargo.toml file, which is in every Rust project. Second, add a line at the top of the main.rs to indicate that this crate is being used. Now the desired crate can be utilized in code.

### 2.3.5 Rust Overall

Overall, Rust can hold its own against more popular languages, and the learning curve of the language is met with staggering benefits in safety and performance. And with safety dealt with by the borrow checker at compile time, the programmer may be at ease when it comes to memory safety. At the same time, there is no cost in performance at runtime because everything is checked at compile time without a garbage collector. The downsides to Rust are minimal, but they do exist. For one, compile time is longer. Also, the compiler is very strict. New Rust programmers often “fight with the borrow checker” to get their code to compile because it is more difficult to write code that abides by Rusts rules [1]. So less time is spent fixing bugs, and more time is spent getting the compiler right. But the compiler is like a safety net because once the code compiles, the programmer knows the result is safe and correct.

## 2.4 FlightGear

FlightGear is an open-source flight simulator application. However, in this thesis, it is being used solely as visual system so as to avoid having to handle graphics generation. FlightGear, although it has its own built-in FDM, is easily configurable to instead use an external, custom FDM. FlightGear can be configured to be interfaced, or communicated with, via network packets to achieve a desired display output. In this case, these packets of data will be sent from the ECS patterned, Rust application to FlightGear and the packets will contain all of the real-time physics calculations of the FDM. FlightGear will interpret these packets to then know how to display the subsequent simulation to the screen.

To properly configure FlightGear for the simulation, it must manually be told to accept the new external FDM, as well as listen for the respective packets [5]. This is done from the command prompt upon running the executable. The command line arguments are listed in Appendix A: Configuring FlightGear as a Visual System.

With FlightGear configured, it will process the received network packets constructed by the Rust application. But FlightGear requires that these packets are defined in a particular format so as to understand them. The format of the packet, before being converted to bytes, must be a struct, which is outlined in the FlightGear source code. The struct contains all of the data needed to define an airplane and how it flies throughout the simulation. This struct, called FGNetFDM, in Rust code is located in Appendix A: Configuring FlightGear as a visual System.

## 2.5 Networking with Rust

Now, it is understood that networking is needed to send packets of data to FlightGear so that it can display the desired simulation. Networking can simply be defined as two programs communicating with each other, sending and receiving data. To communicate with each other, two sockets are required at each end of communication. In this case, the type of socket being used is the Datagram socket. The protocol involving Datagram sockets is called the User Datagram Protocol (UDP). This protocol is considered “connectionless” because an open connection does not need to be maintained.  Packets containing bytes of data may simply be built and sent to a destination without a connection needing to be established prior [7].

In Rust, a UdpSocket can be created and binded to an internet protocol address and port on the local machine. After this, data can be sent and received from any other socket address. The socket created can then be connected to the other socket with its internet protocol address and port. From here, data can be sent, and received with a function call [2]. One important thing to be aware of is that before data packets are sent out to FlightGear, the literal order of the bytes need to be converted from least significant byte first (little endian), to most significant byte first (big endian) [7].

# III.  Methodology

This chapter will discuss the research methods used in this thesis, including what has been accomplished and how it has been accomplished. We will describe the process of building the FDM with SPECS, and how we verified that it does work as expected.

## 3.1 Building the FDM Overview

The approach to building the Rust based, ECS patterned, 3D FDM began with an acquiring an understanding of basic flight physics and the Rust programming language. With those basics understood, the first goal was to build a 2D version because it was inherently less complicated because the physics are simpler. The textbook, Physics for Game Programmers, by Grant Palmer was used to base our 2D FDM on. The textbook provided the C code for his 2D FDM. This FDM was then ported from C into Rust. From here, the 2D FDM was restructured into an ECS design using the SPECS Rust crate, which made this process straightforward. The building of this 2D FDM was essentially a stepping stone to reach the goal of creating the 3D version. We mention this progress, or stepping stone, because the knowledge gained up to this point was indispensable for the main goal in terms of practicing physics, ECS, and Rust.

The 3D FDM is based on the works by David Bourg and his textbook, Physics for Game Developers, which outlines a 3D FDM in C++ with an OOP design. Similar to before, this C++ FDM from Bourg was ported into Rust. From here, it was converted into an ECS design with the help of the SPECS crate. Ideally, one would begin programming their software from the ground up as an ECS. But, knowing we would need the OOP version in Rust regardless for the upcoming performance experiment, it needed to be written. Also, it was simpler to convert the code directly from C++ to Rust in the same OOP design before it would be broken up into the ECS.

Bourg defines a series of classes in C++ that are used to represent a custom vector, quaternion, and matrix. These classes were not originally planned to be used because Rust has a crate available called nalgebra that defines already those structures, and we did not want to reinvent the wheel if we did not need to. However, to no avail, nalgebra did not work out in the end and we fixed the problems with rotations by staying true to the Bourg model. So, the custom classes defined by Bourg were reinterpreted in Rust and used in our FDM. The module that was imported in code which contains the custom structures is named common.

In regard to writing the FDM code, there were very limited resources available describing the process of writing code as an ECS, especially in Rust. The resources used in this thesis boil down to YouTube videos sponsored by Unity, a high-profile proponent of the ECS architecture, and The Specs Book, which is the official documentation on the SPECS framework for Rust. There were many more options to reference for writing Rust code. But the two resource used most for writing Rust code in this thesis was the official Rust book, The Rust Programming Language, and the official code collection called Rust By Example, which showcases executable code snippets.

## 3.2 SPECS Set Up

The Rust crate SPECS was an invaluable framework to implement the FDM as an ECS design. Since examples and resources were limited, The Specs Book was the main resource used, which provided just enough substance to get started.

SPECS laid the framework to create the Entities, Components, and Systems. Components were created similarly to how a regular Rust struct type is created, but SPECS required us to specify that it is being implemented as a Component using a vector as its storage type. Since Entities have Components, when an Entity was made, it must be specified what Components it will have and what those starting values are. From here, we needed Systems to act on the Entities. Systems were created similarly to functions, but when created, it was specified what Components the System needed access to. Also, the Components specified are given either WriteStorage access or ReadStorage access, meaning they can be mutated or not, respectively. This specification of Components for the System also determined what Entities the System would work on. If an Entity had all the Components also specified in a particular System, that System would work on that Entity.

With all that said, in the main of the program, the simulation world was created. The components were registered in the world. A dispatcher was created in code to manage the parallelization of the Systems. The dispatcher can be told to have a System depend on other Systems (i.e. a system must finish before another system begins). Then, the Entities were built with some predetermined starting values as needed. Finally, the main simulation loop was made to run the world at a predetermined frame rate, and the program sleeps if there is extra time after computing that frame of the simulation.

To ensure numerical stability, the time step chosen for the simulation loop inside the main of the FDM was 33 milliseconds. This means that the simulation steps through 33 milliseconds of time from frame to frame. This equals 30 frames per second. With this time step, the simulation is stable. This simulation loop strategy makes frames per second dependent on a constant game speed (33 ms) [28]. This basically means we have a fixed change in time from tick to tick. This strategy was chosen for simplicity sake. This strategy works because we know that the actual integration of the equations of motion reliably takes less time than to update than the chosen time step, and so the simulation will be able to keep up, and maintain a smooth frame rate [27]. This strategy might normally be an issue for slow computers, but the FDM is not very demanding on hardware. The downside is that for fast computers, we leave some frames per second on the table because we have a fixed time step [28]. To make the frame rate or time step easy to modify in code, there is a global variable called FRAME_RATE which may be modified. There is also a global variable representing time step which is DT (Delta Time). The DT depends on the FRAME_RATE value to be calculated (DT = 1.0 / FRAME_RATE).

## 3.3 SPECS Components

Components contain the data variables to be accessed by a System, if required. The three Components we have created are: KeyboardState, RigidBody, and Packet.

### 3.3.1 KeyboardState

The KeyoardState Component holds the boolean variables that correspond to the action that will take place in the simulation when a specified key is pressed on the keyboard. This Component is used by the EquationsOfMotion System and the FlightControl System.

### 3.3.2 RigidBody

The RigidBody Component holds all of the data variables describing an airplane entity, which is a rigid body by definition. This Component is used by the EquationsOfMotion System and the MakePacket System.

### 3.3.3 Packet

The Packet Component consists of a vector which holds the serialized, or made into bytes, FGNetFDM struct, which contains all of the data necessary to be sent to FlightGear in the form of a packet. The FGNetFDM structure that FlightGear accepts is laid out in FlightGear’s GitHub repository. The source code for the structure is located in src/Network/net_fdm.hxx. This Component is used by the MakePacket System and the SendPacket System.

## 3.4 SPECS Systems

There are four Systems required to manage all of the functionality: FlightControl, EquationsOfMotion, MakePacket, and SendPacket. Systems access certain Components that are needed for reading or writing to, and the type of access must be specified. ReadStorage is just read access to the Compoment data, and WriteStorage is reading and also writing over the Component data.

### 3.4.1 FlightControl

This System ultimately handles getting the keypresses from the keyboard and then alters the KeyboardState Component data. So, it will be given WriteStorage access to KeyboardState.

To actually detect the keyboard keypresses, we implemented a Rust crate called Device Query, which is able to read the current state of the keys on the keyboard (i.e. key is being pressed or not). In code, Device Query gets list of all the keys being pressed at the current instance, and we check if any of the keys in that list match the keys that we have tied with a function in our FDM. Since more than one key can be pressed at once, we check all of the keys that we care about, and we toggle true the KeyboardState data that corresponds to that key press. Note: before we toggle to true any of the KeyboardState data, we default the data to false. With all of the states set for this instance, the EquationsOfMotion System will take care of the rest, until the next iteration.

### 3.4.2 EquationsOfMotion

This System is essentially all of the guts of the FDM. This System requires ReadStorage access to KeyboardState and WriteStorage to RigidBody.

The System begins by reading the KeyboardStates and if a state is toggled true, the RigidBody is modified in a certain way depending on the key. For example, if the thrust_up state is true for this instance, then the thrust force is increased, and the FDM will take that into account for the upcoming calculations. That is the part that makes the FDM interactive. Following accounting for keyboard input, this System calculated the loads, or forces, acting on the airplane. From here, the acceleration of the airplane is calculated in Earth space. Then, the velocity of the airplane is calculated in Earth space. The velocity is then directly used in the calculated of the position coordinates in Earth space. In terms of rotations, the angular velocity in body space is then computed. The angular velocity is then used to determine the quaternion, which can be extrapolated to give us the Euler angles. Finally, the airspeed is calculated by getting the magnitude of the velocity. These are all of the steps taken in the System to compute the equations of motion in the FDM.

### 3.4.3 MakePacket

This System is what turns the FDM into a functional flight simulator because it evokes the graphics of FlightGear. It is responsible for sending packets of data to FlightGear via a network socket. The packets contain the data required for FlightGear to know how to position the plane in the simulation, and how to orient the plane accordingly. It accesses both the RigidBody Component and the Packet Component as ReadStorage.

The Packet Component essentially holds a vector which contains the FGNetFDM structure serialized into bytes. Inside of this System, an instance of the FGNetFDM structure is created and updated with the current calculations from the EquationsOfMotion System, which are stored in the RigidBody Component data. But when the new data variables are passed into the FGNetFDM structure, the data has to be first converted into an array of bytes in native byte order, meaning the byte order native to the machine compiling the software. From here, the byte array is then read as a big endian representation and turned into an integer value that FlightGear can interpret.

The data updated is the position of the airplane in space, and also the orientation of that airplane. The FGNetFDM position data updated is latitude, longitude, and altitude. The orientation variables updated are phy, theta, and psi, which represents the roll, pitch, and yaw, respectively. It is updated with the Euler angles. The FlightGear version is also required to be passed on to the FGNetFDM struct. Once all of the FGNetFDM data has been updated with new values from the current RigidBody Component, and converted to big endian byte order, the data can be arranged into a packet of u8 bytes in the form of a vector, and passed into the Packet Component.

### 3.4.4 SendPacket

With the packet constructed and updated, this System simply serves to send that packet off to FlightGear via a socket connected on its port. This system only needs read access to the Packet Component.

## 3.5 FDM Equivalency Verification

Since the focus of this research was building a Rust based, ECS patterned, 3D FDM based on the works by David Bourg in [14], we needed to find out if it indeed works properly. When flying the airplane in FlightGear, from the naked eye, the flight mechanics look to be correct. But to ensure our FDM’s works exactly as expected, we investigated if it replicates the same realistic flight data generated by its C++ benchmark FDM as described in Bourg’s textbook. This investigation ran a series of different flight tests between both FDMs with the objective to check if our FDM’s generated flight data was equivalent to the benchmark in multiple scenarios. The flight data used in the comparison was: position coordinates, Euler angles, and speed. This data represents exactly where the airplane is at a given time, and how it is oriented.

For each set of tests, both the benchmark FDM and our FDM was compiled and ran with identical starting parameters. The initial parameters include: coordinate position, speed, velocity, forces, and execution time. These parameters were altered for each set of tests. Also, for each set of tests, the two FDMs were given the same flight control maneuvering instructions for the airplanes to follow. The flight data at the end of the execution time was recorded for each FDM.

The benchmark C++/OOP FDM was compiled with Visual Studio 2019 Community Edition, and the Rust/ECS FDM was compiled with rustc version 1.47.0. The tests were performed on a Lenovo ThinkPad T440p machine, running Windows 10, with an intel i74800mq processor.

### 3.5.1 Test One

| Variables | Initial Value |
| --- | --- |
| Position (x, y, z) | -5000.0, 0.0, 2000.0 |
| Velocity (x, y, z) | 60.0, 0.0, 0.0 |
| Forces (x, y, z) | 500.0, 0.0, 0.0 |
| Speed (knots) | 60.0 |
| Execution Time (s) | 30 |

| Flight Control Instructions | None |
| --- | --- |

|  | Rust/ECS Output | C++/OOP Output |
| --- | --- | --- |
| Position (x, y , z) | -1045.2231130699374, 0.008302614662798928, 420.11927606534243 | -1045.37, -0.0444084, 421.399 |
| Euler Angles (roll, pitch, yaw) | 0.002817044095946979, -16.14774176260282, 0.004196830341169564 | -0.00279299, -16.1436, -0.00508352 |
| Speed (knots) | 85.98892180235976 | 85.9906 |

### 3.5.2 Test Two

# IV.  Analysis and Results

Application built together to make something fly. Compared against ecs.

Rigidbody has a position lumped in. Difference in displacement was position. Does it exactly how bourg does it, maybe could be made better. What I saw after doing all this

## Benchmark Comparison

This test was a success because our FDM flight data matched the baseline flight data on a predetermined flight path, ensuring our FDM works as it should.

## Investigative Questions Answered

Text

## Summary

Text

# V.  Conclusions and Recommendations

## Chapter Overview

Text

## Conclusions of Research

Text

## Significance of Research

Text

## Recommendations for Action

Text

## Recommendations for Future Research

Text

## Summary

Text

# Appendix: Configuring FlightGear as a Visual System

fgfs.exe – runs the executable
--aircraft=ufo – select an aircraft name
--disable-panel – disable instrument panel
--disable-sound – disable sounds
--enable-hud – enable heads up display 
--disable-random-objects – disable random scenery objects
--fdm=null – turn off the built-in FDM
--vc=0 – specify initial airspeed
--timeofday=noon – time of day 
--native-fdm=socket,in,30,,5500,udp – sets up FlightGear to receive data from an external source. Open a socket, which takes in packets at a speed of 30 hz on port 5500 [6].

Altogether, when running FlightGear, the command line is:

fgfs.exe --aircraft=ufo --disable-panel --disable-sound --enable-hud --disable-random-objects --fdm=null --vc=0 --timeofday=noon --native-fdm=socket,in,30,,5500,udp

(change to 60 hz?)

The struct representing an airplane in FlightGear, written in Rust:

struct FGNetFDM

{

version: u32, // increment when data values change

padding: f32, // padding

// // Positions

longitude: f64, // geodetic (radians)

latitude: f64, // geodetic (radians)

altitude: f64, // above sea level (meters)

agl: f32, // above ground level (meters)

phi: f32, // roll (radians)

theta: f32, // pitch (radians)

psi: f32, // yaw or true heading (radians)

alpha: f32, // angle of attack (radians)

beta: f32, // side slip angle (radians)

// // Velocities

phidot: f32, // roll rate (radians/sec)

thetadot: f32, // pitch rate (radians/sec)

psidot: f32, // yaw rate (radians/sec)

vcas: f32, // calibrated airspeed

climb_rate: f32, // feet per second

v_north: f32, // north velocity in local/body frame, fps

v_east: f32, // east velocity in local/body frame, fps

v_down: f32, // down/vertical velocity in local/body frame, fps

v_body_u: f32, // ECEF velocity in body frame

v_body_v: f32, // ECEF velocity in body frame

v_body_w: f32, // ECEF velocity in body frame

// // Accelerations

A_X_pilot: f32, // X accel in body frame ft/sec^2

A_y_pilot: f32, // Y accel in body frame ft/sec^2

A_Z_pilot: f32, // Z accel in body frame ft/sec^2

// // Stall

stall_warning: f32, // 0.0 - 1.0 indicating the amount of stall

slip_deg: f32, // slip ball deflection

// // Pressure

// // Engine status

num_engines: u32, // Number of valid engines

eng_state: [f32; 4], // Engine state (off, cranking, running)

rpm: [f32; 4], // // Engine RPM rev/min

fuel_flow: [f32; 4], // Fuel flow gallons/hr

fuel_px: [f32; 4], // Fuel pressure psi

egt: [f32; 4], // Exhuast gas temp deg F

cht: [f32; 4], // Cylinder head temp deg F

mp_osi: [f32; 4], // Manifold pressure

tit: [f32; 4], // Turbine Inlet Temperature

oil_temp: [f32; 4], // Oil temp deg F

oil_px: [f32; 4], // Oil pressure psi

// // Consumables

num_tanks: u32, // Max number of fuel tanks

fuel_quantity: [f32; 4],

// // Gear status

num_wheels: u32,

wow: [f32; 3],

gear_pos: [f32; 3],

gear_steer: [f32; 3],

gear_compression: [f32; 3],

// // Environment

cur_time: f32, // current unix time

warp: f32, // offset in seconds to unix time

visibility: f32, // visibility in meters (for env. effects)

// // Control surface positions (normalized values)

elevator: f32,

elevator_trim_tab: f32,

left_flap: f32,

right_flap: f32,

left_aileron: f32,

right_aileron: f32,

rudder: f32,

nose_wheel: f32,

speedbrake: f32,

spoilers: f32

}

# Bibliography

[1] T.S. McNamara. Rust In Action, 2019.

[2] Steve Klabnik and Carol Nichols. The Rust Programming Language, 2019.

[3] Richard Apodaca. Rust Ownership By Example, 2020. [Online; accessed 07/01/2020.

[4] Oliver Gould and Lachie Everson. “All About Rust.” YouTube, uploaded by Microsoft Developer, 2019.

[5] wiki.flightgear.org. Command line options, 2019. [Online accessed 07/02/2020.

[6] wiki.flightgear.org. Property Tree/Sockets, 2019. [Online accessed 07/02/2020.

[7] Brian Hall. Beej’s Guide to Network Programming Using Internet Sockets, 2019.

[8] Mike Geig. “ECS Overview - Intro To The Entity Component System And C# Job System 1/5.” YouTube, uploaded by Unity, 2018.

[9] Ben. “Entity Component System Overview in 7 Minutes.” YouTube, uploaded by Board To Bits Gaming, 2019.

[10] Adam. Entity Systems are the future of MMOG development – Part 2, 2007. . [Online; accessed 07/01/2020.

[11] Mike Geig. “ECS Overview - Intro To The Entity Component System And C# Job System 2/5.” YouTube, uploaded by Unity, 2018.

[12] Thomas Schaller. The Specs Book. Crates.io, 1st edition, 2018.

[13] wow what was this source… (fidelity paragraph)

[14] David Bourg and Bryan Bywalec. Physics for Game Developers, 2nd Edition, 2013.

[15] https://unity.com/solutions/game. Create a world with more play, 2020. [Online accessed 09/01/2020.

[16] https://unity.com/case-study/mediatonic-fall-guys. Unity helps bring the masses together in Mediatonic’s absurd multiplayer, 2020. [Online accessed 09/01/2020].

[17] Albert Rhemann. A Handbook of Flight Simulation Fidelity Requirements for Human Factors Research, 1995.

[18] Gaétan Dussart, Vilius Portapas, Alessandro Pontillo and Mudassir Lone. Flight Dynamic Modelling and Simulation of Large Flexible Aircraft, 2018.

[19] https://crates.io/. The Rust’s Community Crate Registry. [Online accessed 09/01/2020.

[20] Wayne Durham. AIRCRAFT FLIGHT DYNAMICS AND CONTROL, 2013.

[21] Vinayak D. “What is Flight Dynamics? - Derivation of Equations of Motion for an Aircraft.” YouTube, uploaded by VDEngineering, 2019.

[22] https://en.wikipedia.org/wiki/Moment_of_inertia. Moment of Inertia, 2020. [Online accessed 09/01/2020.

[23] https://en.wikipedia.org/wiki/Airfoil. Airfoil, 2020. [Online accessed 09/01/2020.

[24] Steve Klabnik and Carol Nichols. The Rustonomicon, 2019.

[25] D. Rose. Converting Between Earth-Centered, Earth-Fixed and Geodetic Coordinates, 2014.

[26] gaffer on games integration basics

[27] gaffer on games fix your loop

[28] dewitters game loop

[29] HISTORY OF UNITED STATES MILITARY SIMULATION  Raymond R. Hill, J. O. Miller

[30] grant palmer

[31] rust by example https://doc.rust-lang.org/stable/rust-by-example/

[32] https://gameprogrammingpatterns.com/game-loop.html

[33] http://wiki.flightgear.org/Flight_Dynamics_Model

[34] vagedes

Fgnetfdm struct

https://github.com/FlightGear/flightgear/blob/8e6500ae47ff74539cd2c0a9c4eebe96e69ca81d/src/Network/net_fdm.hxx

# Acronyms