# A RUST BASED, ECS PATTERNED, INTERACTIVE FLIGHT SIMULATOR

# THESIS

# Chad A. Willis, 2d Lt, USAF

# AFIT-ENG-MS-21-M-XXX

# DEPARTMENT OF THE AIR FORCE

# AIR UNIVERSITY

###### AIR FORCE INSTITUTE OF TECHNOLOGY

# Wright-Patterson Air Force Base, Ohio

# DISTRIBUTION STATEMENT A.

# APPROVED FOR PUBLIC RELEASE; DISTRIBUTION UNLIMITED.

# The views expressed in this thesis are those of the author and do not reflect the official policy or position of the United States Air Force, Department of Defense, or the United States Government.  This material is declared a work of the U.S. Government and is not subject to copyright protection in the United States.
AFIT-ENG-MS-21-M-XXX

# A RUST BASED, ECS PATTERNED, INTERATIVE FLIGHT SIMULATOR

THESIS

Presented to the Faculty

Department of Aeronautics and Astronautics

Graduate School of Engineering and Management

Air Force Institute of Technology

Air University

Air Education and Training Command

In Partial Fulfillment of the Requirements for the

Degree of Master of Science in Computer Science

Chad A. Willis, BS

2d Lt, USAF

March 2021

# DISTRIBUTION STATEMENT A.
APPROVED FOR PUBLIC RELEASE; DISTRIBUTION UNLIMITED.

AFIT-ENG-MS-21-M-XXX

# A Rust based, ecs patterned, interactive flight simulator

# THESIS

Chad A. Willis, BS

2d Lt, USAF

Committee Membership:

Dr. Douglas D. Hodson, PhD

Chair

Maj. Richard A. Dill, PhD

Member

Dr. Scott L. Nykl, PhD

Member

AFIT-ENG-MS-21-M-XXX

# Abstract

This thesis explores the Entity-Component-System (ECS) software design pattern for use in military computer training-based flight simulation, by attempting to implement a 3-dimensional (3D) flight dynamics model (FDM) using this design pattern. Through the learning process of building this 3D flight dynamics model, we also build a 2-dimensional (2D) FDM based off the works of Grant Palmer, who authored the textbook Physics for Game Programmers. However, this thesis revolves around building a 3D FDM, which is based off the works of David Bourg in his textbook titled Physics for Game Developers. The final product 3D FDM takes into account various principles of flight explained in these books such as forces, moments, equations of motion, and quaternions, all of which assist in replicating realistic flight physics.

The choice design pattern of the FDM software is the modern ECS architecture, as opposed to other software programming paradigms, such as Object Oriented Programming (OOP). The ECS design, which focuses on data composition instead of data hierarchies, was chosen because of the performance benefits and improvements in the ability to easily modify and add code when compared to OOP.

Rust is programming language of choice for building the FDM. Rust is a new systems programming language, with the first stable release in 2015, which was designed on the basis of memory safety and performance. Rust can guarantee software is completely free of all memory issues, while remaining highly performant. This choice of programming language was made to leverage the useful features which will be explained.

A flight simulator application named FlightGear is used exclusively as a visualization system for the simulation. The FDM is built to send network packets to FlightGear that contains the data it needs to generate the simulation to the screen. When the interactive simulation is executed, the user can control the airplane with keyboard input.

A series of experiments is developed to investigate the performance difference in the Rust based, 3D ECS FDM program versus the same FDM program, but organized as a single threaded OOP. The experiments incorporate various amounts of flying and maneuvering airplane entities in the simulation world, and challenges both FDM programs to process them as quickly as possible for a finite amount of time. The results indicate that when the simulation is scaled larger with more entities that…

# Table of Contents

Page

Abstract	1

Table of Contents	3

List of Figures	4

I.  Introduction	5

1.1 Problem Statement	9

1.2 Research Objective	9

1.3 Hypothesis	10

1.4 Contribution	10

1.5 Approach	11

1.6 Assumptions/Limitations	11

1.7 Thesis Overview	12

II. Background	12

2.1 Physics of Flight Modeling	13

2.2 Entity Component System	22

2.3 Rust	26

2.4 FlightGear	30

2.5 Networking with Rust	31

III.  Methodology	32

3.1 Building the FDM	32

3.2 The FDM Works	33

3.2 ECS and OOP Performance Experiment	34

IV.  Analysis and Results	34

Chapter Overview	34

Results of Simulation Scenarios	34

Investigative Questions Answered	34

Summary	35

V.  Conclusions and Recommendations	35

Chapter Overview	35

Conclusions of Research	35

Significance of Research	35

Recommendations for Action	35

Recommendations for Future Research	35

Summary	35

Appendix: Configuring FlightGear as a Visual System	36

Bibliography	39

# List of Figures

Figure 1: Forces Visualized	13

Figure 2: Airplane Components	14

Figure 3: Airfoil Cross-section	15

Figure 4: Airplane Axes of Rotation	16

Figure 5: ECEF Coordinates Visualized	17

Figure 6: Lift Coefficient vs Angle of Attack	19

Figure 7: Entity Component System Visualized	23

# I.  Introduction

Computer training-based simulations are important tools for the military. They are used in order to be prepare all servicemen and servicewomen for a possible conflict. Through training with computer simulations, the military practices strategies that hope to achieve a positive outcome during a real-life conflict [29].  Purssian Field Marshal Helmuth von Moltke, regarded as the grandfather of the modern military simulations, once asserted that “No Battle Plan Survives Contact with the Enemy” [29]. This assertion, however, only reinforces the need to train with simulations.

The reasons to use a virtual computer simulation over real-life simulation is because it is less expensive, more accessible, more versatile, and completely safe. Virtual simulations are less expensive because you do not have vehicles, like airplanes, burning gas and requiring manpower, like maintenance. They are more accessible because all you need is a computer to use it. They are more versatile because you can simulate scenarios that might not be possible to put together in real-life. And finally, they are completely safe because users cannot hurt themselves.

Today, military computer training-based simulations exist to simulate everything that could take place at war, such as troops on the ground. But in this thesis, we focus on computer-based 3-Dimensional (3D) flight simulations that can simulate multiple airplanes. These kinds of simulations are not only used for full-scale aerial battle planning, but also to sharpen an individual pilot’s skills.

The piece of a flight simulation software that actually calculates how an airplane should fly in real-life is referred to as a flight dynamics model (FDM), which is essentially all of the math that makes an airplane act like an airplane. As a plane is flying, many things are happening. The pilot of the airplane, or in this case, the simulation user, could be deciding to accelerate, turn, and change altitude, sometimes all at once. With that user input, the FDM calculates and determines how the plane should react, while taking into account external forces, like gravity, as well as the physical properties of the airplane. The term FDM will be used when referring more specifically to the physics piece of the whole flight simulator.

Although the software engineering of FDMs has been solved, and the math to mimic realistic physics is a standard practice, this thesis describes building a FDM that is unlike any other in terms of how the code is organized. A lot of the time, FDMs are written in a systems level programming language, specifically C++, and organized in code as an Object-Oriented Programming (OOP) design pattern. But a military flight simulation could consist of one, or hundreds, or even thousands of airplanes flying around in a space. And complex simulations like this which have a FDM constructed as OOP are missing out on some powerful modern features that this thesis seeks to elucidate. This thesis explores building an interactive flight simulator using two alternate technologies that provide some useful benefits over more typical programming methods.

The first technology used in this thesis is the Entity Component System (ECS) programming design pattern. In the gaming industry, the implementation of ECS in video games and simulations is growing in popularity. Notably, Unity uses an ECS architecture in its game engine. Unity is a platform for creating and operating real-time 3D games [15]. The massively popular battle royale game Fall Guys: Ultimate Knockout, for example, was created with Unity, which relies on the ECS design [16]. In this thesis, the leveraging of the ECS design in a military oriented simulation is of particular interest because it has never been done before. The ECS architecture is simply a new way of organizing data, alternatively to using Object-Oriented Programming (OOP) to organize data. ECS is referred to as a Data-Oriented Design (DOD). Where OOP relies on hierarchies of data, ECS relies on compartmentalization of data, where data content and its functionality on the data are separated. This design, which is discussed in more detail in Chapter 2, leads to code that is more reusable, readable, and performant. But we will also discuss any possible pitfalls in the software created.

Along with ECS, the second technology choice used in this thesis is the Rust programming language. Rust, created in 2015, introduces a combination of important features not available in any other language. These provided features stem from two concepts that Rust focuses on: memory safety and performance. Rust will always guarantee memory safety, avoiding issues such as data races, dangling references, and buffer overflows in software. It accomplishes this safety by checking that memory is squared away and cleaned up at compile time. Other languages cannot make this guarantee because they rely on a highly-experienced programmer to be sure the software is memory safe. Although some languages are also memory safe like Rust, they lack in performance in order to accomplish memory safety. This is because they use a garbage collector at runtime to ensure memory safety – this not ideal for performance. Rust’s performance, however, is a positive result of the way it tackles memory issues because it does not need a garbage collector and checks for issues at compile time.  These features are undoubtedly worthwhile to take advantage of in a flight simulator. But Rust is not without drawbacks compared to other languages, which we will discuss. In order to implement the ECS design within the Rust language, the framework, SPECS, is the library, or, as the Rust programming language calls it, crate, that is imported in code.

## 1.1 Problem Statement

In real-time military flight simulations, it is common to see a 3D FDM using a single threaded OOP design approach, and be written in a systems level programming language like C++. It is likely that the simulation could have hundreds or even thousands of airplanes or entities flying and maneuvering at one time. In a complex simulation like this, a systems language, single threaded OOP is faced with some problems. With OOP, runtime performance can seriously decrease with increasing entities to process at once on a single thread. Also, the entanglement of functionality and data memory in hierarchies can cause a mess when trying to modify or add functionality and entities. With systems programming languages, it is also all too easy to introduce memory vulnerabilities in code. An expert programmer would be required to ensure that memory is cleaned up properly.

(so should I not mention the ease of code modification with ecs, and also not mention the ensurance of no memory issues with rust? And just focus on performance of ecs vs single threaded OOP?)

## 1.2 Research Objective

The modern ECS architecture is a viable alternative which can provide some important advantages over OOP in such a simulation: runtime performance, and code modifiability. Furthermore, the Rust programming language solves the memory safety burden that plagues systems level languages. The goal of this research is to build an interactive 3D FDM implemented as a Rust based, ECS design. An additional objective is to investigate a performance difference between a single threaded OOP design implementation of the goal FDM. The FDM is based on the works by David Bourg in [14].

## 1.3 Hypothesis

The thesis hypothesizes that a 3D FDM implemented as a Rust based, ECS design will be able to more quickly process a large amount of airplane entities than the same FDM organized with the single threaded OOP paradigm. By using ECS to organize data differently, we can separate, or decouple, functionality from the actual data in memory, as opposed to having functionality and memory tied together in hierarchies. This is powerful and popular strategy in designing complex video game engines, which have many game entities, such as bullets, and many functions on those game entities. This decoupling allows for the functionality on game entities to be easily parallelized and fast. In addition to performance, because ECS decouples code, the ability to modify existing functionality or entities, or add new functionality or entities is a much smoother process. This will be explained in more detail in the next chapter. So, for the same reasons game designers choose to use the ECS design in a video game, we can also benefit for the use of ECS in our FDM powering the simulation created in this thesis.

## 1.4 Contribution

The contribution of this thesis lies in the field of flight modeling and simulations for military use. The successful build of a 3D Rust based, ECS designed, real-time FDM demonstrates an alternate, modern, and powerful design strategy over current FDMs. Not only is this strategy available for use, but it consistently outperforms its single threaded OOP counterpart, which is a representative of current military FDMs. This strategy also increases code maintainability due to the nature of the ECS architecture; this is a valuable feature that could save time and effort when updating a FDMs code later in its lifespan.

## 1.5 Approach

The methodology of this research is explained in full detail in Chapter 3, but an

overview of the approach will be given here.

Firstly, a 2D, Rust based, ECS patterned FDM is created based on the works by Grant Palmer in [30]. The building of this 2D FDM was essentially a stepping stone to reach the goal of creating the 3D version.

Next, the focus of this research is building a Rust based, ECS patterned, 3D FDM based on the works by David Bourg in [14]. To ensure our FDM’s accuracy, we will find out if it replicates the realistic flight data generated by its C++ baseline FDM as described in Bourg’s textbook.

With the proof of concept shown, we will then investigate a performance difference between the 3D FDM implemented as a Rust based, ECS design and an OOP design implementation of the same FDM.

## 1.6 Assumptions/Limitations

The first limitation is that the flight simulator built is not a standalone application; a graphics generation program is needed to display the simulation. However, if a user does require graphics, the flight simulator can be run without an external visualization system. The simulator does assume that FlightGear is being used as the visualization system. The packets being sent out by the flight simulator are configured in a specific manner for FlightGear to interpret properly. The packet structure, however, could be easily modified for different visualization systems.

Another limitation is that the airplane being simulated is only set up to match that of the airplane modeled in Bourg’s textbook. This no-name airplane is shown in all of the figures shown in the next chapter. The airplane characteristics could be modified in the code, however.

(if I cannot find out how to simulate multiple planes in flightgear another limitation may be that for a sim with greater than 1 airplane, there is no graphics, or at least, only the one plane can be in the flightgear world, while the others are invisible, which seems to defeat the point of the simulation. In that case collision would make sense to have because at least the console output can tell if a collision occurs)

## 1.7 Thesis Overview

This thesis is arranged into five chapters. Chapter II explains relevant background information on the physics of flight modeling, Entity Component Systems, Rust programming and FlightGear. Chapter III provides the thesis methodology. Chapter IV is a results analysis of Chapter III. Finally, Chapter V summarizes the research and impact of the research.

(should any of these headings be deleted, or lumped together as a different heading name? or different heading names? Am I missing the point on any of this?)

# II. Background

This chapter will provide an overview of the components that have gone into building this FDM. First, the background explains the basics of modeling flight physics in a FDM. Next, the ECS architecture, and an implementation if it, SPECS, will be described. Furthermore, the fundamentals of the Rust programming language will be laid out. Next, the use of the FlightGear flight simulator will be explained. Lastly, networking and sending packets of data to FlightGear with Rust will be described.

## 2.1 Physics of Flight Modeling

A flight dynamics model is the process and product used to calculate all of the math and physics required to simulate an airplane. Specifically, we are simulating a rigid-body airplane moving in real-time through both a 2D and a 3D world. With that, this section will provide an overview of what is involved in each FDM in order to approximate real-life flight characteristics of an airplane. Each FDM, 2D and 3D, are similar in many ways, but they do have different considerations in regards to the airplanes rotations, which will be described. But the final result for both is integrating the equations of motion. This boils down to determining the position of the airplane, as well as the airplane’s velocity, which is the speed and direction of the airplane, over time. [20]. But before the equations of motion are solved, an understanding of forces, airplane components, airfoils, rotations, and coordinate systems is needed.

### 2.1.1 Forces

There are four major forces that act on an airplane in flight: gravity, lift, thrust, and drag. These forces govern the motion of the airplane. Newton’s Second Law of Motion says: the acceleration of a body is proportional to the resultant force acting on the body. The resulting equation is F = m * a. Gravity pulls the airplane to the ground and lift is generated by the airplane wings to counteract gravity to keep the airplane aloft. Thrust is generated by the airplanes jet engine or propeller to increase velocity and to keep the airplane generating lift. Drag is the opposite of thrust; drag impedes an airplanes motion. Torque, also known as moment, is similar to force, but force is what causes linear acceleration, and torque is what causes rotational acceleration.  In order to fly, an airplane must have a lift force acting upon it greater than or equal to gravity. Figure 1 is a visual of the four forces [14].

Figure 1: Forces Visualized

### 2.1.2 Airplane Components

An airplane is composed of different parts that affect the state of the airplane. The wings are the rectangular parts coming out of the main body. The span is the length of the wing, whereas the chord is the width of the wing. The ratio of span squared to wing area is called aspect ratio. The ailerons are on the outside end of the wings, and the flaps are on the inside of the wings. The elevators are on the tail, and look like small flaps. The rudder is the vertical flap on the tail. Figure 2 shows these airplane components [14].

Figure 2: Airplane Components

### 2.1.3 Airfoil

The cross-sectional structure of the wings, called an airfoil, is also important in understanding forces. Camber represents the asymmetry between the top surface and bottom surface of the airfoil which creates a curvature. The angle of attack is the angle between the chord line and the direction of travel, or oncoming flow that the plane is moving through [14]. Figure 3 shows a labeled cross-section of an airfoil with an angle of attack [23].

Figure 3: Airfoil Cross-section

### 2.1.4 Rotations

The orientation of the airplane when it is rotating can be described by three angles: pitch, roll, and yaw. These angles are called the Euler angles and represent the x, y, and z axis, on a coordinate plane, respectively. When an airplane is flying in 2D, it can only rotate about 2 axes relative to the pilot. It can pitch, and roll, while yaw is fixed. On the other hand, in 3D, it can rotate about all three axes relative to the pilot. The ailerons create the roll rotation. The elevators create the pitch rotation. The rudder creates the yaw rotation. The flaps do not rotate the plane, but they do affect lift by altering the chord and camber of the wing, which has to do with the shape of the cross-section of each wing. Figure 4 shows the three types of airplane rotations [14].

Figure 4: Airplane Axes of Rotation

### 2.1.5 Coordinate Systems

Coordinate systems are important to understand when determining the location of the airplane on the Earth. Earth-Fixed, Earth-Centered (ECEF) coordinates tell us where the plane is in relation a fixed point, which is the center of the Earth. This is the coordinate system, represented by Cartesian coordinates (x, y, z), used in the FDMs [25]. However, it is important to realize that FlightGear, the visual system, describes an airplane on Earth with the geodetic coordinate system, and not ECEF the coordinate system. The geodetic coordinate system uses latitude, longitude, and altitude to represent location [25]. To work around this, the ECEF coordinates calculated by the FDM need to be transformed into geodetic coordinates so that FlightGear can understand where the airplane is located on Earth. To actually do the conversion to geodetic coordinates, a Rust crate, called coord transforms is used in the FDM given the ECEF coordinates. Figure 5 shows ECEF coordinates and a plane that is relative to that point. Figure 5 also visualizes the coordinate system representing rotations about the fixed point on the pilot [21].

Figure 5: ECEF Coordinates Visualized

### 2.1.6 FDM Calculation Steps

With an understanding of the airplane components, airfoils, forces, rotations, and the coordinate systems, we can begin to explain the steps the FDM takes:

- The mass properties of the airplane are calculated, or decided

- The load, or forces and moments on the airplane are calculated

- The equations of motion are integrated

#### 2.1.6.1 Mass Properties

The mass properties involve calculating how the mass and physical components of the airplane will affect the forces acting upon it. To begin, one must decide which airplane they are trying to model. In Palmer’s model, the Cessna 172 is being modeled. In Bourg’s model, an actual airplane is not being modeled, but the model representation is that of the picture of Figure 2. With that, the mass properties are chosen to represent that desired airplane. For example, the airplanes total wing area will affect both lift and drag forces, and mass will affect gravity.

In terms of 3D, mass properties become more complicated. The rigid body airplane’s center of gravity needs to be calculated, along with airplane’s moment of inertia tensor. The center of gravity, or center of mass, is where the forces are applied on the airplane. The moment of inertia is essentially the airplanes resistance to rotation. It can also be described as a distribution of mass. It is the amount of torque needed to make the airplane rotate about an axis relative to another point (i.e. center of mass). The measure of how fast the airplane actually rotates is called angular velocity [22]. The moment of inertia tensor is the mathematical expression, represented as a 3x3 matrix, that has magnitude and direction, and holds the values of the moments of inertia as body coordinates [14].

#### 2.1.6.2 Forces

With mass properties handled, the forces and moments acting on the airplane at all times can be calculated. Lift and drag are not as simple to calculate compared to gravity and thrust. There are many components on the airplane that affect lift and drag. But to generalize, the total lift force on an airfoil is composed of two components: the lift due to camber and the lift due to attack angle. The airplane’s flaps are used to alter camber. Usually the flaps are used are low speeds to increase lift. Drag increases with increased angle of attack. If an airplane is flying at an angle of attack high enough, it will not be able to create any lift and will stall. Figure 6 shows a graph of increasing lift with increase angle of attack, however, at some point a potential stall occurs with lift dropping off rapidly. Also, shown in Figure 6 is that with flaps lowered, lift is increased [14].

Figure 6: Lift Coefficient vs Angle of Attack

There are also other variables that affect lift such as air density, speed, viscosity, surface area, and aspect ratio. External forces also exist that can act on the airplane such as air drag, and winds.

Gravity is simply based on mass of the airplane and the acceleration of gravity, which is a constant -9.8 m/s2. So, gravity can be calculated by the mass of the airplane x -9.8 m/s2, which is added to the force acting on the z axis (down). Similarly, thrust, expressed in pounds, is force added to the x axis (forward). Without thrust propulsion, the airplane cannot make lift. The thrust will change depending on the simulation users input with the use of the throttle.

#### 2.1.6.3 Equations of Motion

With forces dealt with, the equations of motion are integrated so that the simulations can progress through time, determining the position of the airplane, as well as the airplane’s velocity in each step of time. In computer simulations, the equations of motion are approximated by solving nonlinear ordinary differential equations using numerical integration algorithms [20]. The integration of the equations of motion are considered an approximation because there is always some error introduced depending on the technique chosen [14]. Simply put, the integration of the equations of motion, given everything else talked about, approximates the future velocity and position based on the previous values from the last tick of the simulation [26].

The amount of finite time chosen to represent a tick in the real-time simulation loop is important in numerical integration. A correctly chosen time step will result in stable numerical integration. Stability has to do with how closely an approximate solution converges to the exact solution [14]. A smaller time step will give a result closer to the exact solution but this also means that there are more steps and a larger buildup of numerical precision error. With that said, there is a practical balance as to what the time step should be to achieve a stable solution, and also minimize the buildup of error [14].

With numerical stability in mind, the time step chosen for the simulation loop in the 3D model is 33 milliseconds. This means that the simulation steps through 33 milliseconds of time from frame to frame. This equals 30 frames per second. With this time step, the simulation is stable. Along with that, the simulation loop strategy is that of making frames per second dependent on a constant game speed (33 ms) [28]. This basically means we have a fixed change in time from tick to tick. This strategy was chosen for simplicity sake. This strategy works because we know that the actual integration of the equations of motion takes less time than the chosen 1 tick step time, and so the simulation will be able to keep up [27]. This strategy might normally be an issue for slow computers, but the model is not demanding on hardware. The downside is that for fast computers, we leave some frames per second on the table because we have a fixed time step [28].

There are multiple methods to actually implement numerical integration to calculate the equations of motion. Using Euler’s method is by far the most common numerical integration technique [20]. In both the 2D and 3D model, we use an improved and more accurate version of Euler’s method, called the Range-Kutta method, which is laid out as code in Bourg’s textbook and Palmer’s textbook.

The last aspect to mention about the equations of motion is that, during this process, in the 3D model, the orientation of the plane is tracked with the use of a quaternion. A quaternion is a way to represent orientation of something in 3D. Quaternions get updated with new Euler angles in each time step to reflect the new orientation using the angular velocities of that instant [14].

### 2.1.5 Fidelity

(do we even care about this?) maybe bits of this paragraph can go elsewhere

With the FDMs described, fidelity is the degree to which the characteristics of a flight simulation matches that of the real world [17]. In this case, the FDMs are of “medium” fidelity because the characteristics of the flight simulation match that of real life, but is not computationally expensive, and requires minimal data [18]. One may think that a flight model with the highest fidelity, or accuracy, is the best – this is not always true. Very advanced flight modeling will require large, detailed sets of data about the stability and performance of an aircraft that it is modeling. Wind tunnel data analysis is an example of the kind of data needed for a high fidelity flight model. This data is not readily available and often costs hundreds of thousands of dollars [13].  The physics implemented in these FDM are real and plenty accurate to compared the real world.

(this paragraph on fidelity probably not needed but it does help add merit to the models, is any of this paragraph worth keeping?)

## 2.2 Entity Component System

The FDM will be organized, and implemented in code as an ECS architecture. ECS is a different way to organize data, and a viable alternative to organizing code with OOP. The ECS design is popular in designing game engines. The Unity game engine, for example, uses the ECS design pattern [8]. ECS focuses on the composition of data, whereas OOP focuses on inheritance and hierarchies. ECS gets rid of the troubles that arise with inheritance.

### 2.2.1 Object-Oriented Problem

Suppose we have a space or world that we want to populate with things. These things may have similar attributes as well as unique attributes. With OOP, you would have a base class called Thing that all other Thing types will inherit. The limitation is that one child class can only inherit directly from one parent. For example, a Mammal type may have a child Dog which inherits a fur attribute, and a Bird may have a child Duck which inherits the lays eggs attribute. But in this case, a platypus, which has fur and lays eggs needs to inherit both attributes [9]. This complicates things, and becomes a larger problem with a larger hierarchy. The hierarchy chains that are created in game design can become too large to work with and code becomes too highly dependent on other code [10]. This makes code readability, maintenance, and the addition of features a pain with OOP. However, this is where ECS shines.

### 2.2.2 Entity Component System Example

In an ECS structure, you do not have hierarchy levels – you have these things called Entities, Components, and Systems. You start with an Entity, which represent concrete objects in the world, like a bullet. Entities on their own are useless; they are just containers which Components can be added to. Components hold data and represent a feature. For example, Position is a Component of an Entity. The Components called Position, Render, and Spawning may be added to the Bullet Entity. The mixing and matching of Components can create unique Entities. A Player Entity can be created by adding the Components Position, Render, and Health. Systems are what give functionality to an ECS architecture. A System could be created called Render, which handles all of the rendering functionality of the world. Or a System called Health could handle all of the health functions. Systems have something called a filter. Filters look for every Entity which has some combination of specified Components. For example, the Render System’s filter wants to find every Entity with the Render data Component and Position data Component. Once these Entities are found, they are returned to the Render System to perform some job on it [11]. This is this example visualized [11]:

Figure 7: Entity Component System Visualized

As seen in Figure 7, the Systems are separate from the Components. The Systems, when performing a job, does not care what Entity it is operating on. The whole point is that the data Components are completely separated from the Systems functionality [12]. This separates code dependencies, making code easy to refactor, and makes it simple to implement concurrency. With data separated from functionality, a job can be operated on multiple Entities concurrently. Also, because a Systems does a job on specified data Components, they only iterate over Entities that they care about. This is all good for performance.

### 2.2.3 Flight Dynamics Model as an Entity Component System

In terms of ECS, both the 2D and 3D models are set up similarly. They each have a singular airplane Entity. This Entity has several Components that describe things such as the airplanes position, the airplanes component characteristics (i.e. mass, airfoil), the airplanes current state (i.e. airspeed, altitude), data being modified by keypresses (i.e. thrust, rotations), the current state of actual keyboard presses, and the data that will explicitly be sent to FlightGear as a packet.

In regard to Systems which will perform a job on the Component data, there are three Systems required to run the simulation. There is a System to compute the equations of motion, a System to track the keyboard presses, and a System to send the packets of data calculated to be interpreted by FlightGear.

### 2.2.4 SPECS Framework

To implement the ECS, we will use the SPECS framework, which is a Rust crate that is imported in the code. SPECS makes it easy to define Entities, Components, and Systems to leverage the ECS. SPECS maintains the reasons one would choose to use an ECS design: the resulting code is highly parallel, flexible, and fast [12]. This is good for the FDM because it will be easy to use parallelism, and the code will be easy to modify.

### 2.2.5 ECS Overall

The ECS architecture has proven itself as a dominant strategy in improving game programming. It allows simple parallelization of complex Systems, simple code modification, and quick accesses to the data that matters. With all of these useful attributes, however, ECS does come with some downsides. The learning curve of ECS is absolutely present. To go from coding OOP to ECS, one must change their way of thinking because there is no inheritance, which every programmer is already experienced in. Also, planning ahead when coding in ECS is a necessity; one must know what systems will use what components, and changing components after the fact is not ideal. Since Systems are sometimes dependent on each other to complete, it may be difficult to add another System in the middle of two consecutive Systems. Planning ahead could help to avoid that situation. Also, because parallelization is a big benefit of ECS, it may not be worth using ECS if a program does not need concurrency. For example, a program that computes something and only consist of one system does not benefit from being an ECS. But overall, ECS can introduce some nice benefits once the programmer wraps their head around it.

## 2.3 Rust

Rust is the programming language being leveraged to build the flight dynamics models. It is the language of choice given its advantageous features: memory safety and performance. Rust was released in 2015 by Mozilla, which has been behind the language’s success. Rust will guarantee that a program will be memory safe, with no invalid data accesses, all while remaining just as fast as other popular programming languages [1]. The C++ programming language, for example, is a systems level language, giving developers complete access to data inside memory, with no memory safety precautions. This is only great when the programmer is an expert on writing code that is free of memory issues – this is hard to do. On the other hand, the Python programming language takes away a developer’s access of low level memory, but does provide complete memory safety by using a garbage collector to clean up unneeded memory – but this imposes a runtime cost [1]. Rust is unique because it is a systems level language, it does not have a garbage collector, and it will still guarantee memory safety. This means Rust provides control, is fast, and protects against memory issues such as data races, and dangling pointers [1]. These features built-in to Rust make it the choice programming language for building a FDM.

The safety assurance of Rust is accomplished by its system of Ownership, Borrowing, and Lifetimes. All languages have a way to manage memory while a program is running. Some languages have a garbage collector which actively checks for memory that is no longer being used during runtime. Other languages make the programmer manually allocate and then free memory. Rust takes neither approach; it instead has something called a “borrow checker.” At compile time, the Rust borrow checker will check that all accesses to data is legal and valid [1]. The borrow checker deals with three important Rust concepts: Ownership, Borrowing, and Lifetimes.

### 2.3.1 Safe Rust and Unsafe Rust

Before we begin to discuss what specific features make Rust a “safe” programming language, it is important to know that there is an “unsafe” side of Rust, like C++. By safe, it is meant that the code is completely bug free of memory issues, and undefined behavior such as data races (two processes writing data in the same memory location at the same time), and dangling references (accessing unintended data). The unsafe side of Rust will let the programmer do things that are not safe. And sometimes it is necessary to do unsafe things. For example, sometimes it is required to go into unsafe mode to dereference raw pointers, which are possibly invalid, or out of scope at access [24]. In safe Rust, this is impossible to do because of the protection feature implemented called Lifetimes. The other implemented features, Borrowing and Ownership, also empower safe Rust. We will discuss these features in this chapter now. Overall, the great thing about Rust is that it allows the programmer the tools to write code in safe mode or unsafe mode, whereas in other languages, that mode is unchangeable.

### 2.3.2 Ownership

Ownership refers to the fact that every use of a value needs to be valid and cleaned up when it is done being used. Ownership is accomplished with three rules that the programmer must follow in Rust [2]:

- Every value has a variable which is called its owner

- There may be only one owner at a time

- When an owner goes out of scope, the value is dropped

### 2.3.3 Borrowing

Borrowing is the next important concept in Rust. Borrowing allows data to be “borrowed” from one owner (variable binding) to another owner and accessed for a moment without actually changing Ownership of that data. When Ownership of memory is transferred to another owner, the original owner cannot legally be used, so Borrowing is the solution. The borrow checker will statically guarantee that references always point to valid objects. So, if a reference to an object exists, it cannot be destroyed. There are two rules to Borrowing [2]:

- Any borrow must last for a scope no greater than that of the owner,

- You may have one or the other of these two kinds of borrows, but not both at the same time:

- One or more references to a resource

- Exactly one mutable reference

These rules avoid the possibility of a data race because although there can be more than one pointer to a resource, only one can be accessing and changing that resource at a time. This is good for performance, and uses less memory overhead [1].

### 2.3.4 Lifetimes

The next important Rust concept is Lifetimes. A value has a Lifetime, which is the period that accessing that value is valid to do [2]. A Lifetime starts when a value is created and that Lifetime ends when it is destroyed. Rust’s compiler, or borrow checker, is smart enough to know a values Lifetime in many common circumstances [3]. In these common circumstances, the Lifetime of a value does not need to be explicitly written, but otherwise, a Lifetime parameter needs to be written. The borrow checker tries to limit the period of a Lifetime. A value should not be alive any longer than it absolutely needs to be because, otherwise, it could be referenced by accident. This could cause a dangling reference, which is a reference to unintended data [1]. So, to keep some piece of data alive for only as long as completely necessary, a Lifetime is specified for that data.

### 2.3.5 Rust Overall

With safety dealt with by the borrow checker at compile time, the programmer may be at ease when it comes to memory safety. At the same time, there is no cost in performance at runtime because everything is checked at compile time without a garbage collector. The downsides to Rust are minimal, but they do exist. For one, compile time is longer. Also, the compiler is very strict. New Rust programmers often “fight with the borrow checker” to get their code to compile because it is more difficult to write code that abides by Rusts rules [1]. So less time is spent fixing bugs, and more time is spent getting the compiler right. But the compiler is like a safety net because once the code compiles, the programmer knows it is safe and correct.

Overall, Rust can hold its own against more popular languages, and the learning curve of the language is met with staggering benefits in safety and performance. One will learn that programming in Rust is less about execution flow and what the CPU is doing, and is more about how data is laid out in memory, and how Ownership of memory is given to different parts of the program [4]. The Rust community is active and quickly growing. The Rust ecosystem is also rapidly expanding, with now over 46,000 crates available to download [19].

## 2.4 FlightGear

FlightGear is an open-source flight simulator application. However, it is being used solely as visual system so as to avoid having to handle graphics generation in this thesis. FlightGear, although it has its own built-in FDM, is easily configurable to instead use an external, custom FDM. FlightGear can be configured to be interfaced, or communicated with, via network packets to achieve a desired display output. In this case, these packets of data will be sent from the ECS patterned, Rust application to FlightGear and the packets will contain all of the real-time physics calculations of the FDM. FlightGear will interpret these packets to then know how to display the subsequent simulation to the screen.

To properly configure FlightGear for the simulation, it must manually be told to accept the new external FDM, as well as listen for the respective packets [5]. This is done from the command prompt upon running the executable. The command line arguments are listed in Appendix A: Configuring FlightGear as a Visual System.

With FlightGear configured, it will process the received network packets constructed by the Rust application. But FlightGear requires that these packets are defined in a particular format so as to understand them. The format of the packet, before being converted to bytes, must be a struct, which is outlined in the FlightGear source code. The struct contains all of the data needed to define an airplane and how it flies throughout the simulation. This struct, called FGNetFDM, in Rust code is located in Appendix A: Configuring FlightGear as a visual System.

## 2.5 Networking with Rust

Now, it is understood that networking is needed to send packets of data to FlightGear so that it can display the desired simulation. Networking can simply be defined as two programs communicating with each other, sending and receiving data. To communicate with each other, two sockets are required at each end of communication. In this case, the type of socket being used is the Datagram socket. The protocol involving Datagram sockets is called the User Datagram Protocol (UDP). This protocol is considered “connectionless” because an open connection does not need to be maintained.  Packets containing bytes of data may simply be built and sent to a destination without a connection needing to be established prior [7].

In Rust, a UdpSocket can be created and binded to an internet protocol address and port on the local machine. After this, data can be sent and received from any other socket address. The socket created can then be connected to the other socket with its internet protocol address and port. From here, data can be sent, and received with a function call [2]. One important thing to be aware of is that before data packets are sent out to FlightGear, the literal order of the bytes need to be converted from least significant byte first (little endian), to most significant byte first (big endian) [7].

# III.  Methodology

This chapter will discuss the research methods used in this thesis, including what has been accomplished and how it has been accomplished. We will describe the process of building the desired FDM, and how we certified that it does work as expected – along with the experiment comparing the performance of the built FDM to an OOP counterpart.

## 3.1 Building the FDM

The approach to building the Rust based, ECS patterned, 3D FDM began with an acquiring an understanding of basic flight physics and the Rust programming language. With those basics understood, the first goal was to build a 2D version because the physics are less complicated. The textbook, Physics for Game Programmers, by Grant Palmer provided the C code for his 2D FDM. This FDM was then ported from C into Rust. From here, the 2D FDM was restructured into an ECS design using the SPECS Rust crate, which made this process straightforward. The building of this 2D FDM was essentially a stepping stone to reach the goal of creating the 3D version. We mention this stepping stone progress because the knowledge gained from it is required for the main goal.

The 3D FDM is based on the works by David Bourg and his textbook, Physics for Game Developers, which outlines a 3D FDM in C++ with an OOP design. Similar to before, this C++ FDM from Bourg was ported into Rust. From here, it was converted into an ECS design with the help of the SPECS crate. Ideally, one would begin programming their software from the ground up as an ECS. But, knowing we would need the OOP version in Rust regardless for the upcoming experiment it needed to be written regardless. Also, it made more sense to convert the code directly from C++ to Rust in the same OOP design before it would be broken up into the ECS.

In regard to writing the FDM code, there were very limited resources available describing the process of writing code as an ECS, especially in Rust. The resources used in this thesis boil down to YouTube videos sponsored by Unity, a high-profile proponent of the ECS architecture, and The Specs Book, which is the official documentation on the SPECS framework for Rust. There were many more options to reference for writing Rust code. But the two resource used most for writing Rust code in this thesis was the official Rust book, The Rust Programming Language, and the official code collection called Rust By Example, which showcases executable code snippets.

## 3.2 Interactive Flight Simulator

Talk about how it runs in flight gear and keyboard inputs

## 3.2 The FDM Works

The focus of this research is building a Rust based, ECS patterned, 3D FDM based on the works by David Bourg in [14]. From the naked eye, the FDM looks to fly an airplane correctly in FlightGear. But to ensure our FDM’s works as exactly as expected, we wanted to find out if it replicated the same realistic flight data generated by its C++ baseline FDM as described in Bourg’s textbook. The generated flight data we checked against our FDM is: position coordinates, Euler angles, and time elapsed. This data represents exactly where the airplane is at a given time, and how it is oriented in space. Both the baseline program and our program was compiled and ran with identical instructions as to how to maneuver the airplane at the same time during the simulation for a finite amount of time. This test was a success because our FDM flight data matched the baseline flight data on a predetermined flight path, ensuring our FDM works as it should.

(I will get more specific when I have done the actual test)
(I assume this test should be talked about?)

## 3.2 ECS and OOP Performance Experiment

This experiment was an investigation of performance between the 3D FDM implemented as a Rust based, ECS design and an OOP design implementation of the same FDM.

### 3.2.1 Experimental Design

The OOP version was benchmarked to assess the performance of how quickly it could process all x amount of the airplane entities flying from one spot to another for x seconds. This benchmark was repeated with different variables. One went straight with so much force and no rotation. Two rolled left for the entirety of the test. Three pitched up for the entirety of the benchmark…. More experiments involving x varying entities doing different things and from different positions in the world for different amounts of time. So I will want to organize these. More entities and more time doing more things at once.

The same experiments done for oop will be benchmarked for ecs and those computation times to complete will be compared to see the difference…..

# IV.  Analysis and Results

## Chapter Overview

Text

## Results of Simulation Scenarios

Text

## Investigative Questions Answered

Text

## Summary

Text

# V.  Conclusions and Recommendations

## Chapter Overview

Text

## Conclusions of Research

Text

## Significance of Research

Text

## Recommendations for Action

Text

## Recommendations for Future Research

Text

## Summary

Text

# Appendix: Configuring FlightGear as a Visual System

fgfs.exe – runs the executable
--aircraft=ufo – select an aircraft name
--disable-panel – disable instrument panel
--disable-sound – disable sounds
--enable-hud – enable heads up display 
--disable-random-objects – disable random scenery objects
--fdm=null – turn off the built-in FDM
--vc=0 – specify initial airspeed
--timeofday=noon – time of day 
--native-fdm=socket,in,30,,5500,udp – sets up FlightGear to receive data from an external source. Open a socket, which takes in packets at a speed of 30 hz on port 5500 [6].

Altogether, when running FlightGear, the command line is:

fgfs.exe --aircraft=ufo --disable-panel --disable-sound --enable-hud --disable-random-objects --fdm=null --vc=0 --timeofday=noon --native-fdm=socket,in,30,,5500,udp

(change to 60 hz?)

The struct representing an airplane in FlightGear, written in Rust:

struct FGNetFDM

{

version: u32, // increment when data values change

padding: f32, // padding

// // Positions

longitude: f64, // geodetic (radians)

latitude: f64, // geodetic (radians)

altitude: f64, // above sea level (meters)

agl: f32, // above ground level (meters)

phi: f32, // roll (radians)

theta: f32, // pitch (radians)

psi: f32, // yaw or true heading (radians)

alpha: f32, // angle of attack (radians)

beta: f32, // side slip angle (radians)

// // Velocities

phidot: f32, // roll rate (radians/sec)

thetadot: f32, // pitch rate (radians/sec)

psidot: f32, // yaw rate (radians/sec)

vcas: f32, // calibrated airspeed

climb_rate: f32, // feet per second

v_north: f32, // north velocity in local/body frame, fps

v_east: f32, // east velocity in local/body frame, fps

v_down: f32, // down/vertical velocity in local/body frame, fps

v_body_u: f32, // ECEF velocity in body frame

v_body_v: f32, // ECEF velocity in body frame

v_body_w: f32, // ECEF velocity in body frame

// // Accelerations

A_X_pilot: f32, // X accel in body frame ft/sec^2

A_y_pilot: f32, // Y accel in body frame ft/sec^2

A_Z_pilot: f32, // Z accel in body frame ft/sec^2

// // Stall

stall_warning: f32, // 0.0 - 1.0 indicating the amount of stall

slip_deg: f32, // slip ball deflection

// // Pressure

// // Engine status

num_engines: u32, // Number of valid engines

eng_state: [f32; 4], // Engine state (off, cranking, running)

rpm: [f32; 4], // // Engine RPM rev/min

fuel_flow: [f32; 4], // Fuel flow gallons/hr

fuel_px: [f32; 4], // Fuel pressure psi

egt: [f32; 4], // Exhuast gas temp deg F

cht: [f32; 4], // Cylinder head temp deg F

mp_osi: [f32; 4], // Manifold pressure

tit: [f32; 4], // Turbine Inlet Temperature

oil_temp: [f32; 4], // Oil temp deg F

oil_px: [f32; 4], // Oil pressure psi

// // Consumables

num_tanks: u32, // Max number of fuel tanks

fuel_quantity: [f32; 4],

// // Gear status

num_wheels: u32,

wow: [f32; 3],

gear_pos: [f32; 3],

gear_steer: [f32; 3],

gear_compression: [f32; 3],

// // Environment

cur_time: f32, // current unix time

warp: f32, // offset in seconds to unix time

visibility: f32, // visibility in meters (for env. effects)

// // Control surface positions (normalized values)

elevator: f32,

elevator_trim_tab: f32,

left_flap: f32,

right_flap: f32,

left_aileron: f32,

right_aileron: f32,

rudder: f32,

nose_wheel: f32,

speedbrake: f32,

spoilers: f32

}

# Bibliography

[1] T.S. McNamara. Rust In Action, 2019.

[2] Steve Klabnik and Carol Nichols. The Rust Programming Language, 2019.

[3] Richard Apodaca. Rust Ownership By Example, 2020. [Online; accessed 07/01/2020.

[4] Oliver Gould and Lachie Everson. “All About Rust.” YouTube, uploaded by Microsoft Developer, 2019.

[5] wiki.flightgear.org. Command line options, 2019. [Online accessed 07/02/2020.

[6] wiki.flightgear.org. Property Tree/Sockets, 2019. [Online accessed 07/02/2020.

[7] Brian Hall. Beej’s Guide to Network Programming Using Internet Sockets, 2019.

[8] Mike Geig. “ECS Overview - Intro To The Entity Component System And C# Job System 1/5.” YouTube, uploaded by Unity, 2018.

[9] Ben. “Entity Component System Overview in 7 Minutes.” YouTube, uploaded by Board To Bits Gaming, 2019.

[10] Adam. Entity Systems are the future of MMOG development – Part 2, 2007. . [Online; accessed 07/01/2020.

[11] Mike Geig. “ECS Overview - Intro To The Entity Component System And C# Job System 2/5.” YouTube, uploaded by Unity, 2018.

[12] Thomas Schaller. The Specs Book. Crates.io, 1st edition, 2018.

[13] wow what was this source… (fidelity paragraph)

[14] David Bourg and Bryan Bywalec. Physics for Game Developers, 2nd Edition, 2013.

[15] https://unity.com/solutions/game. Create a world with more play, 2020. [Online accessed 09/01/2020.

[16] https://unity.com/case-study/mediatonic-fall-guys. Unity helps bring the masses together in Mediatonic’s absurd multiplayer, 2020. [Online accessed 09/01/2020].

[17] Albert Rhemann. A Handbook of Flight Simulation Fidelity Requirements for Human Factors Research, 1995.

[18] Gaétan Dussart, Vilius Portapas, Alessandro Pontillo and Mudassir Lone. Flight Dynamic Modelling and Simulation of Large Flexible Aircraft, 2018.

[19] https://crates.io/. The Rust’s Community Crate Registry. [Online accessed 09/01/2020.

[20] Wayne Durham. AIRCRAFT FLIGHT DYNAMICS AND CONTROL, 2013.

[21] Vinayak D. “What is Flight Dynamics? - Derivation of Equations of Motion for an Aircraft.” YouTube, uploaded by VDEngineering, 2019.

[22] https://en.wikipedia.org/wiki/Moment_of_inertia. Moment of Inertia, 2020. [Online accessed 09/01/2020.

[23] https://en.wikipedia.org/wiki/Airfoil. Airfoil, 2020. [Online accessed 09/01/2020.

[24] Steve Klabnik and Carol Nichols. The Rustonomicon, 2019.

[25] D. Rose. Converting Between Earth-Centered, Earth-Fixed and Geodetic Coordinates, 2014.

[26] gaffer on games integration basics

[27] gaffer on games fix your loop

[28] dewitters game loop

[29] HISTORY OF UNITED STATES MILITARY SIMULATION  Raymond R. Hill, J. O. Miller

[30] grant palmer

[31] rust by example https://doc.rust-lang.org/stable/rust-by-example/